SA WG2 Meeting #S2-166	S2-2411290

18 - 22 November, 2024, Orlando, USA

3GPP TSG CT WG3 Meeting #137	C3-245096

# Hefei, CN, 14th – 18th October, 2024

Title:	LS on Clarification related to the QoS and QoE parameters for the support of real time UAV flight path monitoring assistance.

Response to:	NA

Release:	Rel-19

Work Item:	UASAPP_Ph3

Source:	3GPP CT3

To:	3GPP SA6

Cc:	3GPP SA2, 3GPP CT1

Contact person:	Rajesh Babu Natarajan

rajesh underscore babu dot natarajan at nokia dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

As per TS 23.255:

#### 7.9.2.1	Real time UAV flight path monitoring assistance procedure

1.	The UAE server receives a Flight path reporting configuration request from the UAS application specific server including QoS and QoE related triggering events (QoS thresholds, QoE thresholds, periodicity to collect QoS and QoE information), validity scope (flight path waypoints and/or geographical area, time). The QoS and QoE related triggering events can be provided separately for Uu and for PC5.

#### 7.9.2.2	Real time flight path monitoring assistance configuration of UAE Client

1.	The UAE server sends to the UAE client a Flight path reporting client configuration request including the UAS identifier, including QoS and QoE related triggering events (QoS thresholds, QoE thresholds, periodicity to collect QoS and QoE information), validity scope (flight path waypoints and/or geographical area, time). In case of complete removal of the flight path event, the request shall only include the UAS identifier.

#### 7.9.3.1	Flight path reporting configuration request

Table 7.9.3.1-1 describes the flight path reporting configuration request from the UAS application specific server to the UAE server.

Table 7.9.3.1-1: Flight path reporting configuration request

| Information element | Status | Description |
| --- | --- | --- |
| … | … | … |
| > QoS parameters | O | Parameters for flight path configuration related with QoS. - QoS thresholds - QoS periodicity |
| > QoE parameters | O | Parameters for flight path configuration related with QoE. - QoE thresholds - QoE periodicity |
| … | … | ... |

#### 7.9.3.4	Flight path reporting client configuration request

Table 7.9.3.4-1 describes the information flow flight path reporting client configuration request from the UAE server to the UAE client.

Table 7.9.3.4-1: Flight path reporting client configuration request

| Information element | Status | Description |
| --- | --- | --- |
| … | … | … |
| > QoS parameters | O | Parameters for flight path configuration related with QoS. - QoS thresholds - QoS periodicity |
| > QoE parameters | O | Parameters for flight path configuration related with QoE. - QoE thresholds - QoE periodicity |
| … | … | … |

Question 1: What are the QoS parameters and QoE parameters to be considered, whose thresholds define the triggering events in the above mentioned use case?

Question 2: How the UAS application specific server obtains/does the measurement of these QoS and QoE parameters values?

# 2	Actions

To SA6

ACTION: 	CT3 kindly asks SA6 to answer the above questions and update the SA6 specifications respectively, if necessary.

# 3	Dates of next TSG CT WG 3 meetings

CT3 Meeting calendar can be found at:

https://www.3gpp.org/dynareport?code=Meetings-C3.htm