3GPP TSG-SA2 Meeting #166	S2-2411374

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1236 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2: VFL services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | See considerations in S2-2411373 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define new services to support VFL |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL cannot be implemted as services are missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1, 7.10, new 7.X, new X, new Y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

## 7.1	General

Table 7.1-1 illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, MFAF, LMF |
|  | Transfer | Request / Response | NWDAF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF |
|  | ContextTransfer | Request / Response | NWDAF |
| Nnwdaf_DataManagement | Subscribe | Subscribe / Notify | NWDAF, DCCF |
|  | Notify |  | NWDAF, DCCF, MFAF, ADRF |
|  | Fetch | Request / Response | NWDAF, DCCF, MFAF, ADRF |
| Nnwdaf_MLModelProvision | Subscribe | Subscribe / Notify | NWDAF, LMF |
|  | Unsubscribe |  | NWDAF, LMF |
|  | Notify |  | NWDAF, LMF |
| Nnwdaf_MLModelInfo | Request | Request / Response | NWDAF, LMF |
| Nnwdaf_MLModelMonitor | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
|  | Register | Request / Response | NWDAF |
|  | Request |  | NWDAF |
| Nnwdaf_MLModelTraining | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelTrainingInfo | Request | Request / Response | NWDAF |
| Nnwdaf_RoamingAnalytics | Subscribe | Subscribe / Notify | H-RE-NWDAF, V-RE-NWDAF |
|  | Unsubscribe |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Notify |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Request | Request / Response | H-RE-NWDAF, V-RE-NWDAF |
| Nnwdaf_RoamingData | Subscribe | Subscribe / Notify | H-RE-NWDAF, V-RE-NWDAF |
|  | Unsubscribe |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Notify |  | H-RE-NWDAF, V-RE-NWDAF |
| Nnwdaf_VflInference | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
|  | Request | Request / Response | NWDAF |
| NOTE 1: How OAM consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.550 [7] Annex H and out of the scope of this TS. NOTE 2: How CEF consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.201 [21] and out of the scope of this TS. NOTE 3: The Nnwdaf_MLModelProvision service and the Nnwdaf_MLModelInfo service are provided by an NWDAF containing MTLF and consumed by an NWDAF containing AnLF or provided by an NWDAF containing MTLF supporting FL as a server and consumed by an NWDAF containing MTLF. |  |  |  |

Editor´s note: It is FFS whether AF and NEF can also be consumers of the Nnwdaf_VflInference service.

Table 7.1-2 shows the analytics information provided by NWDAF service.

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Load level provided as number of UE registrations and number of PDU sessions for a Network Slice and Network Slice instances as well as resource utilization for Network Slice instances. |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience statistics or predictions may be provided for a Network Slice or an Application. They may be derived from an individual UE, a group of UEs or any UE. For slice service experience, they may be derived from an Application, a set of Applications or all Applications on the Network Slice. |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. When visited AOI(s) is included in the Analytics Filter information, only statistics on UE mobility can be provided. |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | List of observed or expected exceptions, with Exception ID, Exception Level and other information, depending on the observed or expected exceptions. |
| End-to-end data volume transfer time | Analytics ID: E2E data volume transfer time | Analytics on E2E data volume transfer time. |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |
| Session Management Congestion Control Experience | Analytics ID: Session Management Congestion Control Experience | Statistics on session management congestion control experience for specific DNN and/or S-NSSAI. |
| Redundant Transmission Experience | Analytics ID: Redundant Transmission Experience | Statistics or predictions aimed at supporting redundant transmission decisions for URLLC services. |
| WLAN performance | Analytics ID: WLAN performance | Statistics or predictions on WLAN performance of UE. |
| Dispersion | Analytics ID: UE Dispersion | Statistics or predictions that identify the location (i.e. areas of interest) or network slice(s) where a UE, or a group of UEs disperse their data volume, or disperse mobility or session management transactions or both. |
| DN Performance | Analytics ID: DN Performance | Statistics or predictions on user plane performance for a specific Edge Computing application. |
| PFD Determination | Analytics ID: PFD Determination | Statistics on PFD information for a known application identifier(s). |
| Movement Behaviour | Analytics ID: Movement Behaviour | Statistics or predictions on movement behaviour for an applicable area. |
| Location Accuracy | Analytics ID: Location Accuracy | Predictions on Location Accuracy. |
| Relative Proximity | Analytics ID: Relative Proximity | Statistics or predictions on Relative Proximity among UEs. |
| PDU Session traffic | Analytics ID: PDU Session traffic | Statistics on whether traffic of UEs via one or multiple PDU sessions is according to the information provided by the service consumer. |

2nd change

## 7.10	Nnwdaf_MLModelTraining Service

### 7.10.1	General

Service Description: This service enables the consumer to subscribe/unsubscribe/notify/modify for ML Model training.

NOTE:	In this release of the specification, the service provider and consumer are limited to NWDAF containing MTLF.

When used for Horizontal Federated Learning, this service enables FL server NWDAF as consumer to enable Horizontal Federated Learning while providing global ML Model information to FL Client NWDAF and getting local ML Model information and status report of FL training as defined in clause 6.2C.2.3 from the FL Client NWDAF.

When used for Vertical Federated Learning, this service enables a VFL server (NWDAF or AF) as consumer to enable Vertical Federated by requesting an NWDAF acting as VFL client to train a local model for a feature as defined in clause 6.2H.2.3.

For HFL, this service may also be used by the consumer (i.e. FL Server NWDAF) to check if the service provider (i.e. FL Client NWDAF) can meet the ML Model training requirement as described in clause 6.2F.1.

For VFL, this service may also be used by the consumer (i.e. FL Server) to prepare the VFL training as described in in clause 6.2H.2.2.

This service may also be used by the consumer (i.e. HFL Server NWDAF) to request the service provider (i.e. HFL Client NWDAF) to calculate and provide Model Accuracy of the global ML Model as described in clause 6.2F.1.

Editor´s note: VFL related Parameters of the service operations are FFS

### 7.10.2	Nnwdaf_MLModelTraining_Subscribe service operation

Service operation name: Nnwdaf_MLModelTraining_Subscribe

Description: Subscribes to NWDAF ML Model training with specific parameters.

Inputs, Required:

-	Analytics ID as defined in Table 7.1-2 or with an unstandardized value;

-	ML Model Interoperability information;

-	Notification Target Address (+ Notification Correlation ID).

Inputs, Optional:

-	Not applicable for VFL: ML Model Interoperability information;

-	Not applicable for VFL: ML Model identifier: identifies the provided ML Model.

-	Not applicable for VFL: ML Model Information (as defined in clause 6.2A.2);

-	Not applicable for VFL: ML Model file;

-	Subscription Correlation ID (in the case of modification of the ML Model Training subscription);

-	ML Training Information, i.e. data availability requirement, time availability requirement.

-	ML Preparation Flag;

-	Not applicable for VFL: ML Model Accuracy Check Flag;

-	Not applicable for VFL: ML Correlation ID;

-	Training Filter Information;

-	Target of Training Reporting;

-	Training Reporting Information as defined in clause 6.2F.2;

-	Not applicable for VFL: Use case context;

-	Iteration round ID;

-	Expiry time.

-	Indication of skipping the current FL round.

-	VFL Correlation ID

-	VFL backward local ML model training information

-	VFL Feature ID

Outputs Required: When the request is accepted: Subscription Correlation ID (required for management of this subscription). When the request is not accepted, an error response with cause code (e.g. NWDAF does not meet the ML training requirements, ML training is not complete, NWDAF overload, not available for the FL process anymore, etc.).

NOTE:	The detail reasons in the cause code are up to Stage 3.

Outputs, Optional: Not applicable for VFL ML Correlation ID (e.g. confirm of the subscription for this FL process).

### 7.10.3	Nnwdaf_MLModelTraining_Unsubscribe service operation

Service operation name: Nnwdaf_MLModelTraining_Unsubscribe

Description: Terminate NWDAF ML Model training.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Cause code (e.g. FL Client NWDAF is unselected by the FL Server NWDAF for the FL process, or the FL process is suspended or finished, etc.).For HFL, final aggregated ML Model information (if FL has finished) or updated aggregated ML Model information (if FL is suspended).

### 7.10.4	Nnwdaf_MLModelTraining_Notify service operation

Service operation name: Nnwdaf_MLModelTraining_Notify

Description: NWDAF notifies the consumer instance that has subscribed to the specific NWDAF service of the trained ML Model (for HFL) or of client intermediate training result of the local ML mode (for VFL). The NWDAF can also use this service to indicate to consumer it will terminate the ML Model training.

Inputs, Required:

-	Notification Correlation Information: this parameter indicates the Notification Correlation ID that has been assigned by the consumer during ML Model training.

Inputs, Optional:

-	Not applicable for VFL: Set of the tuple (Analytics ID, ML Model Information as defined in clause 6.2F.2;

-	Not applicable for VFL: ML Correlation ID, when for Horizontal Federated Learning;

-	Not applicable for VFL: Corresponding Use case context;

-	Termination Request: this parameter indicates that NWDAF requests to terminate the ML Model training, i.e. NWDAF will not provide further notifications related to this request, with cause code (e.g. NWDAF overload, not available for the FL process anymore, etc.);

-	Not applicable for VFL: ML Model identifier: this parameter identifies the provisioned ML Model;

-	Not applicable for VFL: Global ML Model Accuracy information: The model accuracy of the global ML Model, which is calculate by the FL Client NWDAF using the local training data as the testing dataset;

-	Not applicable for VFL: Status report of FL training: local ML Model accuracy information and Training Input Data Information (e.g. areas covered by the data set, sampling ratio, maximum/minimum of value of each dimension, etc.), which are generated by the FL Client NWDAF during FL procedure;

-	Delay Event Notification: as defined in clause 6.2F.2;

-	Iteration round ID.

-	For VFL, client intermediate training result of the local ML mode

NOTE:	The detail reasons in the cause code are up to stage 3.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

3rd change

## 7.X	Nnwdaf_VflInference Service

### 7.X.1	General

Service Description: This service is provided by an NWDAF/AnLF acting as VFL client and enables an VFL server as consumer to request or subscribe/unsubscribe for a VFL inference.

When the subscription is accepted by the NWDAF containing AnLF, the consumer receives from the NWDAF an identifier (Subscription Correlation ID) allowing to further manage (modify, delete) this subscription.

Editor´s note: Parameters of the service operations are FFS

### 7.X.2	Nnwdaf_VflInference_Subscribe service operation

Service operation name: Nnwdaf_ VflInference_Subscribe

Description: Subscribe to VFL inference.

Inputs, Required:

-	Analytics ID as defined in Table 7.1-2;

-	Target of Analytics Reporting;

-	Notification Target Address (+ Notification Correlation ID);

-	VFL Correlation ID

-	Vendor-specific featore information

Inputs, Optional:

-	Analytics Filter Information;

-	Subscription Correlation ID (in the case of modification of the subscription);

-	Preferred level of accuracy;

-	Reporting Thresholds;

-	Maximum number of objects requested (max);

-	Maximum number of SUPIs requested (SUPImax);

-	Time when information is needed;

-	Analytics Metadata Request;

-	Data time window;

-	Consumer NF's serving area or NF ID;

NOTE 4:	Other input parameters specific for different analytics ID can be also needed, as specified in the corresponding clause for each analytics ID.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: none

### 7.X.3	Nnwdaf_ VflInference_Unsubscribe service operation

Service operation name: Nnwdaf_ VflInference_Unsubscribe

Description: Unsubscribe to VFL inference.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

### 7.X.4	Nnwdaf_ VflInference_Notify service operation

Service operation name: Nnwdaf_VflInference_Notify

Description: Notify VFL inference result.

Inputs, Required:

- Notification Correlation Information: this parameter indicates the Notification Correlation ID that has been assigned by the consumer during analytics subscription.

Inputs, Optional:

- intermediate local inference results

-	Timestamp of intermediate result generation (required when ADRF is deployed).

-	Validity period.

-	Confidence

-	Client Analytics Metadata Information.

-	Termination Request: this parameter indicates that NWDAF requests to terminate the VFL inference subscription, i.e. NWDAF will not provide further notifications related to this subscription, with cause value (e.g. NWDAF overload, UE moved out of NWDAF serving area, etc.).

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

### 7.X.5	Nnwdaf_VflInference_Request service operation

Service operation name: Nnwdaf_VflInference_Request

Description: The consumer requests the NWDAF to perform a one-time VFL inference.

Inputs, Required:

- Analytics ID as defined in Table 7.1-2,

-	Target of Analytics Reporting,

-	VFL model Correlation ID

-	Vendor-specific featore information

Inputs, Optional:

-	Analytics Filter Information;

-	Preferred level of accuracy;

-	Reporting Thresholds;

-	Maximum number of objects requested (max);

-	Maximum number of SUPIs requested (SUPImax);

-	Time when information is needed;

-	Analytics Metadata Request;

-	Data time window;

-	Consumer NF's serving area or NF ID;

Outputs, Required: If the request is accepted, then intermediate local inference results. When the request is not accepted, an error response.

Outputs, Optional:

-	Timestamp of intermediate result generation (required when ADRF is deployed).

-	Validity period.

-	Confidence

-	Client Analytics Metadata Information.

NOTE 1:	Other input parameters specific for different analytics ID can be also needed and are left to the impementation.

4th change

# X	AF Services to support to support network data analytics

## X.1	General

Table X.1-1 illustrates the AF Services to support to support network data analytics.

Table X.1-1: NF services provided by AF to support to support network data analytics

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Naf_VflTraining | Subscribe | Subscribe / Notify | NWDAF, NEF |
|  | Unsubscribe |  | NWDAF, NEF |
|  | Notify |  | NWDAF, NEF |
| Naf_VflInference | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
|  | Request | Request / Response | NWDAF |

## X.2	Naf_VflTraining Service

### X.2.1	General

Service Description: This service is provided by an AF acting as VFL client and enables an NWDAF VFL server or an NEF acting on its behalf as consumer to request the AF to participate in VFL model training as VFL client and train a local model for a feature as defined in clause 6.2H.2.3

For VFL, this service may also be used by the consumer (i.e. FL Server) to prepare the VFL training as described in in clause 6.2H.2.2.

Editor´s note: Parameters of the service operations are FFS

### X.2.2	Naf_VflTraining_Subscribe service operation

Service operation name: Naf_VflTraining_Subscribe

Description: Subscribes to VFL ML Model training with AF as VFL client.

Inputs, Required:

-	Analytics ID as defined in Table 7.1-2;

-	Notification Target Address (+ Notification Correlation ID).

Inputs, Optional:

-	Subscription Correlation ID (in the case of modification of the ML Model Training subscription);

-	ML Training Information, i.e. data availability requirement, time availability requirement.

-	ML Preparation Flag;

-	Training Filter Information;

-	Target of Training Reporting;

-	Training Reporting Information as defined in clause 6.2F.2;

-	Iteration round ID;

-	Expiry time.

-	Indication of skipping the current FL round.

-	VFL Correlation ID

-	VFL backward local ML model training information

-	VFL Feature ID

Outputs Required: When the request is accepted: Subscription Correlation ID (required for management of this subscription). When the request is not accepted, an error response with cause code (e.g. AF does not meet the ML training requirements, ML training is not complete, AF overload, not available for the FL process anymore, etc.).

NOTE:	The detail reasons in the cause code are up to Stage 3.

Outputs, Optional: none

### X.2.3	Naf_VflTraining_Unsubscribe service operation

Service operation name: Naf_VflTraining_Unsubscribe

Description: Terminate AF VFL ML Model training.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Cause code (e.g. FL Client AF is unselected by the FL Server NWDAF for the FL process, or the FL process is suspended or finished, etc.).

### X.2.4	Naf_VflTraining _Notify service operation

Service operation name: Naf_VflTraining_Notify

Description: AF notifies the consumer of client intermediate training result of the local ML mode (for VFL)..

Inputs, Required:

-	Notification Correlation Information: this parameter indicates the Notification Correlation ID that has been assigned by the consumer during ML Model training.

Inputs, Optional:

-	Termination Request: this parameter indicates that AF requests to terminate the ML Model training, i.e. AF will not provide further notifications related to this request, with cause code (e.g. AF overload, not available for the FL process anymore, etc.);

-	Delay Event Notification: as defined in clause 6.2F.2;

-	Iteration round ID.

-	client intermediate training result of the local ML mode

NOTE:	The detail reasons in the cause code are up to stage 3.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

## X.3	Naf_VflInference Service

### X.3.1	General

Service Description: This service is provided by AF acting as VFL client and enables an VFL server as consumer to request or subscribe/unsubscribe for a VFL inference.

When the subscription is accepted by the AF, the consumer receives from the NWDAF an identifier (Subscription Correlation ID) allowing to further manage (modify, delete) this subscription.

Editor´s note: Parameters of the service operations are FFS

### X.3.2	Naf_VflInference_Subscribe service operation

Service operation name: Naf_ VflInference_Subscribe

Description: Subscribe to VFL inference.

Inputs, Required:

-	Analytics ID as defined in Table 7.1-2;

-	Target of Analytics Reporting;

-	Notification Target Address (+ Notification Correlation ID);

-	VFL model Correlation ID

-	Vendor-specific featore information

Inputs, Optional:

-	Analytics Filter Information;

-	Subscription Correlation ID (in the case of modification of the subscription);

-	Preferred level of accuracy;

-	Reporting Thresholds;

-	Maximum number of objects requested (max);

-	Maximum number of SUPIs requested (SUPImax);

-	Time when information is needed;

-	Analytics Metadata Request;

-	Data time window;

-	Consumer NF's serving area or NF ID;

NOTE 4:	Other input parameters specific for different analytics ID can be also needed, as specified in the corresponding clause for each analytics ID.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: none

### X.3.3	Naf_VflInference_Unsubscribe service operation

Service operation name: Naf_ VflInference_Unsubscribe

Description: Unsubscribe to VFL inference.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

### X.3.4	Naf_VflInference_Notify service operation

Service operation name: Naf_VflInference_Notify

Description: Notify VFL inference result.

Inputs, Required:

- Notification Correlation Information: this parameter indicates the Notification Correlation ID that has been assigned by the consumer during analytics subscription.

Inputs, Optional:

- intermediate local inference results

-	Timestamp of intermediate result generation (required when ADRF is deployed).

-	Validity period.

-	Confidence

-	Client Analytics Metadata Information.

-	Termination Request: this parameter indicates that AF requests to terminate the VFL inference subscription, i.e. AF will not provide further notifications related to this subscription, with cause value (e.g. AF overload, UE moved out of AF serving area, etc.).

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

### X.3.5	Naf_VflInference_Request service operation

Service operation name: Naf_VflInference_Request

Description: The consumer requests the AF to perform a one-time VFL inference.

Inputs, Required:

- Analytics ID as defined in Table 7.1-2,

-	Target of Analytics Reporting,

-	VFL model Correlation ID

-	Vendor-specific featore information

Inputs, Optional:

-	Analytics Filter Information;

-	Preferred level of accuracy;

-	Reporting Thresholds;

-	Maximum number of objects requested (max);

-	Maximum number of SUPIs requested (SUPImax);

-	Time when information is needed;

-	Analytics Metadata Request;

-	Data time window;

-	Consumer NF's serving area or NF ID;

Outputs, Required: If the request is accepted, then intermediate local inference results. When the request is not accepted, an error response.

Outputs, Optional:

-	Timestamp of intermediate result generation (required when ADRF is deployed).

-	Validity period.

-	Confidence

-	Client Analytics Metadata Information.

NOTE 1:	Other input parameters specific for different analytics ID can be also needed and are left to the impementation.

5th change

# Y	NEF Services to support to support network data analytics

## Y.1	General

Table Y.1-1 illustrates the NEF Services to support to support network data analytics.

Table Y.1-1: NF services provided by NEF to support to support network data analytics

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_VflTrainingAfClient | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnef_VflInferenceAfClient | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
|  | Request | Request / Response | NWDAF |
| Nnef_VflNwdafClient | NwdafDiscovery | Request / Response | AF |
|  | NwdafRelease | Request / Response | AF |
|  | TrainingSubscribe | Subscribe / Notify | AF |
|  | TrainingUnsubscribe |  | AF |
|  | TrainingNotify |  | AF |
|  | InferenceSubscribe | Subscribe / Notify | AF |
|  | InferenceUnsubscribe |  | AF |
|  | InferenceNotify |  | AF |
|  | InferenceRequest | Request / Response | AF |

## Y.2	Nnef_VflTrainingAfClient Service

### Y.2.1	General

Service Description: This service is provided by an NEF on behalf of an AF acting as VFL client and enables an NWDAF VFL server as consumer to request the AF to participate in VFL model training as VFL client and train a local model for a feature as defined in clause 6.2H.2.3

For VFL, this service may also be used by the consumer (i.e. FL Server) to prepare the VFL training as described in in clause 6.2H.2.2.

Editor´s note: Parameters of the service operations are FFS

### Y.2.2	Nnef_VflTrainingAfClient_Subscribe service operation

Service operation name: Nnef_VflTrainingAfClient_Subscribe

Description: Subscribes to VFL ML Model training with AF as VFL client.

Inputs, Required:

-	Analytics ID as defined in Table 7.1-2;

-	Notification Target Address (+ Notification Correlation ID).

Inputs, Optional:

-	Subscription Correlation ID (in the case of modification of the ML Model Training subscription);

-	ML Training Information, i.e. data availability requirement, time availability requirement.

-	ML Preparation Flag;

-	Training Filter Information;

-	Target of Training Reporting;

-	Training Reporting Information as defined in clause 6.2F.2;

-	Iteration round ID;

-	Expiry time.

-	Indication of skipping the current FL round.

-	VFL Correlation ID

-	VFL backward local ML model training information

-	VFL Feature ID

Outputs Required: When the request is accepted: Subscription Correlation ID (required for management of this subscription). When the request is not accepted, an error response with cause code (e.g. AF does not meet the ML training requirements, ML training is not complete, AF overload, not available for the FL process anymore, etc.).

NOTE:	The detail reasons in the cause code are up to Stage 3.

Outputs, Optional: none

### Y.2.3	Nnef_VflTrainingAfClient_Unsubscribe service operation

Service operation name: Nnef_VflTrainingAfClient_Unsubscribe

Description: Terminate AF VFL ML Model training.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Cause code (e.g. FL Client AF is unselected by the FL Server NWDAF for the FL process, or the FL process is suspended or finished, etc.).

### Y.2.4	Nnef_VflTrainingAfClient_Notify service operation

Service operation name: Nnef_VflTrainingAfClient_Notify

Description: AF notifies the consumer of client intermediate training result of the local ML mode.

Inputs, Required:

-	Notification Correlation Information: this parameter indicates the Notification Correlation ID that has been assigned by the consumer during ML Model training.

Inputs, Optional:

-	Termination Request: this parameter indicates that AF requests to terminate the ML Model training, i.e. AF will not provide further notifications related to this request, with cause code (e.g. AF overload, not available for the FL process anymore, etc.);

-	Delay Event Notification: as defined in clause 6.2F.2;

-	Iteration round ID.

-	client intermediate training result of the local ML mode

NOTE:	The detail reasons in the cause code are up to stage 3.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

## Y.3	Nnef_VflInferenceAfClient Service

### Y.3.1	General

Service Description: This service is provided by by an NEF on behalf of an AF acting as VFL client and enables an VFL server as consumer to request or subscribe/unsubscribe for a VFL inference.

When the subscription is accepted by the AF, the consumer receives from the NWDAF an identifier (Subscription Correlation ID) allowing to further manage (modify, delete) this subscription.

Editor´s note: Parameters of the service operations are FFS

### Y.3.2	Nnef_VflInference_Subscribe service operation

Service operation name: Nnef_ VflInference_Subscribe

Description: Subscribe to VFL inference.

Inputs, Required:

-	Analytics ID as defined in Table 7.1-2;

-	Target of Analytics Reporting;

-	Notification Target Address (+ Notification Correlation ID);

-	VFL Correlation ID

-	Vendor-specific featore information

Inputs, Optional:

-	Analytics Filter Information;

-	Subscription Correlation ID (in the case of modification of the subscription);

-	Preferred level of accuracy;

-	Reporting Thresholds;

-	Maximum number of objects requested (max);

-	Maximum number of SUPIs requested (SUPImax);

-	Time when information is needed;

-	Analytics Metadata Request;

-	Data time window;

-	Consumer NF's serving area or NF ID;

NOTE 4:	Other input parameters specific for different analytics ID can be also needed, as specified in the corresponding clause for each analytics ID.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: none

### Y.3.3	Nnef_VflInference_Unsubscribe service operation

Service operation name: Nnef_ VflInference_Unsubscribe

Description: Unsubscribe to VFL inference.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

### Y.3.4	Nnef_VflInference_Notify service operation

Service operation name: Nnef_VflInference_Notify

Description: Notify VFL inference result.

Inputs, Required:

- Notification Correlation Information: this parameter indicates the Notification Correlation ID that has been assigned by the consumer during analytics subscription.

Inputs, Optional:

- intermediate local inference results

-	Timestamp of intermediate result generation (required when ADRF is deployed).

-	Validity period.

-	Confidence

-	Client Analytics Metadata Information.

-	Termination Request: this parameter indicates that NWDAF requests to terminate the VFL inference subscription, i.e. NWDAF will not provide further notifications related to this subscription, with cause value (e.g. NWDAF overload, UE moved out of NWDAF serving area, etc.).

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

### Y.3.5	Nnef_VflInference_Request service operation

Service operation name: Nnef_VflInference_Request

Description: The consumer requests the NWDAF to perform a one-time VFL inference.

Inputs, Required:

- Analytics ID as defined in Table 7.1-2,

-	Target of Analytics Reporting,

-	VFL model Correlation ID

-	Vendor-specific featore information

Inputs, Optional:

-	Analytics Filter Information;

-	Preferred level of accuracy;

-	Reporting Thresholds;

-	Maximum number of objects requested (max);

-	Maximum number of SUPIs requested (SUPImax);

-	Time when information is needed;

-	Analytics Metadata Request;

-	Data time window;

-	Consumer NF's serving area or NF ID;

Outputs, Required: If the request is accepted, then intermediate local inference results. When the request is not accepted, an error response.

Outputs, Optional:

-	Timestamp of intermediate result generation (required when ADRF is deployed).

-	Validity period.

-	Confidence

-	Client Analytics Metadata Information.

NOTE 1:	Other input parameters specific for different analytics ID can be also needed and are left to the impementation.

## Y.4	Nnef_VflNwdafClient Service

### Y.4.1	General

Service Description: This service is provided by by an NEF towards an untrusted AF acting as VFL server to enable the AF to interact with NWDAFs acting as VFL client. It enables the AF to detect NWDAFs as VFL clients as described in in clause 6.2H.2.1, to prepare the VFL training as described in in clause 6.2H.2.2., to request NWDAFs to participate in VFL model training as VFL client and train a local model for a feature as defined in clause 6.2H.2.3, and enables an AF VFL server as consumer to request or subscribe/unsubscribe at NWDAFs as VFL clients for a VFL inference as defined in clause 6.2H.2.4.

Editor´s note: Parameters of the service operations are FFS

### Y.4.2	Nnef_VflNwdafClient_NwdafDiscovery service operation

Service operation name: Nnef_VflNwdafClient_NwdafDiscovery

Description: The consumer requests the NEF to discover an NWDAF that is to act as VFL client.

Inputs, Required:

- Analytics ID as defined in Table 7.1-2 or with unstandardized value

-	required NF type (i.e. NWDAF type)

-	VFL capability type (i.e. FL client),

Inputs, Optional:

-	Vendor-specific feature information

-	Time Period of Interest

-	optional Service Area;

Outputs, Required: If the request is accepted, temporary NWDAF ID. When the request is not accepted, an error response.

Outputs, Optional:

-	none.

### Y.4.3	Nnef_VflNwdafClient_NwdafRelease service operation

Service operation name: Nnef_VflNwdafClient_NwdafDiscovery

Description: The consumer informs the NEF that it mo longer wants .to use an assigned temporary NWDAF ID.

Inputs, Required:

-	temporary NWDAF ID

Inputs, Optional:

-	None;

Outputs, Required:.

-	None;

Outputs, Optional:

-	none.

### Y.4.4	Nnef_VflNwdafClient_TrainingSubscribe service operation

Service operation name: Nnef_VflNwdafClient_TrainingSubscribe

Description: Subscribes to VFL ML Model training with NWDAF as VFL client.

Inputs, Required:

-	Analytics ID as defined in Table 7.1-2;

-	Notification Target Address (+ Notification Correlation ID).

-	temporary NWDAF ID

Inputs, Optional:

-	Subscription Correlation ID (in the case of modification of the ML Model Training subscription);

-	ML Training Information, i.e. data availability requirement, time availability requirement.

-	ML Preparation Flag;

-	Training Filter Information;

-	Target of Training Reporting;

-	Training Reporting Information as defined in clause 6.2F.2;

-	Iteration round ID;

-	Expiry time.

-	Indication of skipping the current FL round.

-	VFL Correlation ID

-	VFL backward local ML model training information

-	VFL Feature ID

Outputs Required: When the request is accepted: Subscription Correlation ID (required for management of this subscription). When the request is not accepted, an error response with cause code (e.g. NWDAF does not meet the ML training requirements, ML training is not complete, NWDAF overload, not available for the FL process anymore, etc.).

NOTE:	The detail reasons in the cause code are up to Stage 3.

Outputs, Optional: none

### Y.4.5	Nnef_VflNwdafClient_TrainingUnubscribe service operation

Service operation name: Nnef_VflNwdafClient_TrainingUnubscribe

Description: Terminate NWDAF VFL ML Model training.

Inputs, Required:

-	temporary NWDAF ID

-	Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Cause code (e.g. FL Client NWDAF is unselected by the FL Server AF for the FL process, or the FL process is suspended or finished, etc.).

### Y.4.6	Nnef_VflNwdafClient_TrainingNotify service operation

Service operation name: Nnef_VflNwdafClient_TrainingNotify

Description: NWDAF notifies the consumer of client intermediate training result of the local ML mode.

Inputs, Required:

-	temporary NWDAF ID

-	Notification Correlation Information: this parameter indicates the Notification Correlation ID that has been assigned by the consumer during ML Model training.

Inputs, Optional:

-	Termination Request: this parameter indicates that AF requests to terminate the ML Model training, i.e. NWDAF will not provide further notifications related to this request, with cause code (e.g. NWDAF overload, not available for the FL process anymore, etc.);

-	Delay Event Notification: as defined in clause 6.2F.2;

-	Iteration round ID.

-	client intermediate training result of the local ML mode

NOTE:	The detail reasons in the cause code are up to stage 3.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

### Y.4.7	Nnef_VflNwdafClient_InferenceSubscribe service operation

Service operation name: Nnef_VflNwdafClient_InferenceSubscribe

Description: Subscribe to VFL inference with NWDAF as VFL client.

Inputs, Required:

-	Analytics ID as defined in Table 7.1-2;

-	Target of Analytics Reporting;

-	Notification Target Address (+ Notification Correlation ID);

-	VFL Correlation ID

-	Vendor-specific featore information

-	temporary NWDAF ID

Inputs, Optional:

-	Analytics Filter Information;

-	Subscription Correlation ID (in the case of modification of the subscription);

-	Preferred level of accuracy;

-	Reporting Thresholds;

-	Maximum number of objects requested (max);

-	Maximum number of SUPIs requested (SUPImax);

-	Time when information is needed;

-	Analytics Metadata Request;

-	Data time window;

-	Consumer NF's serving area or NF ID;

NOTE 4:	Other input parameters specific for different analytics ID can be also needed, as specified in the corresponding clause for each analytics ID.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: none

### Y.4.8	Nnef_VflNwdafClient_InferenceUnsubscribe service operation

Service operation name: Nnef_VflNwdafClient_InferenceUnsubscribe

Description: Unsubscribe to VFL inference.

Inputs, Required:

-	Subscription Correlation ID.

-	temporary NWDAF ID

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

### Y.4.9	Nnef_VflNwdafClient_InferenceNotify service operation

Service operation name: Nnef_VflNwdafClient_InferenceNotify

Description: Notify VFL inference result.

Inputs, Required:

- Notification Correlation Information: this parameter indicates the Notification Correlation ID that has been assigned by the consumer during analytics subscription.

- temporary NWDAF ID

Inputs, Optional:

- intermediate local inference results

-	Timestamp of intermediate result generation (required when ADRF is deployed).

-	Validity period.

-	Confidence

-	Client Analytics Metadata Information.

-	Termination Request: this parameter indicates that NWDAF requests to terminate the VFL inference subscription, i.e. NWDAF will not provide further notifications related to this subscription, with cause value (e.g. NWDAF overload, UE moved out of NWDAF serving area, etc.).

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

### Y.4.10	Nnef_VflNwdafClient_InferenceRequest service operation

Service operation name: Nnef_VflNwdafClient_InferenceRequest

Description: The consumer requests the NWDAF to perform a one-time VFL inference.

Inputs, Required:

- Analytics ID as defined in Table 7.1-2,

-	Target of Analytics Reporting,

-	VFL model Correlation ID

-	Vendor-specific featore information

-	temporary NWDAF ID

Inputs, Optional:

-	Analytics Filter Information;

-	Preferred level of accuracy;

-	Reporting Thresholds;

-	Maximum number of objects requested (max);

-	Maximum number of SUPIs requested (SUPImax);

-	Time when information is needed;

-	Analytics Metadata Request;

-	Data time window;

-	Consumer NF's serving area or NF ID;

Outputs, Required: If the request is accepted, then intermediate local inference results. When the request is not accepted, an error response.

Outputs, Optional:

-	Timestamp of intermediate result generation (required when ADRF is deployed).

-	Validity period.

-	Confidence

-	Client Analytics Metadata Information.

NOTE 1:	Other input parameters specific for different analytics ID can be also needed and are left to the impementation.

End of changes