3GPP TSG SA WG2 Meeting #166                                                     	S2-2411484

Orlando, USA, 18 - 22 Nov, 2024

Source:	Ericsson

Title:	Discussion of IMS Disaster Prevention and Restoration Enhancement  CT TR 29.866

Document for:	Discussion

Agenda item:	19.50

Abstract: Incoming LS is in C4-244485, this paper proposes input to SA2 related work and potential direction to CT WGs before proceeding to normative work.

# 1. Introduction

CT concluded a study documented in TR 29.866, https://www.3gpp.org/ftp/Specs/archive/29_series/29.866/29866-120.zip on IMS restoration prevention and restoration enhancement that included several key issues. The proposed solutions and the conclusion raise some issues for consideration from overall architecture perspective.  They are detailed below as potential input to CT WGs work progress.

# 2. Discussion

TR 29.866 identified the following four key issues to be addressed by the study and attached report:

Key Issue #1: Continue service for registered users in case of HSS/UDM failure or overload.

Key Issue #2: Handle all PCRF/PCF instances failure scenario with minimum impact to the registered IMS users.

Key Issue #3: Speed up the restoration procedure of the terminating session when EPC/5GC NF fails.

Key Issue #4: Preventing the IMS disaster in the HSS overload scenario.

As can be seen in the table below extracted from the report, several solutions have been proposed for each key issue.

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| 1 | X |  |  |  |
| 2 |  |  |  | X |
| 3 | X |  |  |  |
| 4 | X |  |  |  |
| 5 |  | X |  |  |
| 6 |  |  | X |  |
| 7 |  |  | X |  |
| 8 |  |  | X |  |
| 9 |  |  | X |  |
| 10 |  |  |  | X |

An evaluation of the conclusion from the report lead to the following observations:

Observation 1: For KI#1, solutions 1, 3 and 4 are proposed and TR proposes in clause 8.1 to standardize all of them. Evaluation of solutions is provided within clause 7.1.

Solution 1 proposes two procedures the "IMS re-registration procedure for routing without I-CSCF" and the "IMS re-registration procedure for routing with I-CSCF", and clause 6.4 indicates "The method of routing without I-CSCF deviates from the signaling flows specified in 3GPP TS 23.228 [3], so this approach needs to be discussed in Stage 2.

Additionally, it is also questionable if there is a need to standardize all solutions, and how can they co-exist as they all attempt to solve the same problem.

Observation 2:  For KI #2 only solution #5 is proposed to be standardized. It introduces additional requirements to the PGW-U/UPF, PGW-C/SMF, P-CSCF and IMS-AGW and will increase the system overhead of these NFs. Additionally, as listed in clause 6.5.4 "This solution does not support scenarios where there is a need to interact with the PCRF/PCF, e.g., for Network Provided Location Information as specified in 3GPP TS 23.203 [10] and 3GPP TS 23.503 [7], or to get the roaming status of the UE, or to modify the QoS parameters of the media session, or build the new dedicated bearer(s) after the media re-negotiation." It needs to be analyzed if from overall architecture perspective, SA2 would be fine with such limitations, and whether supplementary actions are required by various nodes when restoration is complete.

Observation 3: For KI#3, selected solution 6 and 9 in the conclusion in clause 8.3 are contradictory when it comes to the health check triggered by the P-CSCF.  Solution 6 uses a session-less approach with the PCF, while solution 9 uses a session-based approach. We believe solution 9 is the right solution and should be the one considered. The P-CSCF interaction with the PCF is session based and there is no reason to change that.

Observation 4:  For KI #4 "Preventing the IMS disaster in the HSS overload scenario" solutions 2 and 10 are proposed. For solution 2 clause 6.2.2 indicates "Compared to procedures defined in 3GPP TS 23.228 [3], some enhancements to the S-CSCF are needed for the optimization" and clause 8.4 "Updates of S-CSCF in 3GPP TS 23.380 [2], and collaborate with SA2 for updating 3GPP TS 23.228 [3] and 3GPP TS 24.229 [6]." So again, SA2 needs to analyze the impacts.

# 3. Conclusion & proposal

Given that CT WGs are stage 2 owner for restoration procedures and also responsible for such a study, they are  seeking  SA2 input on all recommended solutions due to the nature of the potential impacts. And some solutions potentially change the IMS architecture and procedures which will require updates to TS 23.228.  SA2 thus must undertake the necessary analysis of the conclusions in the report before sending  response to CT and in the meantime requests CT WGs to hold off progressing into any normative work.