3GPP TSG-SA WG2#166	S2-2411522

Orlando, FL,USA, 18-22 November 2024	(was S2-2409916)

Source:	Qualcomm Incorporated

Title:	Discussion on the max number of connected non-3GPP devices per UE

Document for:	Discussion/Approval

Agenda Item:	19.8.2

Work Item / Release:	UIA_ARC

Abstract of the contribution: This contribution analyzed the potential requirement and risks on the restriction of maximum number of connected non-3GPP devices per UE, and proposed a way forward.

# 1.	Discussion

Following conclusion on KI#4 is agreed in TR23.700-32:

-	The optional restriction within the 5GS on max number of simultaneously active Device Identifiers per UE/5G-RG for device whose traffic need to be differentiated will be handled at the normative stage.

Any restriction of this kind in order to become a “payable service” e.g. someone to pay more to have 10 active device identifiers vs. 5 has to make sense from end use user perspective.

The concept of a non-3GPP device being active is though ambiguous, because the device will often send and receive data in the background without the user noticing them as a “service”. On the UE/5G-RG, it is also not clear how to enforce the restriction of active non-3GPP devices since only expected control plane interaction is possibly when allocating (private) IP address using DHCP and the typical lease time of IP address allocation is significantly longer than any service duration e.g. 24h or longer.

It is also not clear from user point view how he/she will understand what it means by “device being active” since the service termination may not be associated entirely with the time the corresponding dedicated QoS Flow will be active. From user’s perceptive, it’s very difficult to understand the policy e.g., a user has subscribed for 10 devices for QoS differentiation, but only 5 of them are allowed to be connected simultaneously. For example, a user has subscribed a XR device with specific QoS treatment, but when the user initiates a XR game he isn’t able to receive the expected service quality due to the restriction of max number of simultaneously active Device Identifiers. It’s too complex for a user to identify which subscribed device is currently served with QoS differentiation, which is not, and even harder for an end user to manage the connected devices to comply with this kind of quantitative restriction. In general, that restriction leads poor user experiences against the service provided by MNO.

Observation 1: The max number of simultaneously active Device Identifiers is not enforceable since the duration that a dedicated QoS Flow corresponding to a non-3GPP device will be active is not deterministic.

Observation 2: The restriction of max number of simultaneously active Device Identifiers may cause poor user experiences from the service provided by MNO for QoS differentiation of non-3GPP device traffic.

# 2.	Proposal

It is proposed to agree the following principles:

-	Simplify the restriction from “max number of simultaneously active Device Identifiers” to “max number of subscribed Device Identifiers”.

-	5GS doesn’t need to perform the enforce the restriction in real time e.g., during PDU Session Establishment/Modification.

-	The “max number of subscribed Device Identifiers” is statically configured in UDR, and enforced by UDR when a new non-3GPP Device Identifier is added in the UE subscription.

More details of the proposal is available in the CR (S2-240xxxx).