3GPP TSG-SA2 Meeting #166	 S2-2411567

# 18 – 22 November 2024, Orlando, USA

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR |  | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to the procedures of data collection for LMF-based AI/ML Positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CEWiT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR intends to add more clarity to the procedures of data collection for LMF-based AI/ML Positioning. The clarity to the procedure is provided by adding detailed description of procedures, corrections in call flow and some editorial changes |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes are as follows: - In 6.x.2 data collection trigger conditions are specified; thus the corresponding Editor note is removed. - In 6.x.3 to align with the general representation of NF discovery and selection, the figure 6.x.3-1 is updated. - Editorial corrections in multiple clauses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedure for data collection for LMF based AIML positioning would lack clarity. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x.1, 6.x.2, 6.x.3, 6.x.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 6.X	Procedures of data collection for LMF-based AI/ML Positioning

### 6.x.1	General

This clause describes the procedure for data collection by the LMF for input data to train LMF-based AI/ML models for Direct AI/ML positioning. The procedures may be used for AI/ML model performance monitoring as well.

### 6.x.2	Data Collection at LMF to train or monitor the LMF-based AI/ML positioning model to perform positioning based on UE measurements.

The LMF needs to obtain input data for LMF-based AI/ML Positioning model training and may also request the UE to provide the UE location. The LMF-based AI/ML Positioning model is trained to perform UE positioning for UEs located in an area of interest that may expand over multiple TA or cover multiple NG-RAN nodes.

Figure 6.X.2-1: Data collection by LMF to train the LMF-based AI/ML Positioning model using UE measurements

1.	The LMF determines that data collection is required e.g. to train an AI/ML Model for UE positioning. The trigger for the data collection is up to the LMF implementation.

2.  The LMF starts data collection for the purpose to train an AI/ML Model for UE Positioning (this may be based on an internal trigger in the LMF or a request from the NWDAF). The LMF subscribes to AMF to retrieve the list of SUPIs located in an area of interest and the UE Positioning Capability for each UE using Namf_EventExposure_Subscriber_Request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest" and optionally “UE Positioning Capability” and “UE User Plane Positioning Capabilities”).

3.	The AMF send Namf_EventExposure_Subscriber_Response ("list of SUPIs in the area of interest"). If “UE Positioning Capability”  and/or “UE User Plane Positioning Capabilities” are requested in step 1, AMF includes UE Positinong Capabiliy, UE User Plane Positioning Capabilities if available for each SUPI in the response message sent to LMF.

4.	The LMF further determines the UEs from the list of SUPIs that received from AMF in step 2 for data collection to train the AI/ML based positioning model based on UE Positioning Capability, UE User Plane Positioning Capabilities, the PRU information available in the LMF, and operator’s policy. Further, for each determined UE, the LMF checks with UDM, the user consent for data collection. Alternatively, if the UE has PRU functionality, the LMF may skip the user consent check.

Editor's note:	Further details on user consent for data collection for model training for LMF-based AI/ML Model positioning will be aligned with SA WG3.

5.	For each UE that provides consent to data collection for model training and can support data collection the LMF initiates a request for input data.

Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the UE is FFS. The procedure will be coordinated with RAN WGs.

Editor's Note:	The input data used for AI/ML based positioning will be decided by RAN WGs.

Editor’s Note:	Conditions for the UE to report measurement data are FFS

Editor’s Note:	Data collection for performance monitoring is FFS

At the time the LMF decides that the input data for a UE are not needed, e.g. the user consent for data collection for model training is revoked, or UE is out of the AoI, the LMF stops to collect the input data from the UE to train the LMF-based AI/ML Positioning model.

# * * * * Next change * * * *

### 6.x.3	Data Collection at LMF to train or monitor the LMF-based AI/ML positioning model to perform positioning based on NG-RAN measurements.

The procedure for data collection from NG-RAN for LMF-based AI/ML positioning is used to e.g. train the AI/ML Model.

Figure 6.x.3-1: Data collection by LMF to train the AI/ML based positioning model using NG-RAN measurements

- The LMF determines that data collection is required e.g. to train an AI/ML Model for UE positioning.

- The LMF may know the SUPIs of these UEs, e.g. when training a AI/ML model for a PRU UE, in this case step 5 follows. If the LMF does not know the SUPIs of those UEs, the LMF try to get the list of SUPIs from the AMF, before that the LMF finds the AMFs via NRF using Nnrf_NFDiscoveryRequest that may include a list of TAIs to identify a more concrete list of AMFs.

- For each AMF serving a specific area. the LMF subscribes to retrieve the list of SUPIs located in an area of interest using Namf_EventExposure_Subscriber_Request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest" and optionally “UE Positioning Capability”).

- The AMF send Namf_EventExposure_Subscriber_Response or Namf_EventExposure_Notify ("list of SUPIs in/out the area of interest").

For each SUPI provided by the AMF that is in the area of interest then steps 5 to step 6 are performed.

- The LMF checks whether the SUPI provided user consent for data collection for the purpose of training with UDM, alternatively if the UE has PRU functionality, the LMF does not need to check user consent. If consent is provided, then step 6 follows. if user consent is not provided, the LMF does not perform step 6 for this UE, and the UE will not be part of the data collection, and its data is not used for the LMF-based AI/ML Model positioning.

Editor's note:	Further details on user consent for data collection for model training training for LMF-based AI/ML Model positioning will be aligned with SA WG3.

- The LMF initiates a request for input data towards the NGRAN for the UEs.

- The LMF receives ground truth data from the UE.

The measurements and ground truth data are used for training and the ground truth data is used for performance monitoring.

The LMF may initiate data collection for multiple UEs simultaneously, as such step 6 may occur in parallel for a number of SUPIs as determined by the LMF.

Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the NG-RAN subject to RAN WG progress and feedback.

# * * * * Next change * * * *

### 6.X.4	Data collection by NWDAF for AI/ML positioning ML model training or ML model performance monitoring

The NWDAF containing MTLF may subscribe to input data (i.e. location measurement related data) from LMF for ML model training or ML model performance monitoring for LMF-based AI/ML Positioning.

Editor's note: 	It’s FFS how to adapt this procedure to ML model performance monitoring.

Figure 6.X.4-1: Procedure of input data collection from LMF

1.	NWDAF containing MTLF determines to train a ML model for LMF-based AI/ML Positioning based on the request from LMF or internal trigger, or the NWDAF containing MTLF determines to perform ML model performance monitoring for LMF-based AI/ML Positioning.

2.	If the request to train the ML model for LMF-based AI/ML Positioning is from the NWDAF internal trigger, then, the NWDAF invokes an Nnrf_NFDiscovery_Request service operation to an NRF to discover an LMF, the service operation includes an AoI and the Nlmf_DataExposure service as discovery parameters. If the NWDAF wants to collect the input data of PRUs, the NWDAF may also include a PRU existence indication for discovering the LMF(s) associated with PRUs (the PRU association procedures are defined in clause 6.17). The NRF selects one or more LMFs based on the AoI, the Nlmf_DataExposure service, and the PRU existence indication (if available), and sends an Nnrf_NFDiscovery_Request Response which includes the profiles of the selected LMFs to the NWDAF.

If the request is from the LMF, then the NWDAF does not perform step 2.

Editor's note: 	The input data used for LMF-based AI/ML Positioning will be decided by RAN WGs.

3.	The NWDAF subscribes to or cancels subscription to input data from LMF by invoking Nlmf_DataExposure_Subscribe service operation. The NWDAF includes an AoI and a notification target address to request the input data from LMF. The NWDAF may also include requested number for data samples and time window for data collection.

To cancel the subscription, the NWDAF uses Nlmf_DataExposure_UnSubscribe service operation.

Editor's note:	Whether the NWDAF needs to be aware of PRU information and indicate to obtain the PRU information from LMF is FFS. Whether the NWDAF needs to indicate Data source type (UE or RAN measurements) and/or data type(s) and/or a target ground truth label precision is also FFS.

4.	The LMF performs steps 1 to 5 in clause 6.X.2, or steps 1 to 7 in clause 6.X.3.

5.	The LMF sends the collected data samples to the NWDAF by invoking Nlmf_DataExposure_Notify service operation, then the NWDAF trains the ML model or performs ML model performance monitoring based on the data samples.

# * * * * End of Change * * *