3GPP TSG-SA2 Meeting #166	S2-2411589

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024				   (revision of S2-2411011)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1489 | rev | 4 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network Capability Exposure Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Qualcomm, Samsung, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 8.2 in 3GPP TS 23.700-77 concludes KI#2 on "Impact on IMS architecture, interfaces and procedures to support IMS capability exposure in the context of IMS data channel session". The corresponding normative work need to be implemented. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the procedures of IMS data channel capabilies exposure. Rev-4: 1. Resolving the EN: “Whether and how to have intermediate call notification during the procedure is FFS”. In the current procedure, the DC AS needs to wait until both UEs have accepted or rejected the session (or session modification) until it receives the response to the request (HTTP 2xx). It is proposed to add a subscribe/notify operations under the Nimsas_IMSSessionManagement service, so that the response indicating whether the request is accepted or rejected can be provided immediately, and the actual IMS session progress with the UEs is reported via notify service operation. In this way, the NEF uses the callback URI to find the IMS AS, and discovery from HSS is not needed, and the EN “the service for step 2 is FFS” on how to discover the IMS AS can be removed. 2. The current procedure refers to OMA API that can be used to notify the non-DC related IMS session events to the DC AS. The OMA API needs to provide a callback URI that the DC AS can use to target the update-request to the same IMS AS that send the notification (and who is anchoring the IMS session). It should be noted that OMA API cannot be used to notify for DC-related events; the item 1 in this CR is used for DC-related events. 3. Resolving the EN “The need for a new event in step 4 is FFS”: it is proposed to re-use the existing Nimsas_SessionEventControl_Notify service operation. The notify request must be enhanced to include the Media information set that the IMS AS has received in Nimsas_ImsSessionManagement_Create or _Update request from the NEF, and new Event IDs: “3rd party session update” and “3rd party session creation”. The media information set indicates to the DCSF the DC type (P2A, P2A2P, P2P, or BDC) and e.g. the MDC2 media end point address if a new DC is added with the DC AS. 4. It is proposed to merge the procedures for adding of data channels and removing of a data channels. 5. Editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No normative specification for conclusions of KI#2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.2.4.2, Annex XX (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# Annex XX (normative): IMS Capability Exposure Framework in IMS

## XX.1	Description

Two scenarios utilizing IMS data channel capability exposure services are depicted in XX.2:

-	Adding data channel to an existing IMS session, for which Nimsas_SessionManagement_Update is used

-	Creating a standalone data channel session, for which Nimsas_SessionManagement_Create is used.

Editor’s Note:	The procedures need to further align with the final parameter design of the service used in those procedures.

## XX.2		IMS data channel capability exposure procedures

### XX.2.1	Adding or removing Data Channel to an Existing IMS Session

Figure XX.2.1-1 shows the procedure of adding bootstrap data channel(s) or application data channel(s) to an existing IMS session.

Figure XX.2.1-1: Procedure of adding data channel(s) to an existing IMS session

0.	DC AS may determine to add or remove data channel to an existing IMS session based on local policy or event notification of the related IMS Session which DC AS subscribes to.

NOTE 1:	It is assumed that the OMA API can be used for non-DC session notification in step 0.

1.	The DC AS sends a Nnef_IMSSessionManagement_Update request to the NEF requiring to add or remove bootstrap data channel(s) or application data channel(s) in the exisiting IMS session.

The DC AS may include a subscription to the DC events to the Nnef_IMSSessionManagement_Update request to be notified for the progress of the session update.

NOTE 2: It is assumed the OMA API provides a callback URI in the notification in step 0 that is included to the Nnef_IMSSessionManagement_Update request to address the target IMS AS.

2.	The NEF discovers the IMS AS from NRF if needed.

3.	The NEF sends a Nimsas_IMSSessionManagement_Update request to the IMS AS-2 requiring to add or remove bootstrap data channel(s) or application data channel(s) in the specific session.

4.	The IMS AS-2 validates the user subscription data and determines whether the data channel request should be notified to DCSF-2. If the serving UE does not have subscription of IMS data channel, then the request shall be rejected with appropriate cause and skip the following step. If the IMS AS-2 allows the request to proceed, it returns a successful Nimsas_SessionManagement_Update response to the NEF.

5. 	The NEF returns a successful Nnef_SessionManagement_Update response to the DC AS.

6.

The IMS AS-2 sends a Nimsas_SessionEventControl_Notify request to the DCSF-2 with the indication of adding or removing data channel(s). The notification includes the Media operation set as received in Step 3

The DCSF-2 instructs the IMS AS-2 to set up or remove the data channel(s) by sending a Nimsas_MediaControl request. Depending on the instruction from DCSF-2, the IMS AS-2 may interact with the MF-2 to allocate data channel resources. The procedures depicted in clause AC.7 are reused for interaction with MF.

NOTE 2:	The kind of data channel established is dependent on the instruction given by DCSF.

7.	The IMS AS-2 generates a re-INVITE request(s) in which the SDP offer contains the media information of the data channel required by the DC AS with the other existing media descriptions.

The IMS AS-2 sends the re-INVITE request(s) according to the instructions received from DCSF-2 in Step 4.

NOTE 3: The IMS DC session update procedures described in clauses AC.7.1 and AC.7.2 are followed.

8.	The DC media negotiation is completed and corresponding data channel(s) are established.

9. 	The IMS AS notifies the DCSF for the IMS session events with the Nimsas_SessionEventControl_Notify service operation as described in clause AC.7.

If the DC AS included a subscription to the DC session events to the Nnef_IMSSessionManagement_Update request in Step 1, the IMS AS notifies the NEF for the progress of the session update with the Nimsas_SessionManagement_Notify service operation.

10.	The NEF notifies the DC AS for the progress of the session update with the Nnef_SessionManagement_Notify service operation.

### XX.2.2	Establishing IMS Session with Standalone Data Channel

Figure XX.2.2-1 shows the establishment procedure of a standalone data channel session.

Figure XX.2.2-1: Establishment procedure of a standalone data channel session

0.	The DC AS decides to establish a standalone data channel session.

1.	The DC AS sends a Nnef_IMSSessionManagement_Create request to the NEF requiring to create a standalone DC session with UE A.

The DC AS may include a subscription to the DC events to the Nnef_IMSSessionManagement_Create request to be notified for the progress of the session update.

2.	The NEF queries the HSS for the address of the IMS AS serving the service number of the enterprise if needed. If the service number is unregistered, that is, the HSS fails to return the address of IMS AS, the NEF shall reject the Nnef_ImsSessionManagement_Update request from the DC AS with an error response and the following steps shall be skipped.

3.	The NEF sends a Nimsas_SessionManagement_Create request to the IMS AS-2 requiring to create a standalone DC session.

4.	The IMS AS-2 validates the user subscription data and determines whether the data channel request should be notified to DCSF-2. If the service number does not have subscription of IMS data channel, then the request shall be rejected with appropriate cause. If the IMS AS-2 allows the request to proceed, it returns a successful Nimsas_SessionManagement_Create response to the NEF.

5. 	The NEF returns a successful Nnef_SessionManagement_Create response to the DC AS.

6.

The IMS AS-2 sends a Nimsas_SessionEventControl_Notify request to the DCSF-2 with the indication of creating a data channel. The notification includes the Media operation set as received in Step 3.

The DCSF-2 instructs the IMS AS-2 to set up the data channel by sending a Nimsas_MediaControl request. The IMS AS-2 interacts with the MF-2 to allocate data channel resource. The procedures in clause AC.7 are reused to interact with MF.

NOTE 1:	The kind of data channel established is dependent on the instruction given by DCSF.

7.	The IMS AS-2 generates an INVITE request(s) in which the SDP offer contains the media information of the data channel(s) required by the DC AS.

The IMS AS-2 sends the INVITE request(s) according to the IMS DC session establishment procedures described in clauses AC.7.1 and AC7.2.

8.	The DC media negotiation is completed and corresponding data channel are established.

9. 	The IMS AS notifies the DCSF for the IMS session events with the Nimsas_SessionEventControl_Notify service operation as described in clause AC.7.

If the DC AS included a subscription to the DC session events to the Nnef_IMSSessionManagement_Create request in Step 1, the IMS AS notifies the NEF for the progress of the session update with the Nimsas_SessionManagement_Notify service operation.

10.	The NEF notifies the DC AS for the progress of the session update with the Nnef_SessionManagement_Notify service operation.

# * * * * Next change * * * *

### AA.2.4.2	Nimsas_SessionEventControl

#### AA.2.4.2.1	General

Service description: This service enables the consumer to be notified about session events when served IMS subscribers take part in IMS sessions.

The following operations are available for this service:

-	Explicit subscription to receive session events. This is not specified in this Release.

-	Notifying IMS session control events of a specific IMS subscriber to NFs, e.g. DCSF, based on local configuration in the IMS AS.

IMS AS processing of the session event is paused until the consumer acknowledges reception of the session event notification. The consumer is given the possibility to apply session media control and policies by invoking the Nimsas_MediaControl service before giving IMS AS a session event notification acknowledgement.

If the consumer does not apply media control/policies to a session event, IMS AS continues processing and forwards the event as is (including the media state) to calling/called party.

#### AA.2.4.2.2	Nimsas_SessionEventControl_Notify service operation

Service operation name: Nimsas_SessionEventControl_Notify

Description: This service operation enables IMS AS to notify consumers of session events related to a specific served IMS subscriber requesting use of IMS data channel media.

Inputs, Required: Session ID, Event ID,

Session ID is the identity of the IMS session for which the event relates to.

Event ID is the event triggered within the IMS session.

Inputs, Optional: Calling ID, Called ID, Session case, Event initiator, Media info list, Media information set.

Calling ID is the public identity of the calling IMS subscriber. Called ID is the public identity of the called IMS Subscriber. Session case indicates if this is an originating or terminating IMS session. Event initiator indicates initiator of the event, i.e. 'served IMS subscriber' vs 'remote IMS subscriber'. 

Media info list includes for each media in the list:

-	media ID: uniquely identifies this media item within the list. The identity is allocated by IMS AS,

-	media specification: This depends on media type including relevant media attributes of interest to the consumer. The media specification includes the following media description attributes:

-	Media Type: DC, Audio, or Video.

When the media type is "DC", the elements below are derived from the SDP received by the IMS AS in an SIP INVITE or a re-INVITE related to an IMS Data Channel and the corresponding DTLS connection.

-	Data Channel Mapping and Configuration Information: This attribute is applicable to Data Channel and includes relevant configuration Information, including the stream ID and application binding information of the Data Channel.

-	Maximum Message Size: This attribute defines the maximum size to be expected.

-	Data Channel Port: This attribute identifies the port for the Data Channel.

-	Security Setup: This attribute identifies the security set up of the DTLS connection.

-	Security Certificate Fingerprint: This attribute identifies the security certificate fingerprint.

-	Security Transport Identity: This attribute identifies transport layer identity.

Media information set:

-	For Event ID "3rdPartySessionCreation", the media information set is described in AA.2.4.4.2.

-	For Event ID "3rdPartySessionUpdate", the media information set is described in AA.2.4.4.3.

Outputs, Required: Result indication.

Outputs, Optional: None.

The table below presents supported EventIDs and related parameters.

Table AA.2.4.2.2-1: List of events and Related Optional parameters

| EventID | Parameters |
| --- | --- |
| SessionEstablishmentRequestEvent | Calling ID, Called ID, Session case, Event initiator, Media info list |
| SessionEstablishmentProgressEvent | Media info list |
| SessionEstablishmentAlertingEvent | Media info list |
| SessionEstablishmentSuccessEvent | Media info list |
| SessionEstablishmentFailureEvent |  |
| MediaChangeRequestEvent. | Event initiator, Media info list, Calling ID, Called ID, Session case |
| MediaChangeSuccessEvent | Media info list |
| MediaChangeFailureEvent | Media info list |
| SessionTerminationEvent | Session case |
| 3rdPartySessionUpdate | Media information set |
| 3rdPartySessionCreation | Media information set |

# * * * * End of changes * * * *