3GPP TSG-SA2 Meeting #	S2-2411644

# 18th – 22nd November 2024, Orlando USA                         was S2-2411194

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1134 | rev | 15 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General training procedure for Vertical Federated Learning between NWDAF(s) and AF(s) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [China Mobile, CATT, ZTE, OPPO, China Telecom, ETRI, Huawei, HiSilicon, Nokia, Futurewei, Samsung], Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-11-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The paper adding procedure of Vertical Federated Learning when NWDAF is acting as VFL server |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new clause for procedure of Vertical Federated Learning when NWDAF is acting as VFL server Changes in rev 15: Solves following EN: Editor’s Note: Further extensions are needed to show the interaction between consumer and VFL server. For example, how the consumer (i.e., NWDAF containing AnLF) sends a subscription request to VFL server. Adds untrusted AF as server in new procedure Aligning with definition and other clauses and use the term Intermediate results. The name may be further discussed during meeting. Since the service names will be discussed in coming meeting, uses the generic service name Nnf_VFLOperation while adding a new EN: #### Editor’s note: The service operation names used for VFL operations in the procedures below are FFS (The CR needs to be correctly formatted before final approval) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL training procedure not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.3 (new), 6.2H.2.3.1 (new) and 6.2H.2.3.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | CRs shall be implemented in the following order: CR 1198, CR 1246, CR1235, CR1208, CR1134 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

#### 6.2H.2.3 Training Procedure for Vertical Federated Learning

##### Editor’s note: The service operation names used for VFL operations in the procedures below are FFS

#### 6.2H.2.3.1	Training Procedure for Vertical Federated Learning when NWDAF is acting as VFL server

The figure 6.2H.2.3.1-1 below shows the training procedure for Vertical Federated Learning when NWDAF is acting as VFL server.

Figure 6.2H.2.3.1-1Training procedure for Vertical Federated Learning when NWDAF is acting as VFL server

Editor’s Note: Whether and how to maintain a Vertical Federation Learning process including dynamical reselection, addition, or removal of VFL Client NWDAF(s) is FFS.

Editor's Note:  How the NEF assists the VFL training process as well as whether the service operations going via NEF is using the existing or new service operation are FFS.

Editor’s Note: The details of the services in the procedure and whether VFL Training Start Flag is needed are FFS.

Editor’s Note: It is FFS whether sample/feature information is required to be provided in each training.

Editor´s Note: Whether and how to include interoperability information in the VFL training procedure is FFS.

Editor’s Note:	Terminology will be aligned in the future.

0.	OPTIONAL If the first NWDAF containing both AnLF and MTLF and can be the VFL server, then step 0 is skipped.

If the first NWDAF is an NWDAF only containing AnLF and does not have a model, it sends a subscription request for a model to NWDAF containing MTLF using Nnwdaf_MLModelProvision_Subscribe including Analytics ID, optionally Target of Analytics Reporting = e.g. UE IDs.

Editor’s note: the following case (steps 0a and 0b) is FFS and shall be removed if trusted AF is to not be included in this clause. If NWDAF containing MTLF does not have a model and realizes it cannot be VFL server, the NWDAF sends a subscription request for a model to VFL server AF using Naf_VFLOperation_Subscribe including Analytics ID, optionally Notification target set to the first NWDAF, optionally Target of Analytics Reporting = e.g. UE IDs.

If decision is triggered on request from an NWDAF, the NWDAF containing MTLF sends in the service subscription response that no model will be available to download.

Either based on the information received, operator policy or internal configuration, VFL server decides to initiate VFL inference procedure with VFL clients.

1.	The NWDAF acting as VFL server determines the VFL clients that participate in VFL procedure in the VFL clients discovery and preparation phase as described in the clause 6.2H.2.1 and clause 6.2H.2.2.

2.	To start VFL training, the VFL server allocates VFL correlation ID. The VFL server sends a request to start the VFL training to each of the selected VFL clients. The VFL server NWDAF sends a Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request to the selected NWDAF clients(s) and Naf_VFLOperation_Subscribe to the selected AF clients(s). The request includes VFL correlation ID.

Editor´s Note: Additional Parameters to be provided in the request are FFS.

Editor´s Note: It is FFS whether and how the local ML model is obtained by VFL Client in VFL training process.

Editor’s Note:	It is FFS which service is used between VFL server and VFL clients for VFL training.

The steps 3-7 are repeated until the training termination condition is reached.

3.	[Optional] Each VFL client collects its local data by using the current mechanism if the VFL client has no local data already available.

4.	During VFL training procedure, each VFL client further trains the local ML model associated with the same VFL Correlation ID based on their own collected or available data and if available based on Server Intermediate results distributed by the VFL server in the previous training iteration, and computes and reports the client intermediate training result of the local ML model to the VFL server.

Editor’s note:	The report may include ID(s) of sample(s) corresponding to the intermediate training result.

Editor´s note: The following is FFS and may depend on the service design: When the clients reports the client intermediate training result, it also includes the corresponding VFL correlation ID.

5.	The VFL server may collect the local data and generate its own local intermediate training result. The NWDAF acting as VFL Server computes the Server Intermediate results (e.g. gradient information or loss information) based on the client intermediate training result received in step 4, its own local intermediate results and the label. The Server Intermediate results is used for updating the models of VFL clients. Different Server Intermediate results may be computed for different VFL clients, respectively.

The VFL server may also compute the ML model metric (e.g. ML model accuracy) based on all the intermediate training result received from VFL clients and the label.

Editor’s Note: Whether weight of feature is computed by VFL server is FFS.

Editor’s Note: Whether VFL server and VFL clients share feature information is FFS.

6.	[Optional] The NWDAF acting as VFL server evaluates (e.g., based on the convergence of a loss function or loss value, the pre-set iteration number is reached) whether VFL Training process converged. If not, the NWDAF acting as a VFL Server determines another round of VFL training is required and repeats step 3 - 7. If yes, it determines the VFL Training is completed. In this case, the VFL Server terminates the current VFL training process via step 7.

The VFL training termination decision may be also made as follows:

Based on the consumer request, the VFL server sends VFL status report to the consumer. The status report may include model metric (e.g. ML model accuracy).

Editor’s note: the following case (including the NOTE) is FFS and shall be removed if trusted AF is to not be included in this clause. If the NWDAF containing MTLF requested an AF to be VFL server and it did not add the Notification target of the first NWDAF, it may in one of the repeated steps add the AF ID in the response.

NOTE: If the AF ID is not added in this step it needs to be added in time in step 8b.

Editor’s Note: The content of the VFL status report is FFS.

Editor’s Note: Whether VFL server sending convergence report to the VFL client and what is convergence report are FFS.

The consumer decides whether the current model can fulfil the requirement, e.g. ML model metric is satisfactory for the consumer and determines to stop or continue the training process. The consumer continues the training process or stops the training process.

Based on the subscription request sent from the consumer, the VFL server updates or terminates the current VFL training process.

Editor’s Note: the following is FFS. The VFL server determines with which VFL Client(s) to continue the VFL. The VFL server may provide to the VFL clients any sample ID(s) if changed.

7.	If the VFL procedure continues in subsequent iterations, the VFL server sends a request for a new VFL training iteration containing the Server Intermediate results to each of the VFL clients for next round of VFL training.

If the server decides to stop the training the VFL server sends VFL training termination message to VFL Client if it decides to terminate the VFL training process. The VFL Server and each VFL Client, stores the VFL correlation ID and latest information related to their locally trained Models

NOTE: The VFL correlation ID is used later for inference.

Editor’s Note: Whether VFL Training termination Flag in the termination message is required is FFS.

8.	At termination the VFL server communicates to the Consumer, which is either the NF requesting the model or to the Notification target set to the first NWDAF.

Editor’s Note: Possible procedures to be executed at the end of the VFL training (to terminate a training session and to store/handle trained models) are FFS.

NOTE 1:	If untrusted AF is involved in VFL Clients, the message between NWDAF acting as VFL Server and the untrusted AF is via NEF.

##### 6.2H.2.3.2	Training Procedure for Vertical Federated Learning when untrusted AF is acting as VFL server

The figure 6.2H.2.3.2-1 below shows the training procedure for Vertical Federated Learning when untrusted AF is acting as VFL server.

Figure 6.2H.2.3.1-1Training procedure for Vertical Federated Learning when NWDAF is acting as VFL server

Editor’s Note: Whether and how to maintain a Vertical Federation Learning process including dynamical reselection, addition, or removal of VFL Client NWDAF(s) is FFS.

Editor's Note:  How the NEF assists the VFL training process as well as whether the service operations going via NEF is using the existing or new service operation are FFS.

Editor’s Note: The details of the services in the procedure and whether VFL Training Start Flag is needed are FFS.

Editor’s Note: It is FFS whether sample/feature information is required to be provided in each training.

Editor´s Note: Whether and how to include interoperability information in the VFL training procedure is FFS.

Editor’s Note:	Terminology will be aligned in the future.

0.	OPTIONAL If the VFL server trigger the training, then step 0 is skipped.

If the first NWDAF is an NWDAF only containing AnLF and does not have a model, then same as step 0 in 6.2H.2.3.1.

If NWDAF containing MTLF does not have a model and realizes it cannot be VFL server, the NWDAF sends a subscription request for a model to VFL server AF using Nnef_VFLOperation_Subscribe including Analytics ID, optionally Notification target set to the first NWDAF, optionally Target of Analytics Reporting = e.g. UE IDs

NEF forwards the subscription request to AF using Naf_VFLOperation_subscribe.

Editor’s note: The role of NEF is FFS.

If decision is triggered on request from an NWDAF, the NWDAF containing MTLF sends in the service subscription response same as in step 0 in 6.2H.2.3.1.

Either based on the information received, operator policy or internal configuration, VFL server decides to initiate VFL inference procedure with VFL clients.

1.	Same as step 1 in 6.2H.2.3.1.

2.	To start VFL training, the VFL server do same as in step 1 in 6.2H.2.3.1, using Nnef_VFLOperation_Subcribe

Editor´s Note: Additional Parameters to be provided in the request are FFS.

Editor´s Note: It is FFS whether and how the local ML model is obtained by VFL Client in VFL training process.

Editor’s Note:	It is FFS which service is used between VFL server and VFL clients for VFL training.

The steps 3-7 are repeated until the training termination condition is reached.

3.	[Optional] Same as step 3 in 6.2H.2.3.1.

4.	Same as step 4 in 6.2H.2.3.1 to NEF using Nnwdaf_VFLOperation_Notify.

NEF forwards the subscription request to AF using Nnef_VFLOperation_Notify.

Editor’s note: The role of NEF is FFS.

Editor’s note:	The report may include ID(s) of sample(s) corresponding to the intermediate training result.

Editor´s note: The following is FFS and may depend on the service design: When the clients reports the client intermediate training result, it also includes the corresponding VFL correlation ID.

5.	Same as step 5 in 6.2H.2.3.1.

Editor’s Note: Whether weight of feature is computed by VFL server is FFS.

Editor’s Note: Whether VFL server and VFL clients share feature information is FFS.

6.	[Optional] Same as step 6 in 6.2H.2.3.1, to NEF using Naf_VFLOperation_Notify.

NEF forwards the subscription request to NWDAF using Nnef_VFLOperation_Notify.

Editor’s note: The role of NEF is FFS.

Editor’s Note: The content of the VFL status report is FFS.

Editor’s Note: Whether VFL server sending convergence report to the VFL client and what is convergence report are FFS

Editor’s Note: the following is FFS. The VFL server determines with which VFL Client(s) to continue the VFL. The VFL server may provide to the VFL clients any sample ID(s) if changed

7.	Same as step 6 in 6.2H.2.3.1, to NEF using Nnef_VFLOperation_subscribe.

NEF forwards the subscription request to AF using Naf_VFLOperation_subscribe.

Editor’s note: The role of NEF is FFS.

NOTE: The VFL correlation ID is used later for inference.

Editor’s Note: Whether VFL Training termination Flag in the termination message is required is FFS.

8.	At termination the VFL server communicates to the Consumer, which is either the NF requesting the model or to the Notification target set to the first NWDAF.

Editor’s Note: Possible procedures to be executed at the end of the VFL training (to terminate a training session and to store/handle trained models) are FFS.

# >>>>END OF CHANGES<<<<