# 3GPP TSG-WG2 Meeting #166	S2-2411669

# Orlando, USA, 2024-11-18 – 2024-11-22

Title:	[Draft] LS on feedback on AIoT progress and interim conclusions

Response to:	-

Release:	Rel-19

Work Item:	FS_AmbientIoT

Source:	SA2

To:	RAN2, RAN3

Cc:	SA3

Contact person:	 TBD

Send any reply LS to:		3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TR 23.700-13 (link)

# 1	Overall description

SA2 has started Interim conclusion phase for the study of Ambient IoT. These interim conclusions are documented in TR 23.700-13.

SA2 understands that the final selection of the overall architecture requires joint efforts with RAN2 and RAN3 and has identified aspects about A-IoT session resource setup information in the Architecture to Support Topology 2. SA2 assumes A-IoT-enabled gNB needs such information to trigger the preparation of the radio resource towards A-IoT-enabled UEs.

There are several options identified on how the A-IoT session resource control information can be delivered to A-IoT-enabled gNB:

- RRC-based option (Direct Path and Indirect Path options): the A-IoT session resource control information is delivered over NGAP from the CN.

- User-plane option: the A-IoT session resource control information can be delivered via a separate N2 request from CN, or in extended GTP-U headers along with the A-IoT upper layer information from CN. As an alternative, UE can request radio resource but gNB does not have A-IoT session resource control information.

- NAS-based option: the A-IoT session resource control information can be delivered via a separate N2 request from CN, or a N2 request along with the NAS message from CN As an alternative, UE can request radio resource but gNB does not have A-IoT session resource control information.

SA2 has decided not to pursue the options of using extended GTP-U headers in user-plane option, and the option of UE request in user-plane option and NAS-based option.

SA2 kindly asks RAN2 and RAN3 to study these options and provide feedback.

SA2 also identified the needs of introducing an indicator in A-IoT paging for inventory, if the enable command is delivered in “inventory and command” manner. This enables addressing stage 1 requirements to be able to temporarily disable and enable AIoT devices. The indicator is to request temporarily disabled devices to respond A-IoT paging to perform the device ID transmission.

SA2 kindly asks RAN2 and RAN3 to provide feedback on the feasibility of such solution.

NOTE: Additional questions may be determined during the discussion at the meeting.

SA2 also understand the study of Ambient IoT needs to be concluded together with RAN WGs and need to be supported by security architecture by SA3.

SA2 would like to ask RAN2 and RAN3 to provide feedback on the interim conclusions in TR 23.700-13.

# 2	Actions

To RAN3

ACTION: SA2 kindly asks RAN3 to study the options and provide feedback on how A-IoT session resource control information is provided to the A-IoT-enabled gNB.

To RAN2

ACTION: SA2 kindly asks RAN2 to provide feedback on the indicator in A-IoT paging for inventory, which requests temporarily disabled devices to respond the A-IoT paging.

To RAN2, RAN3

ACTION: SA2 kindly asks RAN2 and RAN3 to provide feedback on the interim conclusions in TR 23.700-13.

# 3	Date of next TSG SA WG SA2 meetings

3GPPSA2#166-Ad Hoc-e		2025-01-20 – 2025-01-24		 	Online

3GPPSA2#167				2025-02-17 – 2025-02-21			Athens, GR