SA WG2 Meeting #166	S2-2411701

18-22, Nov 2024, Orlando, USA

Source:	OPPO, InterDigital, Nokia, Huawei, HiSilicon, ZTE, Ericsson

Title:	Discussion on URSP for PDU Establishment for non-3GPP Device behind UE/5G-RG

Document for:	Discussion

Agenda Item: 	19.8.2

Work Item / Release:	UIA_ARC /Rel-19

Abstract of the contribution: This discussion paper examines options to support associating traffic from non-3GPP devices with appropriate PDU Sessions via URSP rules, presenting detailed analysis, comparison of different approaches, and a way-forward proposal.

# Background

In the Pre-SA2#165 NWM discussion, Question 12 was raised, “When traffic comes from a non-3GPP device to the UE/5G-RG and the UE/5G-RG has bound a Device Identifier to non-3GPP device, how should the UE/5G-RG associate the traffic to a PDU Session?”

Fifteen companies responded, with all agreeing that URSP rules can be applied to manage traffic in this case. The responses are summarized in [1]:

- All companies agreed that URSP rules are viable for traffic association.

- Seven responses assumed that the Device Identifier can be used as a traffic descriptor.

- Five responses felt that no change to URSP rules were necessary.

It is further proposed by the Rapporteur to proceed with the assumption that URSP Rules are used to associate traffic which is bound to a non-3GPP device to a PDU Session. However, further discussion is needed to determine whether the Traffic Descriptor should include the Non-3GPP Device Identifier.

This discussion paper examines options to support traffic association for non-3GPP devices with appropriate PDU Sessions through URSP rules, presenting detailed analysis, comparison of different approaches, and a way-forward proposal.

# Discussion

KI#4 of TR 23.700-32 [2] concludes that QoS differentiation can be achieved within a PDU Session (using different QoS flows) or between PDU Sessions. Solutions #30, #31 and #32 documented in TR 23.700-32 provide options for enabling QoS differentiation for non-3GPP device traffic between PDU sessions by using URSP.

Solution #30 enables the 5G-RG/UE to create a new PDU session or modify an existing session to handle device data using URSP rules. This solution includes an optional extension for URSP rules like PIN, unless PIN ID can be re-used.

In solution #31, a PDU Session with a default QoS flow is established based on the URSP rule matching a Connectivity Group ID, as specified in clause 4.10b of TS 23.316 [3].

The approach described in solution #32 uses Non-3GPP Device Identifier in the URSP Traffic Descriptor, allowing it to match against the identifier associated with the non-3GPP device.

These solutions represent three main approaches for applying URSP rules to support a Non-3GPP Device Identifier, that is, reusing the Connectivity Group ID, reusing the PIN ID, or defining a new Traffic Descriptor for Non-3GPP Device Identifier. Table 1 summarizes the pros and cons of each option.

Table 1: URSP related solutions documented in TR 23.700-32

|  | Reusing Connectivity Group ID | Reusing PIN ID | Non-3GPP Device Identifier as Traffic Descriptor in URSP |
| --- | --- | --- | --- |
| Description | A PDU Session with a default QoS flow is first established based on the URSP rule matching a Connectivity Group ID. PDU session modification procedure is then used to apply appropriate QoS flow to the Non-3GPP Device Identifier. | PIN ID Traffic Descriptor is used for Non-3GPP Device Identifier. | Non-3GPP Device Identifier is provided in the Traffic Descriptor of the URSP rule, and matched against the Non-3GPP Device Identifier that the non-3GPP device is associated with. |
| Pros | Reuse existing mechanism | Reuse existing mechanism | Non-3GPP device is independent of PIN or Connectivity Group ID deployment. The solution works for all Non-3GPP Device Identifiers and all UE/5G-RGs, without dependency on PIN ID or Connectivity Group ID. |
| Cons | This option relies on the deployment of Connectivity Group ID. In the absence of the Connectivity Group ID deployment for the 5G-RG, this solution may not work. Connectivity Group ID as specified in TS 23.316 is only used for 5G-RG. Solutions relying on Connectivity Group ID does not work for UE. Connectivity Group ID is usually used to support a group of non-3GPP devices, not individual non-3GPP devices. | This option limits to UEs with PEGC roles. It does not extend to all UEs and 5G-RG, reducing applicability in mixed device scenarios. PIN ID is usually used for a PIN network, not for individual device. It is less efficient to use PIN ID in this manner to offer QoS differentiation for individual non-3GPP device. | This option defines Non-3GPP Device Identifier as an extension of URSP Traffic Descriptor. |

In summary, employing PIN ID or Connectivity Group ID for Non-3GPP Device Identifier feature restricts implementation to scenarios where PIN or Connectivity Group ID is deployed, each with specific scopes and usage requirements.

For instance, PIN ID as a Traffic Descriptor is only for PEGC UE registered in the 5G network. Hence, this approach requires that the UE operates in a PEGC role. The URSP rules tied to the PIN ID cannot be delivered to all UEs. This constraint limits applicability for UEs not configured as PEGCs.

Connectivity Group ID, on the other hand, applies only to 5G-RG, which is assigned on the 5G-RG where each Connectivity Group ID corresponds to a separate physical or virtual port on the 5G-RG. All devices that connect to that port are considered part of the same Connectivity Group ID. It is not efficient when the Connectivity Group ID being used to offer QoS differentiation for individual Non-3GPP Device Identifier.

It should be noted that UE local configuration as a fallback option is also restrictive. As specified in TS 23.503 [4], URSP rules with matching Traffic Descriptor are evaluated before locally configured policies. As a result, locally configured policies for the Non-3GPP Device Identifiers are bypassed if there are prioritized URSP rules are matched, preventing the correct PDU session from being applied.

# 2	Proposal

It is proposed to support “Non-3GPP Device Identifier” in Traffic Descriptor of URSP rules and approve the CR.

# 3	References

[1]	S2-2410203, “Pre-SA2#165 NWM discussion for progressing UIA_ARC”, https://www.3gpp.org/ftp/tsg_sa/WG2_Arch/TSGS2_165_Hyderabad_2024-10/Docs/S2-2410203.zip

[2]	TR 23.700-32: “Study on User Identities and Authentication Architecture”.

[3] 	TS 23.316: “Wireless and wireline convergence access support for the 5G System (5GS)”.

[4]	TS 23.503: “Policy and Charging Control Framework for the 5G System”.