3GPP SA WG2 Meeting #166	S2-2411752

Orlando, United States, 18th - 22nd Nov 2024

# Title:	[DRAFT] Reply LS on AIML data collection

# Response to:	RP-242389/S2-2411455 LS on AIML data collection

# Release:	Rel-19

# Work Item:	NR_AIML_Air

Source:	Samsung [to be SA2]

To:	RAN

CC:	RAN1, RAN2, SA, SA3, SA5

Contact person:	Hyesung Kim

hs1207.kim@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

1	Overall description

SA2 thanks RAN for the LS on the AIML data collection. SA2 would like to provide views on the analysis of different data collection options for UE-side model training as follows:

SA2’s view on controllability of MNO on data transfer

SA2 provides the following evaluations on controllability of each option, noting that it is not comprehensive analysis and SA2 only can provide precise input once the controllability is further clarified.

- Controllability of MNO on data transfer for Option 1a and Option 1b should be marked as “Full controllability is not possible”. Further evaluation from SA2 perspective is given as follows:

  - For Option 1a, the core network can offer a low level of controllability on data transfer. For example, the MNO can utilize the existing QoS framework and policy control for managing data transfer for AIML data collection.

  - For Option 1b, the controllability of MNO on data transfer is limited because the data goes through the UP tunnel toward the server. For such a UP tunnel path, the potential involvement of NF is restricted, especially for the server located outside of MNO domain.

  - For Option 2, the MNO can control various aspects on data transfer via CP tunnel as the data goes through the 5GC NF. The NF can be designed to request the initiation and termination of data transfer over CP from UE to the NF and decide when and whether to transfer the data, received from the UE, toward the server.

  - For Option 3, the MNO can control various aspects on data transfer via CP tunnel as the data goes through the OAM. The OAM can be designed to decide when and whether to transfer the data toward the server. However, Table 7.2.1.3.2-1 only indicates RRC procedure as solution for network controllability, which cannot cover comprehensive solution. If MNO wants to have controllability via OAM system, then its related interaction between OAM and gNB is required, which requires SA5 input.

- It should be considered for all the options that UE can decide the initiation and termination of data transfer by itself under certain UE-side conditions.

  - Regarding potential UE operation related to initiation and termination of data transfer, even though network side entity can be designed to request the initiation of data transfer to a UE, the UE can decide the initiation and termination of data transfer by itself, considering UE-side status or condition.

Based on the above observation on the controllability, SA2 would like to suggest the following updates in Table 7.2.1.3.2-1 from SA2 perspective.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| Controllability of MNO on data transfer (Note 1) | No AI/ML specific controllability Full controllability is not possible, FFS: level of controllability | Full controllability is not possible, FFS: level of controllability (Note 5) | Full controllability | Full controllability |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) Example: per PDU sessions | Example: per PDU sessions | Via NAS procedure or FFS other procedures | Via RRC procedure or FFS additional procedure, e.g., between OAM and gNB |

SA2’s view on possible options for visibility of data content in MNO and Data format

For Option 1b, full visibility of MNO on data transfer cannot be guaranteed.

For Option 2 and Option 3, the standardized data content may not be visible for the MNO, and further discussion on the condition for full visibility for MNO is needed.

SA2’s view on feasibility of UP solution for option 2 and option 3

SA2 cannot confirm at this stage if the UP tunnel is feasible for data collection with given information for Option 2 and 3. Especially for Option 3, it requires input from SA5.

SA2’s view on futureproof and extendibility

The futureproofness and extensibility can be achieved with the following principles: (i) No need for a new additional functionality and a minimal impact on existing standardized entities allowing only limited changes to them, (ii) Flexible design to address and incorporate new parameters for future use, and (iii) Generic design to address new scenarios such as roaming, network sharing, etc. With these principles and given information, SA2 thinks that Option 1b, Option 2 and Option 3 cannot guarantee futureproof design at this stage. Also, substantial investigation is required for comparing those options in terms of futureproofness and extendibility.

SA2’s view on secured data collection, data integrity, and confidentiality

SA2 requires SA3’s views on whether the options can satisfy the requirement “The data collected is secured and data integrity and confidentiality for that data is ensured”. Especially, SA2 identifies potential security issues on how to address the given requirement for the data transfer path between CN/OAM and the OTT server outside the MNO domain.

SA2’s view on user data privacy, anonymity and user consent is respected

SA3’s view is required on how user data privacy, anonymity and user consent can be respected for each data collection option.

2	Actions

To RAN

ACTION: 	SA2 kindly asks RAN to take the above information and suggestion into account.

3	Dates of next SA WG2 meetings

SA2#166AH-e:	20 - 24 January, 2025	Electronic meeting

SA2#167:	17 - 21 February, 2025	Athens, Greece