3GPP TSG-SA WG2 Meeting #166	S2-2411764

# Orlando, USA, 18 – 22 November 2024						               (revision of S2-24xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0499 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the Multi-hop U2N Relay IP address allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As pointed out in the CT1 LS (S2-2411288/C1-246002), it is not clear whether "DHCP proxy" in clause 6.5.14 of TS 23.304 refers to "DHCP relay agent" or not. It is also not clear how the IP address is allocated for the Intermediate UE-to-Network Relay. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that IP address of Intermediate UE-to-Network is allocated by the UE-to-Network Relay, and the Intermediate UE-to-Network Relay relays DHCPv4 messages or Router Solicitation/Advertisement messages between Remote UE and UE-to-Network Relay. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear descriptions about Multi-hop U2N Relay IP address allocation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3.8.2, 6.5.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * 1st Change * * * *

6.4.3.8.2	IP address/prefix management for 5G ProSe Layer-3 Multi-hop UE-to-Network Relay

When IP based PDU session types are used by the 5G ProSe UE-to-Network Relay, the Intermediate UE-to-Network Relay needs to act as an IP Router.

The IP address allocation for Remote UE and Intermediate UE-to-Network Relay is performed by the 5G ProSe UE-to-Network Relay which acts a DHCPv4 server or IPv6 default router.

To support DHCP based IPv4 address allocation, the 5G ProSe Intermediate UE-to-Network Relay relays the DHCPv4 messages between the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay. To support IPv6 Stateless Address auto-configuration, the 5G ProSe Intermediate UE-to-Network Relay relays the Router Solicitation and Router Advertisement messages between the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay.

The 5G ProSe Intermediate UE-to-Network Relay builds its local IP routing table based on the Remote UE info from the link modification procedures as described in clause 6.4.3.8.1. The local IP routing table is per RSC and used based on RSC of the link the packet is received.

* * * * 2nd Change * * * *

6.5.1.4	5G ProSe Communication via Multi-hop 5G ProSe Layer-3 UE-to-Network Relay without N3IWF (based on Model B Discovery)

Figure 6.5.1.4-1: Connection establishment via Multi-hop 5G ProSe Layer-3 UE-to-Network Relay without N3IWF support

Connection establishment via Multi-hop 5G ProSe Layer-3 UE-to-Network Relay without N3IWF support as described in clause 6.5.1.1 is used for multi-hop UE-to-Network Relay after Model B Discovery with the following differences and clarifications:

1.	Service authorization and provisioning are performed for the 5G ProSe Layer-3 UE-to-Network Relay, Intermediate Relay(s) and 5G ProSe Layer-3 Remote UE.

2.	The 5G ProSe Layer-3 UE-to-Network Relay may establish a PDU Session for relaying.

3.	The 5G ProSe Layer-3 Remote UE performs Model B discovery of a 5G ProSe Layer-3 UE-to-Network Relay. The Remote UE obtains the path information to the UE-to-Network Relay(s) from the discovery procedure.

4.	The 5G ProSe Layer-3 Remote UE selects a path and establishes a connection for unicast mode communication as described in clause 6.4.3.9.

5.	For IP PDU Session Type and IP traffic over PC5 reference point, the UE-to-Network Relay assigns IPv6 prefix or IPv4 address to Intermediate Relay(s) and the Remote UE as defined in clause 6.4.3.8.2.

6.	The 5G ProSe Layer-3 Remote UE may provide PC5 QoS Info and PC5 QoS rule(s) to the 5G ProSe Layer-3 UE-to-Network Relay using Layer-2 link modification procedure via Intermediate Relay(s). Intermediate Relays may split the PC5 QoS hop-by-hop as described in clause 5.6.2.

Editor's note:	It is FFS how to handle QoS.

7.	For the Remote UE Report, the Remote User ID is an identity of the 5G ProSe Layer-3 Remote UE.

For IP PDU Session Type, the Remote UE info is IP info related to Remote UE.

-	for IPv4, the 5G ProSe Layer-3 UE-to-Network Relay shall report IP address and TCP/UDP port ranges that are assigned to 5G ProSe Layer-3 Remote UE(s) by the UE-to-Network Relay;

-	for IPv6, the 5G ProSe Layer-3 UE-to-Network Relay shall report IPv6 prefix(es) assigned to individual 5G ProSe Layer-3 Remote UE(s).

For Ethernet PDU Session Type, the Remote UE info is Remote UE MAC address which is detected by the 5G ProSe Layer-3 UE-to-Network Relay.

NOTE:	For IP type of PDU Sessions, the Intermediate Relays forward the Downlink packet to the correct PC5 links based on destination IP address of the packet. For Ethernet type of PDU Sessions, the Intermediate Relay forwards the Downlink packets according to the destination Ethernet MAC address.

* * * * End of Changes * * * *