SA WG2 Meeting #166	S2-2411770

# Orlando, USA, 18 – 22 November 2024                         (revision of S2-240xxxx)

Source: 	CATT

Title: 	KI#1 Conclusions on RRC-based option

Document for: 	Approval

Agenda Item: 	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes to update the conclusions on RRC-based option.

1	Discussion

This paper proposes to update the conclusions on RRC-based option.

Two options (i.e. Option A and Option B) are documented for RRC-based option. From deployment point of view, it is reasonable to assume that for Topology 2, the A-IoT-enabled gNB connects to the AMF using the existing N2 interface rather than the new introduced Nx interface for Topology 1. Two options also make the RRC-based solution more complicated. So it is proposed to keep Option B only.

Looking into the protocol stack of the RRC-based option, it is composed of several hops between UE Reader and AIoTF, namely Uu interface, N2 interface and SBI interface, and it is in nature a RAN leading solution. Whether the RRC-based option really works on Uu interface and N2 interface depends on RAN WGs decision. It is not clear to make the end-to-end RRC-based option work, what information is required from CN to RAN to support the AIoT service operation, e.g. whether UE ID needs to be provided.

2	Proposal

It is proposed to update TR 23.700-13 as follows.

# * * * * First change * * * *

8.1.3	Architecture to Support Topology 2

8.1.3.1	General

Two options will be specified for Topology 2:

-	User-plane option as defined in clause 8.1.3.2.

-	RRC-based option as defined in clause 8.1.3.3.

Editor's note:	Whether a NAS-based option is needed in addition is FFS.

8.1.3.2	User-plane option

The following principles apply:

-	As depicted in Figure 8.1.3.2-1, the UE Reader connects to the AIoTF based on the AIoT Application Protocol (AIoT-AP) using an IP PDU Session between the UE and the UPF as transport. The related protocol stack is shown in Figure 8.1.3.2-2. The AIoT AP protocol will support procedures and information to be exchanged as specified by RAN WG2, RAN WG3 and SA WG2.

Figure 8.1.3.2-1: User-plane architecture for Topology 2

Figure 8.1.3.2-2: Protocol stack for the user-plane architecture for Topology 2

NOTE 1:	Which transport protocol to use for AIoT-AP can be decided by CT WG1 in coordination with RAN WG3.

NOTE 2:	Security for AIoT-AP is assumed to be defined by SA WG3.

Editor's note:	Further details are FFS.

Editor's note:	The terminology, e.g. AIoT RAN is FFS.

8.1.3.3	RRC option

The following principles apply:

-	Messages between the UE Reader and the AIoTF are delivered using RRC and a protocol between gNB/AIoT RAN and AIoTF. The related protocol stack options are shown in Figure 8.1.3.3-2.

Editor's note:	How addressing works for UL for Option B is FFS.

Editor's note:	Further details are FFS.

Editor's note:	It is FFS what information (e.g. UE ID) is required from CN to RAN to support the AIoT service operation, and cooperation with RAN WGs is required.

Editor's note:	Final conclusion depends on RAN WGs decision.

Figure 8.1.3.3-2: Protocol Stack for the RRC option (Option B)

# * * * * End of Changes * * * *