3GPP TSG-WG SA2 Meeting #166	S2-2411812

# Orlando, USA, 18th Nov – 22th Nov, 2024                                 (revision of S2-2411192)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1208 | rev | 6 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the general inference procedure for vertical federated learning to resolve ENs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [OPPO], [ZTE], [Samsung], [Nokia], [Lenovo], [Toyota], [ICS], Huawei, HiSilicon, [Ericsson] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Editor's note: How the VFL server determines the VFL clients and whether the VFL server may not select all VFL clients is FFS. is removed since clause 6.2H.2.1 describes the VFL discovery. The UE ID(s) is added in the request send from the VFL server to the VFL client to indicate the VFL client which UE will be used as the sample to let VFL client collect the data and perform the VFL inference. The Editor's note: Whether the VFL model correlation ID is needed or using the model ID is FFS. is removed. The local model in each client can be different. Model ID is designed to identiy the sepcfic model. The model ID is same means the model should have the same topology. However, in the VFL case, the topology for each client local model can be different. It is not suitable to use the same model ID for every client. The VFL model correlation ID is perferred to indicate those local model in each client should used for the same VFL task e.g. for the same analytcis ID. The VFL model correlation ID is added when the VFL client send the VFL inference result to the VFL server. VFL model correlation ID is used to let VFL server know which inference result from which client need to be combined to generate the VFL inference result. Rev 6: The Editor's note: Whether the case that the AnLF can not generathe the analytics output but can determines the VFL server for the requested analytics is exist is FFS. Whether and how such NWDAF is capable to determine the NWDAF as VFL server is FFS. Is removed. Adding clarification on NWDAF generating the analytics output based on VFL inference results when VFL client(s) are selected and when no VFL client is selected. Adding clarification of how the VFL server determine the VFL clients for the VFL inference. The Editor's note: It is FFS additional parameters are needed to send from the VFL client to VFL server. in step 2 is removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Several ENs are resolved for the VFL inference procedure. Parameter from Analytics subscription such as analytics target and analytics filer is added in the request send from the VFL server to the VFL client and in related replies. The VFL model correlation ID is added when then VFL client send the VFL inference result to the VFL server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Can not perform VFL inference sucessully if some open issue can not be resolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2G, 6.2H.1, 6.2H.2.4(new), 6.2H.2.4.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 6: In order to remove the EN about AnLF and the determination of VFL Server, it is proposed that an AnLF not being able to generate the output exist in the following scenarios: - Case A: NF Consumer selected a NWDAF that has the VFL Capability but is a VFL Client for a given VFL correlation ID. - Case B: NF Consumer selected a NWDAF that does not have VFL capability (i.e., is not a VFL Server nor a VFL Client). In order to NWDAF containing AnLF to determine the VFL Server two types of information are defined: - It is proposed to define the parameters that are part of the stored mapping information (created at end of the VFL training) that are analyzed by the NWDAF containing AnLF, in order to determine: a) its role in the VFL inference, b) the applicable VFl correlation ID for the analytics subscription, and the local ML model associated with the application VFL correlation ID. - It is proposed that NWDAF containing AnlF that do not have the VFL Capability are configured with the mapping of which analytics ID and associated target of analytics reporting should be handled with VFL Inference. In case A: The NWDAF containing AnLF is capable to determine that it cannot generate the VFL inference by checking its role in the stored maping information (stored at the end of training) relating an Analytics ID to a VFL correlation ID. Based on the VFL Server information in the stored mapping information, the VFL Client determines and subscribes to the VFL Server. In Case B: The AnLF is only configured to determine that for a given analtyics ID with a given target of analytics reporting, VFL inference should be used. In this case, we propose that NWDAF containing AnLF selects another NWDAF from NRF. The Step 2 describes that the NWDAF as VFL Server may select some VFL Clients or none. However, the current description in Step 2 does not clarify how the analtyics output generation will reflect the VFL inference results in these two situations. This description has been added, including the concept that the VFL Server may use the accuracy as a mechanisms to determine when to generate the output using only its local ML model associated with the VFL correlation ID. The procedures of accuracy monitoring are not proposed in this document. The current text in Step 2 is not clear on how the NWDAF containing AnLF with a VFL Server capability for a VFL correlation ID determine the VFL Clients that may participate in the VFL inference. We propose that the same stored mapping information is used by the VFL Server to check the information of the VFL Clients. The underlying assumption is that a VFl Server and VFl Client that participated in the VFL training will be the same NF that will participate in the VFL Inference. A note has been added reflecting this principle. The EN is removed in Step 2 about the additional parameters from VFL Server and VFL Clients and included the one parameter: inference time information, which indicates the time used to determine inference data, to ensure that different VFL Clients use the inference data collected in the same time period. |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2G	Void

6.2G.1	Void

6.2G.2	Void

6.2G.2.1	Void

# * * * * Next change * * * *

6.2H.1	General

This clause specifies how NWDAF as a VFL server can register and also discover other NWDAFs and/or AFs via NRF

Both the VFL server and VFL client store the VFL model after finishing the VFL training process and use the same VFL local model to perform the VFL inference later based on the VFL correlation ID. The differences between the VFL training and inference are that for the inference there is no check of the labels, nor any server intermediate results are sent and as such only client intermediate results are sent to the server.

Editor's note:	This clause needs to be update to include the description of the subclauses for NWDAF or AF as the server case.

# * * * * Next change * * * *

6.2H.2.4	Inference procedure for vertical federated learning

6.2H.2.4.1	Inference procedure for vertical federated learning when NWDAF is acting as VFL server

Figure 6.2H.2.4.2-1: Inference procedure for vertical federated learning when NWDAF is acting as VFL server

Editor's note:	If any of the Consumer, Server and clients are untrusted AF(s), how the NEF assists the VFL inference process, and whether the existing or new NEF service should be invoked are FFS.

0.	The analytics consumer NF sends an Analytics request/subscribe (Analytics ID, Target of Analytics Reporting= e.g., UE IDs, Analytics Reporting Information=Analytics target period)) to NWDAF containing AnLF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

1.	When the NWDAF containing AnLF receives the subscription for analytics ID from a Consumer NF, the NWDAF Containing AnLF verifies if it has a stored mapping information between the Analytics ID and a VFL correlation ID, additionally considering the target of analtyics (i.e., which is related to the sample information stored for the correlation ID) . If this mapping exists, the NWDAF containing AnLF based on the stored mapping information determines whether it is the VFL Server for such VFL correlation ID  (e.g., checking the VFL Server information).

NOTE:	In this release, the same NF associated with a VFL Server or VFL Client capability during the VFL training is also the same NF during the VFL inference. This information is comprised stored mapping information as per parameters VFL Server information and the VFL Client information for a given VFL correlation ID.

If the NWDAF containing AnLF determines it is the VFL server to generate the VFL inference results for the requested analytics ID, then step 1 is skipped.

If the NWDAF containing AnLF determines it is not the VFL server for the requested analtyics ID by verifying there is no VFL correlation ID stored at the NWDAF containing AnLF, and it can not generate the analytics output but by configuration it determines that VFL inference has to be used for the request based on the Analytics ID and the target of analytics reporting, the NWDAF containing AnLF discovers another NWDAF containing AnLF  for the requested analtyics, sends a subscription request to new NWDAF using Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe including Analytics ID, Target of Analytics Reporting = e.g., UE IDs.

If the NWDAF containing AnLF determines it is not the VFL server for the requested analtyics ID and instead is the VFL Client for the VFL correlation ID, the NWDAF containing AnLF based on its local stored mapping information determines the VFL Server associated with the VFL correlation ID and sends a subscription request to VFL Server. When the VFL Server is an NWDAF, it uses the Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe including Analytics ID, Target of Analytics Reporting = e.g., UE IDs.

2.	Based on the information received in the step 0 or 1, and the stored mapping information the VFL server decides to initiate the VFL inference procedure in order to generate the analytics output based on the VFL result(s). The VFL Server determines based on the stored mapping information the applicable VFL correlation ID for the requested or subscribed analtyics and the local ML model to used for its own generation of the VFL resutls.

VFL Server may select the VFL Clients(s) associated with the VFL correlation ID using the mapping information stored during the VFL training process. The server may select some or no VFL clients, e.g., depending on their contribution to the training result, local configuration, and/or  the current status of the VFL client.

When no VFL Clients are selected, the VFL server generates the VFL inference results based only on its local trained ML model associated with the determined VFL correlation ID, skipping the steps 2 – 6 (and 7 if step 1 was also skipped). The VFL Server triggers the VFL accuracy monitoring for the VFL correlation ID and based on the analtyics accuracy for such VFL correlation ID, whether to provide the analytics output based on the VFL inference results associated only with its local ML model or based on the joint VFL inference with the VFL Clients.

When the NWDAF selects VFL Client(s) to participate in the VFL Inference, if the VFL server is NWDAF and the VFL client(s) are NWDAF(s), the VFL server NWDAF sends a VFL Inference request/subscription to the VFL clients including the UE IDs, VFL correlation ID to indicate to the VFL client which previously well-trained VFL local model associated with this VFL correlation ID will be used, as well as inference time information. The VFL Client, checks the stored mapping information in order to determine the local ML model to be used for the VFL inference process requested by the VFL Server.

Editor's note:	It is FFS how the origin of analytics results can be traced and explained if not all clients participate and results are not satisfying.

When the VFL server is NWDAF and the VFL client(s) are AF(s), VFL server NWDAF sends a VFL Inference request/subscription to the VFL clients which includes the UE ID(s), the VFL correlation ID to indicate to the VFL client which previously well-trained VFL local model associated with this VFL correlation ID will be used, as well as inference time information. If the AF is untrusted, the VFL server NWDAF sends the request to the NEF, and NEF forwards the request to the untrusted AF.

Editor's note:	Which service is used between the VFL server and VFL client is FFS.

3.	Each VFL Client that accepted to join the VFL Inference process collects inference data based on the inference time information. Each VFL Client collects its local data by using the current mechanism if the VFL Client does not have local data available already.

4.	Based on the VFL correlation ID, each VFL Client determines the VFL local model to generate the intermediate local inference results.

Editor's note:	Whether VFL client may also provide local intermediate inference results to other VFL client is FFS.

5.	VFL Client sends the client intermediate local inference results to the VFL server including the VFL correlation ID.

The intermediate local inference results, which are sent from the VFL Client to the VFL Server during the VFL inference process, are the information for the VFL Server to combine and generate the VFL inference results.

Editor's note:	It is FFS additional parameters are needed to send from the VFL client to VFL server.

If the VFL server used an inference subscription in step 2, step 5 may be repeated.

6.	The VFL server may collect its local data and generate the intermediate local inference results.

When the VFL Server, selected VFL Clients to participate in the VFL Inference process, it combines all the intermediate local inference results to generate the VFL inference results for the VFL correlation ID. The VFL server takes into account the participation of each VFL client during the ML training process and the importance of the intermediate results when generates the combined inference output.

Editor's note:	The details on how the VFL server monitoring the accuracy of the VFL is FFS.

7.	If the NWDAF is the VFL server, the VFL server sends Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify to the consumer (i.e NWDAF containing AnLF) including the VFL inference results.

8.	The NWDAF containing AnLF provides the analytics output to the analytics consumer NF based on the VFL inference results by means of either Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 0.

# * * * * End of changes * * * *