3GPP TSG-SA WG2 Meeting #166	S2-2411841

Orlando, USA, 18th – 22nd November 2024	(revision of S2-xxxxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | -1284 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2 Introduction of VFL client training service for AF as the VFL client |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 6.2H.2.3, there is the following EN: Editor’s Note: It is FFS which service is used between VFL server and VFL clients for VFL training. As this EN indicates, there is no service in the existing VFL server that triggers an training request to the AF as VFL client. Therefore, a new service that can enable the VFL server to request an training to the VFL client should be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a service that requests a training to the VFL client. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The VFL client can no longer request an training, and the VFL training procedure cannot be performed when the AF as the VFL cilent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.3.1, new X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

6.2H.2.3 Training Procedure for Vertical Federated Learning

6.2H.2.3.1	Training Procedure for Vertical Federated Learning when NWDAF is acting as VFL server

The figure 6.2H.2.3.1-1 below shows the training procedure for Vertical Federated Learning when NWDAF is acting as VFL server.

Figure 6.2H.2.3.1-1Training procedure for Vertical Federated Learning when NWDAF is acting as VFL server

Editor’s Note: Further extensions are needed to show the interaction between consumer and VFL server. For example, how the consumer (i.e., NWDAF containing AnLF) sends a subscription request to VFL server.

Editor’s Note: Whether and how to maintain a Vertical Federation Learning process including dynamical reselection, addition, or removal of VFL Client NWDAF(s) is FFS.

Editor's Note:  How the NEF assists the VFL training process as well as whether the service operations going via NEF is using the existing or new service operation are FFS.

Editor’s Note: The details of the services in the procedure and whether VFL Training Start Flag is needed are FFS.

Editor’s Note: It is FFS whether sample/feature information is required to be provided in each training.

Editor´s Note: Whether and how to include interoperability information in the VFL training procedure is FFS.

Editor’s Note:	Terminology will be aligned in the future.

Editor’s Note:	It is FFS if the term “backward local ML model training information” needs to be changed into “intermediate results”.

1.	The NWDAF acting as VFL server determines the VFL clients that participate in VFL procedure in the VFL clients discovery and preparation phase as described in the clause 6.2H.2.1 and clause 6.2H.2.2.

NOTE: VFL Server determines to perform VFL training based on the internal trigger or local configuration.

The steps 2-6 are repeated until the training termination condition is reached.

2.	To start VFL training, the VFL server allocates VFL correlation ID. The VFL server sends a request to start the VFL training to each of the selected VFL clients. The VFL server NWDAF sends a Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request to the selected NWDAF clients(s) and Naf_VFLTraining_Subscribe or Naf_VFLTrainingInfo_Request to the selected AF clients(s). The request includes VFL correlation ID, VFL capability type (VFL Client).

If the VFL procedure continues in subsequent iterations, the VFL server sends a request for a new VFL training iteration containing the backward local ML model training information to each of the VFL clients for next round of VFL training.

Editor´s Note: Additional Parameters to be provided in the request are FFS.

Editor´s Note: It is FFS whether and how the local ML model is obtained by VFL Client in VFL training process.

3.	[Optional] Each VFL client collects its local data by using the current mechanism if the VFL client has no local data already available.

4.	During VFL training procedure, each VFL client further trains the local ML model associated with the same VFL Correlation ID based on their own collected or available data and based on possible backward local ML model training information distributed by the VFL server in the previous training iteration, and computes and reports the client intermediate training result of the local ML model to the VFL server.

Editor’s note:	The report may include ID(s) of sample(s) corresponding to the intermediate training result.

Editor´s note: The following is FFS and may depend on the service design: When the clients reports the client intermediate training result, it also includes the corresponding VFL correlation ID.

5.	The VFL server may collect the local data and generate its own local intermediate training result. The NWDAF acting as VFL Server computes the backward local ML model training information (e.g. gradient information or loss information) based on the client intermediate training result received in step 4, its own local intermediate results and the label. The backward local ML model training information is used for updating the models of VFL clients. Different backward local ML model training information may be computed for different VFL clients, respectively.

The VFL server may also compute the ML model metric (e.g. ML model accuracy) based on all the intermediate training result received from VFL clients and the label.

Editor’s Note: Whether weight of feature is computed by VFL server is FFS.

Editor’s Note: Whether VFL server and VFL clients share feature information is FFS.

6.	[Optional] The NWDAF acting as VFL server evaluates (e.g., based on the convergence of a loss function or loss value, the pre-set iteration number is reached) whether VFL Training process converged. If not, the NWDAF acting as a VFL Server determines another round of VFL training is required and repeats step 2 - 6. If yes, it determines the VFL Training is completed. In this case, the VFL Server terminates the current VFL training process via step 7.

The VFL training termination decision may be also made as follows:

Based on the consumer request, the VFL server sends VFL status report to the consumer. The status report may include model metric (e.g. ML model accuracy).

Editor’s Note: The content of the VFL status report is FFS.

Editor’s Note: Whether VFL server sending convergence report to the VFL client and what is convergence report are FFS

The consumer decides whether the current model can fulfil the requirement, e.g. ML model metric is satisfactory for the consumer and determines to stop or continue the training process. The consumer continues the training process or stops the training process.

Based on the subscription request sent from the consumer, the VFL server updates or terminates the current VFL training process.

Editor’s Note: the following is FFS. The VFL server determines with which VFL Client(s) to continue the VFL. The VFL server may provide to the VFL clients any sample ID(s) if changed

7.	The VFL server sends VFL training termination message to VFL Client if it decides to terminate the VFL training process. The VFL Server and each VFL Client, stores the VFL correlation ID and latest information related to their locally trained Models

NOTE: The VFL correlation ID is used later for inference.

Editor’s Note: Whether VFL Training termination Flag in the termination message is required is FFS.

Editor’s Note: Possible procedures to be executed at the end of the VFL training (to terminate a training session and to store/handle trained models) are FFS.

NOTE 1:	If untrusted AF is involved in VFL Clients, the message between NWDAF acting as VFL Server and the untrusted AF is via NEF.

* * * 2nd Change * * * *

X	AF Services to support to support network data analytics

X.1	General

Table X.1-1 illustrates the AF Services to support network data analytics.

Table X.1-1: NF services provided by AF to support to support network data analytics

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Naf_VFLTraining | Subscribe | Subscribe / Notify | NWDAF, NEF |
|  | Unsubscribe |  | NWDAF, NEF |
|  | Notify |  | NWDAF, NEF |
|  | Request | Request / Response | NWDAF, NEF |
| Naf_VFLTrainingInfo | Request | Request / Response | NWDAF, NEF |

X.1	Naf_VFLTraining service

X.1.1	General

Service description: this service enables an VFL server as consumer to request the AF to participate in VFL model training as VFL client.

X.1.2	Naf_VFLTraining_Subscribe service operation

Service operation name: Nnwdaf_ VFLInference_Subscribe

Description: Subscribe to VFL inference.

Inputs, Required:

-	Analytics ID as defined in Table 7.1-2;

-	Notification Target Address (+ Notification Correlation ID);

-	VFL model Correlation ID;

-	VFL capability type.

Inputs, Optional:

-	ML Model identifier: identifies the provided ML Model.

-	ML Model Information (as defined in clause 6.2A.2);

-	ML Model file;

-	Training Filter Information;

-	Target of Training Reporting;

-	Subscription Correlation ID (in the case of modification of the ML Model Training subscription)

-	Expiry time.

-	Indication of skipping the current VFL round.

Outputs Required: When the request is accepted: Subscription Correlation ID (required for management of this subscription). When the request is not accepted, an error response with cause code (e.g. NWDAF does not meet the ML training requirements, ML training is not complete, NWDAF overload, not available for the VFL process anymore, etc.).

Outputs, Optional: none

X.1.3	Naf_VFLTraining_Unsubscribe service operation

Service operation name: Naf_ VFLTraining_Unsubscribe

Description: Terminate VFL Model training.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Cause code (e.g. VFL Client AF is unselected by the VFL Server for the VFL process, or the VFL process is suspended or finished, etc.). Final aggregated ML Model information (if VFL has finished) or updated aggregated ML Model information (if VFL is suspended).

X.1.4	Naf_VFLTraining_Notify service operation

Service operation name: Naf_VFLTraining_Notify

Description: AF notifies the consumer instance of the trained ML Model that has subscribed to the specific AF service. The AF can also use this service to indicate to consumer it will terminate the ML Model training.

Inputs, Required:

- Notification Correlation Information: this parameter indicates the Notification Correlation ID that has been assigned by the consumer during ML Model training.

Inputs, Optional:

-	Analytics ID;

-	VFL model Correlation ID

-	Termination Request: this parameter indicates that AF requests to terminate the ML Model training, i.e. AF will not provide further notifications related to this request, with cause code (e.g. AF overload, not available for the VFL process anymore, etc.);

-	ML Model identifier: this parameter identifies the provisioned ML Model;

-	Global ML Model Accuracy information: The model accuracy of the global ML Model, which is calculate by the VFL Client AF using the local training data as the testing dataset;

-	Status report of VFL training: local ML Model accuracy information and Training Input Data Information (e.g. areas covered by the data set, sampling ratio, maximum/minimum of value of each dimension, etc.), which are generated by the VFL Client AF during VFL procedure;

-	Indication of skipping the current VFL round.

NOTE:	The detail reasons in the cause code are up to stage 3.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

* * * 3rd Change * * * *

X.2	Naf_VFLTrainingInfo Service

X.2.1	General

Service Description: This service enables the consumer to request for the information about ML Model training based on the ML Model file or ML Model information as described in clause 6.2H.2.3 provided by the consumer.

X.2.2	Naf_VFLTrainingInfo_Request service operation

Service operation name: Naf_VFLTrainingInfo_Request

Description: Request information about VFL ML Model training with specific parameters.

Inputs, Required:

-	Analytics ID as defined in Table 7.1-2.

-	VFL model Correlation ID.

Inputs, Optional:

-	ML Model identifier: identifies the provided ML Model.

-	ML Model Information (as defined in clause 6.2A.2).

-	ML Model file.

-	ML Training Information (i.e. data availability requirement, time availability requirement).

-	Termination Request, when terminating the VFL identified by the VFL model Correlation ID and optionally indicating the reason, e.g. VFL Client AF is unselected by the VFL Server for the VFL process, or the VFL process is suspended, etc.

-	Training Filter Information.

-	Target of Training Reporting.

-	Indication of skipping the current VFL round.

Outputs Required: When the request is accepted: Operation execution result indication. When the request is not accepted, an error response with cause code (e.g. AF does not meet the ML training requirements, ML training is not complete, AF overload, not available for the VFL process anymore, etc.).

NOTE:	The detail reasons in the cause code are up to stage 3.

Outputs, Optional:

-	ML Model identifier.

-	Analytics ID.

-	VFL model Correlation ID.

-	Global ML Model Accuracy information: The model accuracy of the global ML Model, which is calculate by the VFL Client AF using the local training data as the testing dataset.

-	Status report of VFL training: local ML Model Accuracy Information and Training Input Data Information (e.g. areas covered by the data set, sampling ratio, maximum/minimum of value of each dimension of data, etc.), which are generated by the FL Client AF during VFL procedure.

* * * END of Change * * * *