3GPP TSG-WG SA2 Meeting #166	S2-2411848

Orlando, USA, Nov 18 – 22, 2024                                  		                (revision of S2-2411161)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0146 | rev | 3 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to multiple USS serving different geographical areas |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Rev 3： SA2#165 agreed to add the description that “UUAA procedure defined in clause 5.2 is performed by the UAV for authorization with the target USS, and NEF/UAS NF uses the target USS address received from the serving USS to forward the UUAA request.” In addition, SA3 conclusion on the support for the multiple USS and changeover from one USS to another (S3-244427) is as follows: - UAS NF receives and maintains info about authorized USS (es). UAS NF stores authorized USS (es) info as part of the UUAA context. It is proposed to update the corresponding part in TS 23.256 based on SA3 conclusion. There are two ways to interprete the “authorized USS (es)”: - The “authorized USS” refers to the USS(es) serving the UAV flight path, or - The “authorized USS” refers to the USS(es) have sueessfully authorized the UAV The “authorized USS(es)” has impact on the feature of USS changeoever therefore clarification is needed. [below texts are from SA2#165]: In the last meeting, one CR about how a 3GPP network may assist the UAV UE to perform a changeover from one USS to another when a UAV moves over geographical areas served by different USSes has been approved. The procedures about how to determine the USS changeover is complete while how to perform the UUAA procedures when the serving USS change happens is not described. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Rev 3: clarify the authorized USSes information used during USS changeover: If Authorized USS(es) information is stored in the UUAA context, the NEF/UAS NF checks whether the target USS address provided by UAV matches one of the Authorized USSes information. If the target USS is among the Authorized USSes information that serves the UAV flight path, the NEF/UAS NF will forward the UAV UUAA request toward the target USS. If the target USS is among the USSes that authorized UAV, the NEF skips the interaction with the target USS and updates the UUAA context and complete the changeover for the UAV. [below texts are from SA2#165]: Clarify the UUAA procedures is performed between UAV and target USS during the USS changeover. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How the NEF transfer the UUAA request between the UAV and the target USS is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.13.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

5.13.1	USS changeover

In cases when a UAV moves over geographical areas served by different USSs, a 3GPP network may assist the UAV UE to perform a changeover from one USS to another. The following describes the interactions in order to perform the changeover:

-	USS can serve the UAV within its geographical area, and each USS may have an overlap with a geographical area primally served by another USS, i.e. USS' areas may not be strictly separated. A geographical area of a USS does not necessary identically match to tracking areas or cells of a 3GPP network. However, TAI(s) or cell IDs can be used by NEF to translate/map this information and vice versa whenever required.

-	Each USS is aware of its neighbouring USSs, border area(s) that it has with its neighbour USSs and possible border-crossing points for UAV.

-	USSs can communicate with each other using means outside the scope of the 3GPP specification.

-	USS can trigger a changeover by determining that a UAV is approaching a border its service and requesting an NEF/UAS NF to assist with the changeover.

Editor's note:	Whether a new service operation needs to be introduced so that a USS can trigger a changeover procedure or the existing (e.g. Naf_Authentication_Notification) can be reused requires further evaluation.

-	USS can determine whether a changeover to another USS needs to take place. The determination can be based, for instance, on a notification from the network and/or information about a UAV's next point in the flight path in a pre-flight planning request. If the changeover is required, a serving USS is able to determine a possible USS (one or more) that can serve as a target USS and can trigger communication with them to determines candidate USS border-crossing point(s) for the UAV.

NOTE:	For this purpose, the serving USS relies on the pre-established knowledge that are outside the scope of 3GPP.

-	Alternatively, the determination can be based on a trigger from a UAV by sending, for example, a pre-flight planning request with a next point in the flight path outside an area of authority/responsibility of a serving USS.

To determine a changeover needs to take place and collecting the required information from all suitable target USSs, a serving USS requests the core network to assist with a changeover. The request goes to the NEF/UAS NF and includes the information received from the suitable target USSs and any additional information the NEF might need to perform this task (e.g. addresses of the suitable target USSs, requirements on the flight path, candidate flight path(s), accuracy level of predictions relevant to the flight monitoring). Based on the information in the request, the NEF/UAS NF determines the relevant NFs and specific service operations it needs to invoke to collect the required information for UAV flight planning/monitoring to determine with a defined likelihood that the UAV will leave the service area of one USS and moves into area of another USS (e.g. NWDAF analytics service for Movement Behaviour analytics, GMLC service for Ranging/Sidelink Positioning location, AMF for UAV's presence in bordering cells/TAs, AMF service for UAV's deviation for the expected/assigned trajectory). The serving USS also subscribes to get notified when it is time to make the changeover.

Once the serving USS detects that the UAV has left or is about to leave its service area (for instance, based on UAV's presence in a border TA/cell), the serving USS may determine to trigger USS changeover to a target USS to serve the UAV. The currently serving USS communicates with the selected target USS and passes the target USS the following information in order to establish communication towards UAV and 5GC:

-	which exposure services and notification are of relevance for the UAV;

-	UAV's identifiers;

-	any other information required by the target USS or by the UAV itself to establish the connection and perform authorization (e.g. a token containing information about the serving USS, the UAV, the target USS, and some other parameters such as issue date, validity data, or expire date).

The serving USS also informs the UAV about the changeover to the new target USS and may instruct the UAV (directly or via NEF/UAS NF) to reconnect and authorize with the target USS as well as trigger the exposure services towards 5GC NFs similar to what the previous serving USS had before the changeover. UUAA procedure defined in clause 5.2 is performed by the UAV for authorization with the target USS, and NEF/UAS NF uses the target USS address received from the serving USS to perform the USS changeover. If Authorized USS(es) information is stored in the UUAA context, the NEF/UAS NF checks whether the target USS address provided by UAV matches one of the Authorized USSes information. If the target USS is among the Authorized USSes information that serves the UAV flight path, the NEF/UAS NF will forward the UAV UUAA request toward the target USS. If the target USS is among the USSes that authorized UAV, the NEF skips the interaction with the target USS and updates the UUAA context and complete the changeover for the UAV. Once the changeover is complete and the UAV enters a TA/cell in the area served by a new USS, the new serving USS informs the old serving USS about the completion.

In order to continue with the changeover procedure, 3GPP system expects the new serving USS to respond to the notification in such a way confirming to the network about the completion of the changeover for the UAV. A UUAA authenticated context shall be established between the UAV and the new USS before the UAV interact with the new USS.

>>>> End of Changes <<<<