3GPP TSG-SA2 Meeting #166	S2-2411865

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1514 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Call setup of UE-Satellite-UE communication in IMS without IMS-AGW on board |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2024-11-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on 5GSAT_Ph3_ARCH WID work task, the functionality and procedure of UE-satellite-UE communication without IMS-AGW deployed on the satellite need to be captured in TS 23.228. For call setup: When the originating P-CSCF receives the initial INVITE request from the originating UE using satellite access, it may not be able to determine whether the UE-Satellite-UE communication can be activated before negotiating with the terminating side. The originating P-CSCF selects an IMS-AGW on the ground for the originating UE after receiving the INVITE message and triggers media negotiation by sending an SDP offer to the terminating side. If the terminating side determines that it is possible to activate UE-Satellite-UE communication, then a re-selection/re-negotiation procedure is needed for the originating P-CSCF to remove the IMS-AGW on the ground from the U-plane path and enable UE-Sat-UE based on SDP belonging to UE address. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of support of UE-Satellite-UE communication in IMS, including: - Call setup procedures for UE-SAT-UE communication without IMS-AGW on-board satellite. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An objective of the 5GSAT_Ph3_ARCH work item is not achieved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AX.6 (new), AX.6.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR should be implemented after TS 23.228 CR 1428. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# FIRST CHANGE

# AX.6	Procedures for optimized media routing without IMS-AGW on-board satellite

## AX.6.1	Call setup

Figure AX.6.1-1 depicts a signalling flow diagram for call setup for optimized media routing for deployment where IMS-AGW is not hosted on-board the satellite.

When the originating P-CSCF receives the initial INVITE request from the originating UE using regenerative NR satellite access, it selects the IMS-AGW on ground and triggers media negotiation by sending SDP offer to the terminating side indicating that satellite access is used on the originating side. If the terminating P-CSCF activates UE-satellite-UE communication, the originating P-CSCF configures the resources in 5GS(I-UPF) on satellite via PCF and SMF.

Figure AX.6.1-1:	Session establishment procedure with activation of optimized media routing without IMS-AGW onboard

1.	UE A sends a SIP INVITE request, containing an initial SDP offer, to P-CSCF A.

2.	After receiving the SIP INVITE request, if UE A is in the non-roaming scenario and the P-CSCF A supports UE-Satellite-UE communication, it may perform following actions to determine whether UE-satellite-UE communication is possible:

- 	P-CSCF A retrieves Access Network Information from PCF based on the mechanism defined in TS 23.503 [95] and determines whether the UE A is using regenerative satellite access or not based on whether a serving satellite identification is obtained as part of the Access Network Information or not.

3.	The P-CSCF A may allocate an IMS-AGW A on the ground. P-CSCF A interacts with IMS-AGW A to allocate transport resources based on the mechanism defined in TS 23.334 [74]. The P-CSCF may also choose not to allocate an IMS-AGW on the ground instead send the UE SDP without any modification.

4.	P-CSCF A updates the SIP INVITE request inserting the identification of the satellite serving UE A and may update SDP offer with the IMS-AGW transport addresses allocated in step 3. Then, P-CSCF A forwards the updated SIP INVITE request to the terminating side via IMS core.

5-6. P-CSCF B forwards the updated SIP INVITE request to UE B. UE B returns a 18X response with SDP Answer.

7.	After receiving the SIP response and the satellite information received in step 4, P-CSCF B retrieves Access Network Information of UE B from PCF network based on the mechanism defined in TS 23.503 [95]. If the UE B is also using the regenerative satellite access, the serving satellite identification is contained in the Access Network Information.

8.	P-CSCF B instructs the PCF to authorize the resources necessary to establish a QoS flow for media via Npcf_PolicyAuthorization_Update service or Rx Session binding operation.

To indicate SMF to insert UL CL/BP and L-PSA UPF for IMS PDU session to enable UE-Satellite-UE communication, P-CSCF B should send satellite information of A side. This will assist SMF to allocate and configure the I-UPF on-board satellite for N6 interface specific information towards satellite on A side.

The I-UPF shall inform the N6 information (e.g. PtP IP/port) towards SMF. The SMF needs to inform the N6 information towards PCF.

9. If in step 7, the ANI information is requested, the PCF invokes the Npcf_PolicyAuthorization_Notify service operation to provide the access network information and N6 information received in step 8. For Rx based interface this notification is done via Diameter RAR.

10.	P-CSCF B updates the 18X response to carry the identifier of the satellite serving UE B to indicate that UE-Satellite-UE communication is activated. Then, P-CSCF B returns the updated 18X response to originating side via IMS core.

11.	Based on the identification of the satellite serving UE B in step 10, P-CSCF A determines to perform UE-satellite-UE communication and instruct the PCF accordingly.

12.	P-CSCF A instructs the PCF to authorize the resources necessary to establish a QoS flow for media via Npcf_PolicyAuthorization_Update service or Rx Session binding operation.

To indicate SMF to insert UL CL/BP and L-PSA UPF for IMS PDU session to enable UE-Satellite-UE communication, P-CSCF A should send satellite information and N6 information of B side. This will assist SMF to allocate and configure the I-UPF on-board satellite for N6 interface specific towards satellite on A side.

The I-UPF shall inform the N6 information (e.g. PtP IP/port) towards SMF. The SMF shall inform the N6 information towards PCF.

13.	P-CSCF A forwards the updated 18X response to UE A.

14.	UE A acknowledges the Response and sends the Response Confirmation to P-CSCF A.

15. If in step 12, the ANI information is requested, the PCF invokes the Npcf_PolicyAuthorization_Notify service operation to provide the access network information and N6 information received in step 8. For Rx based interface this notification is done via Diameter RAR.

16. P-CSCF A releases the IMS-AGW A on ground, if AGW was allocated in step3.

17. P-CSCF A forwards the Response Confirmation to P-CSCF B. The Response Confirmation includes SDP offer modified/generated by P-CSCF A which contains the N6 interface info.

18-19. The P-CSCF provides (via PCF) the SMF with addressing information corresponding to the remote outer tunnel N6 breakout point. This information is sent to the SMF via traffic steering information in a PCC rule (refer to clause 6.3.1 in TS 23.503 [4] for "Application Function influence on traffic routing Enforcement Control")

the SMF Sends a corresponding N4 PDR/FAR to the UPF in such a way that media from the local UE to the remote UE is sent by the local PSA (I-UPF) in a tunnel whose destination address corresponds to the addressing information of the remote outer tunnel N6 breakout point.

20. P-CSCF B sends the Response Confirmation to UE B.

21. Procedure continues to setup the call as defined in clause 5.6 and 5.7.

# End of CHANGEs