3GPP TSG-SA WG2#166	S2-2411870

Orlando, US, 18-22 November 2024	(was)

Source:	Samsung

Title:	Conclusions on AIoT KI#1 for Topology 2

Document for:	Discussion/Approval

Agenda Item:	19.14.1

Work Item / Release:	Rel-19 / FS_AmbientIoT

Abstract of the contribution: This paper proposes conclusions on AIoT Architecture to support Topology 2.

1 Discussion

During SA2#165 meeting, initial conclusions on AIoT Architecture to Support Topology 2 are agreed.

2 Proposal

It is proposed to capture the following changes to TR 23.700-13.

START OF CHANGES

8.1.3	Architecture to Support Topology 2

8.1.3.1	General

Two options will be specified for Topology 2:

-	User-plane option as defined in clause 8.1.3.2.

-	RRC-based option as defined in clause 8.1.3.3.

-	NAS-based option as defined in clause 8.1.3.4

8.1.3.2	User-plane option

The following principles apply:

-	As depicted in Figure 8.1.3.2-1, the UE Reader connects to the AIoTF based on the AIoT Application Protocol (AIoT-AP) using an IP PDU Session between the UE and the UPF as transport. The related protocol stack is shown in Figure 8.1.3.2-2. The AIoT AP protocol will support procedures and information to be exchanged as specified by RAN WG2, RAN WG3 and SA WG2.

Figure 8.1.3.2-1: User-plane architecture for Topology 2

Figure 8.1.3.2-2: Protocol stack for the user-plane architecture for Topology 2

NOTE 1:	Which transport protocol to use for AIoT-AP can be decided by CT WG1 in coordination with RAN WG3.

NOTE 2:	Security for AIoT-AP is assumed to be defined by SA WG3.

Editor's note:	Further details are FFS.

Editor's note:	The terminology, e.g. AIoT RAN is FFS.

8.1.3.3	RRC option

The following principles apply:

-	Messages between the UE Reader and the AIoTF are delivered using RRC and a protocol between gNB/AIoT RAN and AIoTF. The related protocol stack options are shown in Figure 8.1.3.3-1 and Figure 8.1.3.3-2.

Editor's note:	Which option to use, i.e. whether gNB/AIoT RAN connect to AIoTF directly (Option A in Figure 8.1.3.3-1) or via AMF (Option B in Figure 8.1.3.3-2) is FFS.

Editor's note:	How addressing works for UL for Option B is FFS.

Editor's note:	Further details are FFS.

Figure 8.1.3.3-1: Protocol Stack for the RRC option (Option A)

Figure 8.1.3.3-2: Protocol Stack for the RRC option (Option B)

8.1.3.4	NAS option

The following principles apply:

-	Messages are delivered between the UE Reader and the AMF using NAS and between the AMF and the AIoTF using SBI protocol. The related protocol stack options are shown in Figure 8.1.3.4-X and Figure 8.1.3.4-Y.

Figure 8.1.3.4-X:	NAS-based architecture for Topology 2

Figure 8.1.3.4-Y:	Protocol Stack for NAS-based architecture for Topology 2

END OF CHANGES