3GPP TSG SA WG2#166	            S2-2411875

# Orlando, USA, November 18 - 22, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5161 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support cell granularity based local services in 5G Femto |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_Femto |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As RAN3 WG’s recommendation, for local services in 5G femto cell we can reuse LADN mechanisms. Accordingly LADN DNN& S-NSSAI and service area can be provisioned to 5GS as the existing paprameter provisioning procedure as described in 4.15.6.3e, which is TA granularity. However, TA may be not enough room to identify uniquely Femto cell(i.e. serice area of local service). So that we may reuse NW slice having S-NSSAI location availabitity information (i.e. NW slice service area set by the Femto cells only). And, UCU is re-used for provisioning such LADN information and S-NSSAI location availability information to UE. There are two alternatives to restrict the local services on the Femo cell; Alt1. an additional information(e.g. CAG ID) can be provisioned together with service area of DNN and S-NSSAI. Alt2. CAG provisioning information may include implicitly use of CAG(e.g. Femto) |  |  |  |  |  |  |  |  |
| Summary of change: |  | (alternative1) - CAG ID is provisioned together with service area in clause 4.15.6.3e (alternative 2) - CAG usage info is implicitly provisioned together with CAG provisioning information in 4.15.6.3h |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Local services in 5G Femto cell are not supported in cell granularity. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.3e, 4.15.6.3h |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** First Change(Alternative 1) **********

4.15.6.3e	DNN and S-NSSAI specific Group Parameters

The DNN and S-NSSAI specific Group Parameters are the parameters sent from an AF by invoking the Nnef_ParameterProvision Service as described in clause 4.15.6.2.

The DNN and S-NSSAI specific Group Parameters are described in Table 4.15.6.3e-1.

Table 4.15.6.3e-1: Description of DNN and S-NSSAI specific Group Parameters

| Parameters | Description |
| --- | --- |
| Default QoS (NOTE 1) | Identifies the requested default QoS parameters (as defined in clause 5.7.2.7 of TS 23.501 [2]) applicable to each UE within a group. [optional] |
| Service Area (NOTE 1, NOTE xx) | Identifies the AF-requested Service Area applicable to each UE within a group. [optional] For cell granularity additional information(e.g. CAG ID) can be provided together with Service Area. |
| NOTE 1: These parameters are applicable per DNN and S-NSSAI. NOTE xx: Femto cell(s) area can be configured by TA. In addition to support finer granularity for local service, AMF should enforce the NS-AoS based on the Service Area. |  |

********** First Change(Alternative 2) **********

4.15.6.3h	5G Femto CAG provisioning information parameters

The 5G Femto information provisioning parameters are the parameters sent from an AF by invoking the Nnef_ParameterProvision Service as described in clause 4.15.6.2. The AF request can be driven by the 5G Femto CAG owner or an authorized administrator. The 5G Femto information provisioning includes the parameters listed in Table 4.15.6.3h-1. According to the received 5G Femto information provisioning parameters at NEF, the NEF updates the UDM with the provided information. The UDM updates the allowed CAG list of the given UE(s) and notifies AMF via Nudm_SDM_Notification Notify message as defined in clause 4.15.6.2. Then, AMF initiates UE Configuration Update procedure to the UE(s) to update the allowed CAG list of the UE(s) along with the provided validity timer as described in clause 4.2.4.2.

NOTE:	The existing CAG concept defined for PNI-NPN in 5.30.3.3 of TS 23.501 [2] is re-used for 5G Femto information provisioning.

Table 4.15.6.3h-1: Description of 5G Femto information provisioning parameters

| 5G Femto information provisioning Parameters | Description |
| --- | --- |
| UE ID(s) | Indicates the unique identifier for the user to be updated with provided CAG information. It can be GPSI or MSISDN. |
| CAG ID(s) | Indicates the CAG ID(s) to be included to the existing UE Allowed CAG list. |
| Validity time | Indicates the expiry time if the CAG information is only valid temporarily. If no expiry time is present, the provided CAG information will be valid until UE leaves the CAG cells or the next update is performed. |
| NOTE xx: it is assumed that the assigned CAG ID(s) come from Femto dedicated CAG ID partitioned room so that its Femto use is implicitly distinguished in 5GC. NOTE yy: For local service AMF may enforce the NS-AoS based on the CAG cell of the TA, when AMF receives from UDM as the CAG is used for Femto service. |  |

********** End of Changes **********