3GPP TSG-SA WG2#166	S2-2411976

Orlando, US, November 18 – 22, 2024	(Revision of S2-240xxxx)

Source:	Sony

Title:	KI#2: Interim Conclusion on Reader authorization

Document for:	Approval

Agenda Item:	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This contribution proposes interim conclusion on Reader authorization and configuration.

# 1.	Discussion

There are two types of readers studied in Rel-19, a RAN Reader which is a base station with Ambient IoT Reader capabilities and a UE Reader which is a UE with Ambient IoT Reader capabilities. How a Base station is authorized and configured is not within SA2 remit and can be referred to other working groups. However, the UE Reader Authorization and configuration/provisioning shall be specified by SA2. We propose that UE Reader authorization and configuration follow the same procedures as other Service authorization for e.g., 5G ProSe.

# 2.	Text proposal

It is proposed to agree the following changes to TR 23.700-13:

# >>>>BEGINNING OF CHANGES<<<<

### 8.2	Interim Conclusion on Key Issue #2

### 8.2.x	Reader Authorization and configuration

#### 8.2.x.1	Topology 1 – RAN Reader

The Authorization and configuration of a RAN Reader is assumed to be handle by OAM as part of the gNB authorization and configuration.

#### 8.2.x.2	Topology 2 – UE Reader

The following are applicable:

- A UE with UE Reader capability performs legacy registration procedure.

- The UE includes the UE Reader capability in the Registration Request.

- The AMF checks the UE subscription whether the UE is allowed to act as a UE Reader. The AMF stores the result in the UE context.

- The AMF discovers a PCF supporting UE Reader provisioning and establishes a UE policy association with the PCF. The PCF provides the UE with UE Reader Policy/Parameters including e.g. Authorization information.

- The AMF also provides the UE Reader authorized indication in an NGAP message and RAN stores the UE Reader authorized indication in the UE context.

- How RAN configures the UE Reader is up do RAN WGs to specify.

# >>>>END OF CHANGES<<<<