3GPP TSG-SA WG2 Meeting #166	S2-2411990

# Orlando, USA, November 18-22, 2024									     (revision of S2-2410986)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1428 | rev | 11 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of UE-Satellite-UE communication in IMS - Functionality |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, [CATT, Samsung, China Telecom,] Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the approved 5GSAT_Ph3_ARCH work item and the conclusion of FS_5GSAT_Ph3_ARCH captured in clause 8.3 of TR 23.700-29, this CR adds description on Support of UE-Satellite-UE communication in IMS focusing on functionality. Rev.10 ENs are resolved. - List of functionalities is enhanced to include IMS AS. AX.2.2.2 is added. - It is proposed not to specify how the P-CSCF uses the satellite identifier to derive whether optimized media routing is possible. - It is proposed not to consider the UE subscription aspect. - For media continuity at IMS AGW change, a phrase is added that IMS AGW on the source satellite is kept for a while. Description for mobility is added as AX.3.3 and AX.4.3. - For AX 3.3, text taken from the last version of the Maastricht meeting is re-inserted. - For AX.4.3 (and AX.2.2.2), text is written under a proposal that IMS AS, requested by P-CSCF, sends SIP re-INVITE to complete IMS AGW relocation. Editorial change - Redundant phrase in AX.1 is removed. - References to AX 5.1, AX.5.2.1, AX. 5.2.2 are added. (expecting those are finalized during this meeting.) - “to” is missing in AX.3.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of Support of UE-Satellite-UE communication in IMS with regards to functionality. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An objective of the 5GSAT_Ph3_ARCH work item is not achieved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex AX (new), AX.1 (new), AX.2 (new), AX.2.1 (new), AX.2.2 (new), AX.2.2.1 (new), AX.2.2.2 (new), AX.3 (new), AX.3.1 (new), AX.3.2 (new), AX.3.3 (new), AX.4 (new), AX.4.1 (new), AX.4.2 (new), AX.4.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Content of clause AX.2.1 comes from CR1427. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change---

Annex AX (normative):
Support of UE-Satellite-UE communication in IMS

AX.1	General

This annex describes IMS architecture enhancements to support UE-Satellite-UE communication in IMS, i.e., optimized media routing via IMS user plane on-board satellite.

In this Release of the specification, this feature is supported only for IMS voice/video service and for UEs belonging to the same PLMN and in the non-roaming scenario.

The term and the 5GS architecture for UE-Satellite-UE communication is defined in TS 23.501 [93]. The optimized media routing described in this annex refers to a routing of media between UEs under the coverage of the same or of different serving satellites with the media not transiting through any network elements on the ground. The ground fallback routing refers to the case with the media transiting through the ground segment.

The P-CSCF is responsible for determining whether to activate UE-satellite-UE communication, that is, whether it is possible to apply the optimized media routing. P-CSCFs serving the MO UE and the MT UE negotiate with each other to determine whether to activate UE-satellite-UE communication.

AX.2	Architecture and functional entities

AX.2.1	Architecture

<void>

AX.2.2	Functional entities

AX.2.2.1		P-CSCF

The P-CSCF used for this feature is enhanced to support the following functionalities:

-	the P-CSCF determines the activation of optimized media routing and interacts with 5GS, as described in Annex AX.3.

-	the P-CSCF interacts with IMS AGW, as described in Annex AX.4.

AX.2.2.2	IMS AS

The IMS AS used for this feature is enhanced to support the following functionalities:

-	the IMS AS, requested by P-CSCF used for this feature, initiates SIP re-INVITE to complete IMS AGW relocation, as described in Annex AX.4.

AX.3	Optimized media routing activation

AX.3.1	General

If UE-Satellite-UE communication in IMS is supported, and if enabled subject to operator policy, the following behaviour described in Annex AX.3 applies.

AX.3.2	At call setup

The P-CSCF receives from the PCF the identifier of the satellite serving the UE and receives from the P-CSCF serving the remote UE the identifier of the satellite serving the remote UE as per Annex AX.5.1. The P-CSCF uses this information to determine whether or not to activate optimized media routing.

NOTE:	In this Release of the specification, how the P-CSCF uses the satellite identifier to derive whether optimized media routing is possible is not specified, and UE subscription aspect is not taken into account for activation of the optimized media routing.

If P-CSCF determines the activation of optimized media routing, the P-CSCF instructs PCF to authorize the necessary resources as per Annex AX.5.1, so that the PCF proceeds to establish a path for optimized media routing.

AX.3.3	At change of satellite

The P-CSCF determines whether optimized media routing continues to be possible between a satellite that is about to serve the local UE and the satellite serving the remote UE based on the identifiers of those satellites. The P-CSCF obtains the identifier of the new satellite from PCF as per Annex AX.5.2.1.

If the P-CSCF determines that optimized media routing can continue at change of satellite, the P-CSCF sends a message to the PCF as defined in TS 23.502 [94] and as per Annex AX.5.2.1 to establish a path on the target satellite for optimized media routing.

If the P-CSCF determines that optimized media routing is no longer possible e.g. due to a change of satellite, the P-CSCF executes the ground fallback procedure as per Annex AX.5.2.2 to reroute the media via the ground.

AX.4	Interaction with IMS AGW

AX.4.1	General

If UE-Satellite-UE communication in IMS is supported, and if enabled subject to operator policy, the following behaviour described in Annex AX.4 applies.

AX.4.2	At call setup

P-CSCF selects an IMS AGW on ground at call setup according to TS 23.334 [74] and proceeds with the determination of whether UE-Satellite-UE communication is possible as per Annex AX.5.1. If the P-CSCF determines the activation of optimized media routing is possible, the P-CSCF releases the IMS AGW on ground when needed and selects instead an IMS AGW on satellite according to TS 23.334 [74] and as per Annex AX.5.1.

AX.4.3	At change of satellite

When P-CSCF is informed that the satellite serving UE changes as defined in TS 23.502 [94] and as per Annex AX.5.2.1, the P-CSCF initiates to establish an IMS AGW on the target satellite as per Annex AX.5.2.1 or on ground as per Annex AX.5.2.2, while keeping the IMS AGW on the source satellite, depending on whether optimized media routing continues to be possible or not.

As the last step of establishment of the IMS AGW on the target satellite or on ground, the P-CSCF requests the IMS AS to initiate SIP re-INVITE to inform the local UE and the remote network to use the newly established IMS AGW(s) as per Annex AX.5.2.1 and AX.5.2.2.

## ---End of the Change---