3GPP TSG-WG SA2 Meeting #166	S2-2412004

# 18-22 Nov, 2024, Orlando, US		(revision of S2-2410978, 0670)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0574 | rev | 7 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LMF enhancements for LMF-based AI/ML Positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi, [Huawei], [HiSilicon], [NTT DOCOMO], [Nokia] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Regarding the following EN in clause 5.18, we introcude a new procedure of Location Measurement data collection from LMF for NWDAF side AI/ML positioning ML model training. Editor's note: The procedure of NWDAF containing MTLF collects input data for LMF-based AI/ML Positioning ML model training is FFS. Clarify that the LMF can request/subscribe to ML model from NWDAF based on the conclusions for KI#1 in TR 23.700-84. When the NWDAF containing MTLF determines to train a ML model for LMF-based AI/ML Positioning, NWDAF needs to get data from LMF. However, currently, the LMF does not have services for exposing the positioning measurement data. Therefore, we propose to define new Nlmf_DataExposure_Subscribe, Nlmf_DataExposure_Notify, and Nlmf_DataExposure_UnSubscribe services for the data collection for NWDAF side AI/ML positioning ML model training. It proposes to remove the EN, since PRU has already associated with LMF, when area of interest (AoI) is provided by NWDAF, LMF will select any UE in the area (AoI) with positioning capability, if PRU(s) is located in the area (AoI), AMF will send the SUPI list in the area including UE and PRU(s). don’t see the need of PRU existence indication to discovery LMF. Editor's note: Whether the NWDAF needs to be aware of PRU information and indicate to obtain the PRU information from LMF is FFS. Whether the NWDAF needs to indicate Data source type (UE or RAN measurements) and/or data type(s) and/or a target ground truth label precision is also FFS. It proposes the case, that dedicated UE(s) is pre-configured for data collection for model training/model performance monitoring in NWDAF, in this case, NWDAF only needs to send the SUPI(s) to LMF. LMF starts to collect the data from the specified UE(s) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new procedure of Location Measurement data collection from LMF for NWDAF side AI/ML positioning ML model training. Define new LMF services for the data collection for NWDAF side AI/ML positioning ML model training. Other enhancements based on conclusions for KI#1 in TR 23.700-84. To remove PRU information and related part of the EN in step 2, whether need the PRU information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The exposure of LMF Location Measurement data and other enhancements based on KI#1 conclusions in TR 23.700-84 are not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.X (new), 4.4.13, 6.X.4 (new), 8.3.1, 8.3.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Regarding the clause 6.X.4, it should be treated together with TS 23.273 CR0584 and CR0597. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (new) * * * *

4.3.X	Network Data Analytics Function, NWDAF

LMF may provide input data to NWDAF for ML model training or ML model performance monitoring for LMF-based AI/ML Positioning.

LMF may obtain ML model from NWDAF for performing AI/ML Positioning.

# * * * * Next change * * * *

4.4.13	NL8 Reference Point

The NL8 reference point supports LMF to receive location related analytics from NWDAF, provide input data for model training or model performance monitoring to NWDAF, or receive AI/ML positioning model from NWDAF, as defined in TS 23.288 [37].

# * * * * Next change (new) * * * *

6.X.4	Input data collection by NWDAF for AI/ML positioning ML model training or ML model performance monitoring

The NWDAF containing MTLF may subscribe to input data (i.e. location measurement related data) from LMF for ML model training or ML model performance monitoring for LMF-based AI/ML Positioning.

Editor's note: 	It’s FFS how to adapt this procedure to ML model performance monitoring.

Figure 6.X.4-1: Procedure of input data collection from LMF

1.	NWDAF containing MTLF determines to train a ML model for LMF-based AI/ML Positioning based on the request from LMF or internal trigger, or the NWDAF containing MTLF determines to perform ML model performance monitoring for LMF-based AI/ML Positioning.

2.	The NWDAF invokes an Nnrf_NFDiscovery_Request service operation to an NRF to discover an LMF, the service operation includes an AoI and the Nlmf_DataExposure service as discovery parameters.. The NRF selects one or more LMFs based on the AoI, the Nlmf_DataExposure service, , and sends an Nnrf_NFDiscovery_Request Response which includes the profiles of the selected LMFs to the NWDAF.

Editor's note: 	The input data used for LMF-based AI/ML Positioning will be decided by RAN WGs.

3.	The NWDAF subscribes to or cancels subscription to input data from LMF by invoking Nlmf_DataExposure_Subscribe / Nlmf_DataExposure_UnSubscribe service operation. The NWDAF includes an AoI and a notification target address to request the input data from LMF. The NWDAF may also include requested number for data samples and time window for data collection. When specific UE(s) are pre-configured in NWDAF for data collection for ML model training/performance monitoring, NWDAF sends the SUPI(s) to LMF, step 2 is skip.

Editor's note:	. Whether the NWDAF needs to indicate Data source type (UE or RAN measurements) and/or data type(s) and/or a target ground truth label precision is also FFS.

4.	The LMF performs steps 1 to 4 in clause 6.X.2, or steps 1 to 7 in clause 6.X.3.

5.	The LMF sends the collected data samples to the NWDAF by invoking Nlmf_DataExposure_Notify service operation, then the NWDAF trains the ML model or performs ML model performance monitoring based on the data samples.

# * * * * Next change * * * *

8.3.1	General

The following table shows the LMF Services and LMF Service Operations.

Table 8.3.1-1: List of LMF Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nlmf_Location | DetermineLocation | Request/Response | AMF |
|  | EventNotify | Notify | GMLC |
|  | CancelLocation | Request/Response | AMF |
|  | LocationContextTransfer | Request/Response | LMF |
|  | MeasurementData | Request/Response | LMF |
|  | UPConfig | Request/Response | AMF |
|  | UPSubcribe | Subscribe/Notify | AMF |
|  | UPNotify |  | AMF |
| Nlmf_Broadcast | CipheringKeyData | Notify | AMF |
| Nlmf_DataExposure | Subscribe | Subscribe/Notify | NWDAF |
|  | Notify |  | NWDAF |
|  | UnSubscribe |  | NWDAF |

# * * * * Next change * * * *

8.3.X	Nlmf_DataExposure service

8.3.X.1	General

Service description: This service enables an NF service consumer to subscribe to input data for model training or model performance monitoring. See Clause 6.X.4 for examples of usage of this service.

8.3.X.2	Nlmf_DataExposure_Subscribe service operation

Service operation name: Nlmf_DataExposure_Subscribe

Description: Allow the consumer NF to subscribe the input data used for ML model training or ML model performance monitoring for LMF-based AI/ML Positioning.

Input, Required: Notification Target Address, Notification Correlation ID, AoI.

Input, Optional: time window when to collect data, number of samples, Subscription Correlation ID (in the case of modification of the subscription).

Editor's note:	Whether the consumer NF (i.e. NWDAF) needs to indicate the data source type (i.e., UE location measurements or NG RAN location measurements) and/or data type(s) is FFS, which needs to be coordinated with RAN WGs. A PRU indication and/or a data sample precision threshold is also FFS.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: None.

When the requested number of samples has been provided or the time window for data collection has elapsed, the subscription is automatically terminated.

8.3.X.3	Nlmf_DataExposure_Notify service operation

Service operation name: Nlmf_DataExposure_Notify

Description: Allow the consumer NF to get notification about the input data used for ML model training or ML model performance monitoring for LMF-based AI/ML Positioning.

Input, Required: Notification Correlation ID.

Input, Optional: data samples (i.e. location measurement related data), Subscription Correlation ID (this parameter shall be present if the notification is for informing the assignment of a new Subscription Correlation ID by the LMF).

Editor's note:		The data samples used for LMF-based AI/ML Positioning will be decided by RAN WGs.

Output, Required: Operation execution result indication.

Output, Optional: None.

8.3.X.4	Nlmf_DataExposure_UnSubscribe service operation

Service operation name: Nlmf_DataExposure_UnSubscribe

Description: Allow the consumer NF to cancel subscription to input data used for ML model training or ML model performance monitoring for LMF-based AI/ML Positioning.

Input, Required: Subscription Correlation ID.

Input, Optional: None.

Output, Required: Operation execution result indication.

Output, Optional: None.

# * * * * End of changes * * * *