3GPP TSG-WG SA2 Meeting #166	S2-2412049

Orlando, Nov 18 – 22, 2024	(revision of S2-24xxxxx)

Source:	Nokia

Title:	Conclusions for KI#3

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: conclusions for KI#3 are proposed.

# 1. Introduction/Discussion

This contribution is prepared to update the conclusions for KI#3.

- It is proposed to remove the temporary disable/enable services as there are no solutions available to support those services. This can be discussed again in the next release.

- The principle of the procedure based on the indirect RRC architecture option is proposed.

- Information that needs to be provided by the AF, how to select and authorize the readers are newly proposed.

# 2. Text Proposal

It is proposed to approve the following changes in TR 23.700-13 v1.1.0.

FIRST CHANGE

## 8.3	Interim Conclusion on Key Issue #3

### 8.3.1	General

The clause concludes the following aspects:

-	Study how to support information transfer for Ambient IoT services and related system functionality, including the information transfer for an Ambient IoT device and for a group of Ambient IoT Devices.

NOTE:	The above aspect includes studying whether there is a need to support session based transfer between Ambient IoT Device and the network considering the device types and capabilities.

-	Study which of the enabled Ambient IoT services are exposed to AF and how, e.g. for the case AF requests Ambient IoT service for an Ambient IoT Device and for a group of Ambient IoT Devices.

### 8.3.2	AIoT services supported by the 5GC

The following Ambient IoT services are agreed to be supported by the 5GC, which apply to both topology 1 and topology 2:

-	Inventory: Request to perform an inventory operation.

-	Read: Request to read information from an AIoT Device.

-	Write: Request to write information to an AIoT Device.

-	Disable: Request that an AIoT Device has its capability to transmit RF permanently disabled.

### 8.3.3	NEF exposure

NEF supports to expose Ambient IoT service towards the AF.

-	New NEF services (e.g., Nnef_AIoTInventory, Nnef_AIoTCommand) to support the AIoT services will be defined.

-	New events (e.g., AIoT reader overload, availability of AIoT UE readers by location) will be defined for exposure by the NEF to external AFs to support AF logic.

### 8.3.4	Principles on the procedures to support AIoT services

- An AF sends an AIoT Service Request message to NEF. The request may include information as concluded in clause 8.3.5.

NOTE 1:	In this release, the AIoT service request is always initiated by the AF.

- The NEF authorizes the AF request to determine whether the AF is authorized to perform AIoT services.

- The NEF discovers and selects the AIOTF based on the information received from the AF in step 1.

- The NEF forwards the AIoT Service Request message to the AIOTF.

- A Session ID is allocated by the AIOTF for each AF-requested AIoT service.

- The AIOTF discovers and selects one or more RAN Reader(s)/UE Reader(s) based on the information received from the NEF, as well as other network information. The details of AIoT Reader selection and AIoT Reader authorization are specified in clause 8.3.6 and clause 8.3.7, respectively. For topology 2, the selected UE(s) is configured with the necessary information for communicating with the AIoT devices and the CN, including the UE role and the aggregation policy.

- The AIOTF forwards the AIoT Service Request message to the RAN Reader via AMF in topology 1 or via AMF and gNB in topology 2.

- The RAN Reader/UE Reader sends the AIoT service request message to the AIoT devices. To reach a group of AIoT devices, device ID MASK information can be used. For targeting one or more specific AIoT devices, device IDs are used.

NOTE  2:	It is up to SA3 to determine whether there are security/privacy concerns when using the device ID or MASK information in A-IoT paging. And if there are, it’s up to SA3 to determine how to resolve them.

- The AIoT device compares the received device filter info (i.e., MASK or Device ID) with its corresponding ID.

- If the AIoT device matches the device filter info, it may perform a Random Access procedure.

NOTE 3:		The steps 8 to 10 are to be defined by RAN2.

- The AIoT device sends the AIoT Service Response message to the AIOTF. For Inventory service operations, the response message includes the Device ID. For Command service operations, the response message includes the Device ID and Command Result information.

In topology 1, the BS Reader forwards the received AIoT device response(s) to the AIOTF over the N2 interface.

In topology 2, the UE Reader forwards the received device response(s) to the AIOTF, over the CP path with RRC and NGAP. The data read from the AIoT devices can be aggregated by the UE reader(s).

- The AIOTF creates and maintains a binding table that indicates the association information between the AIoT devices and their AIoT Reader to avoid performing the AIoT Reader selection procedure each time the 5GC receives an AIoT request for the same AIoT devices. The AIOTF may distinguish and remove duplicated Ambient IoT device information before forwarding it to the NEF.

- The AIOTF sends the AIoT Service Response to the AF via NEF.

### 8.3.5	Information provided by the AF

The AF provides one or some of the following information:

- Information about AIoT services

  - service type indicating e.g., inventory or command

- Information to be used for AIoT reader selection

  - at least one of UE reader ID and target location

- Information about the target AIoT device(s)

  - device IDs

  - filtering information

### 8.3.6	AIoT reader selection

For topology 1, the BS reader(s) are selected by AIOTF based on the target area information.

For topology 2,

- When the target reader ID is provided, the AIOTF validates whether the requested UE reader is reachable and, if reachable, selects the UE reader as requested.

- When target area information is provided, the AIOTF pre-selects a list of candidate UE readers based on location information, reachability, and per-AF authorization. The pre-selected list is then provided to the gNB, which performs a down-selection using RAN-level information.

NOTE :	The specific method by which the gNB narrows down the list will be determined by RAN3.

### 8.3.7	AIoT reader authorization

For topology 1, there is no authorization required for a RAN Reader

For topology 2, a UE Reader needs to be authorized before it performs the AIoT services captured in 8.3.2.

- UE Readers report their AIoT support capability to the AMF and are authorized by the AMF during the registration procedure.

END OF CHANGES