SA WG2 Meeting #166 	S2-2412053

# 18 - 22 November 2024, Orlando, USA									  (revision of S2-2410995

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1253 |  | 4 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support AI/ML model performance monitoring by NWDAF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Xiaomi, Nokia, Ericsson], Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-11-8 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 5.18 of TS 23.273, Either LMF or NWDAF containing MTLF may perform performance monitoring for LMF-based AI/ML Positioning. LMF may determine whether to use the LMF-based AI/ML Positioning to perform location calculation based on the model performance monitoring result. LMF may also trigger the ML model retraining in the training entity based on the result of model performance result. Editor's note: The Performance monitoring procedures are FFS. In this paper it proposes the procedure for NWDAF containing MTLF to perform ML performance monitoring. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To add feature description for AI/ML Model performance monitoring in NWDAF side by referring to data collection by NWDAF with additional enhancements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AIML_CN feature is not completed in Rel-19 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2E.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 23.273 CR 0574 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 4: Adds description that the NWDAF is triggered by the LMF to monitor performance of an ML Model and simplifies the wording for input data. The NWDAF obtains input data from the LMF which include ground truth data from PRUs/UEs and inference output from the LMF |  |  |  |  |  |  |  |  |

* * * First of Change * * *

### 6.2E.X	Procedure for MTLF-based AI/ML model performance monitoring for LMF-based AI/ML Positioning

The LMF requests the NWDAF to monitor the ML model performance by including in the request the ML model identifier of the ML model whose performance needs to be monitored.

The NWDAF containing MTLF  monitors the ML Model performance by collecting the following input data from the LMF:

-	Ground truth information from PRU(s) and/or UE(s) by using the procedure  specified in clause 6.X.4 of TS 23.273 [39].

-	The inference output (location estimation using Direct AI/ML positioning) of the PRU and/or UE providing the ground truth data.

Before collecting the input data from UE(s), LMF needs to check the user consent in UDM during the data collection procedure.

The NWDAF containing MTLF evaluates the ML Model performance by comparing the ground truth data against the location estimation using Direct AI/ML positioning. The result may trigger NWDAF containing MTLF to retrain the ML model, or sent to LMF to trigger LMF to change the positioning method, e.g., from LMF-based AI/ML Positioning to legacy positioning.

* * * End of Changes * * *