3GPP TSG-SA WG2#166	S2-2412055

Orlando, FL,USA, 18-22 November 2024	(was S2-240xxxx)

Source:	NEC

Title:	Handling of data transmission during NTN to TN mobility.

Document for:	Discussion/Approval

Agenda Item:	19.1.2

Work Item / Release:	5GSAT_Ph3_ARCH/Rel.19

Abstract of the contribution: Discusses handling of data transmission during mobility procedure from S&F NTN cell to TN cell.

# 1. Introduction

This discussion paper discusses a scenario where there is a possibility that MO data can be lost during the mobility procedure between cell supporting S&F and normal cell of a Terrestrial Network (TN) or a NTN network which has service link and feeder link always connected. The UE is not aware that the data is lost. The detailed scenario is discussed as below.

# 2.	Discussion

In the current S&F data transmission mechanism when the service link is available, and the UE sends user data to the network it is stored in the MME-SAT. When the feeder link is available the MME-SAT forwards this data to MME-GND and the MME-GND further forwards this data to the SCEF or AF. There can be a scenario when before the MME-SAT connects to the MME-GND (depending on the deployment of satellite constellation it can take hours to get feeder link available), the UE selects a cell of Terrestrial Network (TN) and get attached to the Terrestrial network. The HSS will cancel the UE location to the MME-GND and a UE context including active PDN connections are removed from the MME-GND. By the time when the feeder link is available then the MME-SAT transfer the stored data the UE context is already deleted and hence the user data is lost, and the UE is not aware of it.

The detailed scenario is as described below

1. A UE with S&F operation mode enabled is attached to the network supported S&F mode of operation. A PDN connection is active.

2 A service link is available, and the UE sends a MO data to a PDN connection.

3. The MME-SAT stores the user data.

4. The UE selects a TN cell and registers to the TN network.

5. After the registration procedure to the TN network, the HSS cancel the UE location to the MME-GND.

6. The MME-GND removes the UE context including the active PDN connections.

7. When the feeder link is available, the user data sent in step 2 is given to the MME-GND.

8. The MME-GND have deleted the UE context and will not find any UE context for this user data.

9. Therefore this user data will be lost and the UE is not aware of it.

# 2.	Proposal

SA2 discusses this scenario and captures an ENs in TS to handle this scenario in subsequent SA2 meeting when normative work is done for the basic scenarios.