3GPP TSG-WG SA2 Meeting #166	S2-2411020

# Orlando, USA, 18th Nov – 22nd Nov, 2024												(was S2-2411020)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2136 | rev | 7 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Identifying non-3GPP devices behind 5G-RG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [CableLabs, Nokia, InterDigital, Charter Communications, Deutsche Telekom], Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To specify the support for identifying non-3GPP devices connecting behind a 5G-RG based on the conclusions in clause 8.4 of TR 23.700-32 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specifying conclusions on identification of non-3GPP device for wireline access |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New feature not implemented in the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 4.10b, 4.10x(new), 9.5.2.1, Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | ‘Clause 5.x’ in this CR’s clause 4.10x refers to the clause 5.x in CR 5750 of TS 23.501 ‘Clause 5.9.x’ in this CR’s clause 4.10x refers to the clause 5.9.x in CR 5749 of TS 23.501 |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], TS 23.501 [2] , TS 23.502 [3] and TS 23.503 [4] apply. A term defined in TS 23.501 [2], TS 23.502 [3] or TS 23.503 [4] takes precedence over the definition of the same term, if any, in any other specifications.

RG Level Wireline Access Characteristics: Wireline access technology specific QoS information corresponding to a specific wireline access subscription, which is provided by the AMF to the W-AGF at RG registration.

Wireline access Control Plane protocol (W-CP): Protocol used to transport AS and NAS signalling between the 5G-RG and the W-AGF over the Y4 reference point. W-CP is specified by BBF and CableLabs. There is no assumption that W-CP refers to only a single protocol or only a specific protocol layer.

Wireline access User Plane protocol (W-UP): Protocol used to carry PDU Session user plane traffic between the 5G-RG and the W-AGF over the Y4 reference point. W-UP is specified by BBF and CableLabs. There is no assumption that W-UP refers to only a single protocol or only a specific protocol layer.

Legacy Wireline access Control Plane protocol (L-W-CP): L-W-CP is a legacy control plane protocol between the FN-RG and W-AGF. L-W-CP is specified by BBF and CableLabs. There is no assumption that L-W-CP refers to only a single protocol or only a specific protocol layer.

Legacy Wireline access User Plane protocol (L-W-UP): L-W-UP is a legacy user plane protocol between the FN-RG and W-AGF. W-UP is specified by BBF and CableLabs. There is no assumption that L-W-UP refers to only a single protocol or only a specific protocol layer.

Authenticable Non-3GPP (AUN3) device: A device that does not support NAS signalling, is connected to 5GC via a RG and can be authenticated by 5GC over the RG.

5GS specifications do not support a device using the same subscription to access 5GS as a UE and as an AUN3 device.

Non-3GPP Device Identifier: see definition in TS 23.501

Non-Authenticable Non-3GPP (NAUN3) device: A device that does not support NAS signalling, is connected to 5GC via a RG and for which authentication with 5GC is not supported.

NOTE 1:	AUN3 and NAUN3 device can connect to RG through WLAN (collocated or not collocated with the RG) and/or wired Ethernet connections.

NOTE 2:	A device can operate as a UE over NG-RAN and as a AUN3 or NAUN3 or a non-3GPP device binds to a Non-3GPP Device Identifier for QoS differentiation by 5GC via a RG, if the device implements UE functionality.

# * * * * 2nd change * * * *

## 4.10b	Differentiated services for NAUN3 devices behind 5G-RG

NAUN3 devices cannot be authenticated by 5GC but may e.g. be locally authenticated by the 5G-RG using e.g. pre-shared secret. Differentiated services (QoS, network slicing) may be provided for NAUN3 devices as defined in this clause.

"Connectivity Group IDs" may be defined on the 5G-RG where each Connectivity Group ID corresponds to a separate physical or virtual port on the 5G-RG. These ports could, for example, refer to separate physical ethernet ports and/or to separate WLAN SSIDs and/or to a separate VLAN. The devices that connect to a certain logical port are considered part of the same Connectivity Group ID. How this configuration on the 5G-RG is done is out of scope of this specification.

Each Connectivity Group ID is then mapped to a separate PDU Session that is established by the 5G-RG based on the procedures defined in clause 7. The overall architecture is illustrated in Figure-4.10b-1.

Figure 4.10b-1: Example scenario for NAUN3 devices behind 5G-RG based on connectivity groups

The 5G-RG is configured with the (virtual) port information (e.g. VLANs and SSIDs) based on TR-69 [18], TR-360 and TR-181 [46]. URSP rules can be provided to the 5G-RG to indicate how to map Connectivity Group ID to the parameters of the PDU Session used to carry the traffic of corresponding devices e.g. DNN, S-NSSAI, etc.

NOTE:	In addition, the mapping between a "virtual port" and DNN/S-NSSAI can be configured via TR-69 [18]/TR-181 [46].

Whether and how the NAUN3 devices are configured to use a specific SSID or connect to a certain Ethernet port on the 5G-RG is out of scope of this specification.

Differentiation of charging and QoS may be provided via PCC rules (for different service flows) related with dedicated PDU Sessions for NAUN3 devices. Isolation of devices using a specific Connectivity Group ID into a specific network slice, i.e. with separate S-NSSAI may also be provided.

How the 5G-RG determines that a Non-3GPP device connecting behind a 5G-RG is considered as a NAUN3 or a Non-3GPP device that binds to a Non-3GPP Device Identifier for QoS differentiation as defined in clause 4.10x is outside the scope of 3GPP. The 5G-RG may change whether a Non-3GPP device is treated, i.e. from NAUN3 to Non-3GPP device bound to a Non-3GPP Device Identifier and viceversa, based on mechanism not defined by 3GPP, but it is assumed that a Non-3GPP device connected behind a 5G-RG  never plays both roles simultaneously.

# * * * * 3nd change * * * *

## 4.10x	Differentiated QoS for non-3GPP devices behind 5G-RG

This clause defines the support for identifying the traffic of individual non-3GPP devices behind a 5G-RG and providing them differentiated QoS.

TS 23.501 [2] clauses 5.9.x and 5.x apply to the 5G-RG with the following deltas:

-	The UE is replaced by 5G-RG.

How the 5G-RG determines that a Non-3GPP device connecting behind a 5G-RG is considered as a NAUN3 as defined in clause 4.10a or a Non-3GPP device binds to a Non-3GPP Device Identifier for QoS differentiation is outside the 3GPP scope.

The 5G-RG may change how a Non-3GPP device is considered, i.e. from NAUN3 to Non-3GPP device binds to a Non-3GPP Device Identifier  and viceversa based on mechanism outside the 3GPP scope, but it is assumed that a Non-3GPP connected behind a 5G-RG  never plays both roles at the same time.

Figure 4.10x-1 illustrates an example scenario for the mapping of traffic from individual non-3GPP devices behind 5G-RG to a PDU Session. Non-3GPP devices associated with the same PDU Session can be further differentiated using their Non-3GPP Device Identifiers.  As in this example, two non-3GPP devices mapped to PDU Session A initially used the default QoS Flow (QFI 1); when differentiated QoS is requested for one device, the 5G-RG binds its traffic to a Non-3GPP Device Identifier and its traffic is mapped to a separate QoS Flow (QFI 2). Four non-3GPP devices mapped to PDU Session B based on their Connectivity Group ID X initially used the default QoS Flow (QFI 3); when differentitated QoS is requested for two of those four devices, the 5G-RG binds their traffic to Non-3GPP Device Identifiers and their traffic is mapped to separate QoS Flows (QFI 4 and QFI 5). Similarly, three non-3GPP devices mapped to PDU Session C based on their Connectivity Group ID Y initially used the default QoS Flow (QFI 6); when differentiated, but the same, QoS is requested for two of those three devices, the 5G-RG binds their traffic to Non-3GPP Device Identifiers and their traffic is mapped to a separate QoS Flow (QFI 7).

Figure 4.10x-1: Example scenario for mapping traffic from individual non-3GPP devices behind 5G-RG to a PDU Session

# * * * * 4nd change * * * *

#### 9.5.2.1	5G-RG

This clause specifies the delta related to UE policy distribution defined in TS 23.503 [4] clause 6.1.2.2 and related to URSP defined in TS 23.503 [4] clause 6.6. for 5G-RG.

Only URSP policies are applicable for a 5G -RG.

NOTE:	For example, ANDSP policies, ProSe Layer-3 UE-to-Network Relay Offload Policy are not applicable to 5G-RG,

If the PCF provides a URSP policy to the 5G-RG, the PCF should only provide the following URSP policy information:

-	Rule Precedence.

-	Traffic descriptor/Application descriptors, IP descriptors, Domain descriptors, Non-IP descriptors, DNN, Connection Capabilities.

-	Route Selection Descriptor Precedence.

-	Route selection components/ SSC Mode Selection, Network Slice Selection, DNN Selection, PDU Session Type Selection.

-	Route Selection Validation Criteria/ Time Window.

The 5G-RG shall ignore any of the parameters and policies not listed above, if received from the 5GC.

The 5G-RG shall use the URSP policy as specified in TS 23.503 [4], for example for the association of application and PDU session, slices, etc.

The URSP indicates for the application of Auto-Configuration Server (ACS) which PDU session type, NSSAI and/or DNN is to be used. The 5G-RG establishes the connectivity to the management entity (e.g. ACS) via user plane connection on a PDU session according to the URSP.

UE Policy procedures defined in clause 6.1.2.2 of TS 23.503 [4] are applicable as follows:

-	Roaming is not applicable to W-5GAN access in this release of specification.

-	USRP rule enforcement reporting is not defined for 5G-RG.

In the case of an Authenticable Non-3GPP (AUN3) device behind a 5G-RG only URSP policies are applicable. Each AUN3 device is managed in 5G-RG as a separate device with its own NAS and states, therefore from point of view of 5GC, the AUN3 device is seen as a single device and the URSP received in the NAS dedicated to an AUN3 device shall apply to this AUN3 device only the 5G-RG shall apply URSP received within NAS for AUN3 device acting on behalf of AUN3 device. Therefore to support the case when AUN3 devices may be connected via 5G-RG, specific URSP rules may be configured by the PCF for the SUPI associated with the AUN3 device.

UE Route Selection Policy information targeting an AUN3 device (i.e. sent to a 5G-RG in the NAS connection corresponding to an AUN3 device) follows the structure defined in clause 6.2.2 of TS 23.503 [4] with the following difference:

-	As an AUN3 can have only one PDU Session, its URSP shall contain a match all TD.

USRP rule enforcement reporting is not defined for AUN3 devices.

In order to support the case when NAUN3 devices may be connected via 5G-RG, specific URSP rules may be configured by the PCF on 5G-RG.

URSP rules for NAUN3 devices connected to 5G-RG follow the structure defined in clause 6.6.2 of TS 23.503 [4] and may contain any combination of the following traffic descriptors:

-	IP Descriptors: For IP traffic from NAUN3 devices connected to 5G-RG, IP descriptors are matched against header information contained in IP packets sent by NAUN3 devices; IP descriptors are only applicable for traffic from NAUN3 devices if network address translation (NAT) is performed for that traffic.

-	Non-IP descriptors: For Ethernet traffic from NAUN3 devices connected to 5G-RG, Non-IP descriptors are matched against header information contained in Ethernet frames sent by NAUN3 devices.

-	Connectivity Group ID: For traffic from a NAUN3 device connected to 5G-RG, Connectivity Group ID in the URSP rule is matched against the Connectivity Group ID that the NAUN3 device is associated with (see clause 4.10b).

In order to support the case when a Non-3GPP device connected via 5G-RG and it is bind by 5G-RG  to the Non-3GPP Device Identifier specific URSP rules may be configured by the PCF on 5G-RG. URSP rules for Non-3GPP Device Identifier follow the structure defined in clause 6.6.2 of TS 23.503 [4] and may contain any combination of the following traffic descriptors:

-	IP Descriptors: For IP traffic binds to Non-3GPP Device Identifier from the Non-3GPP device connected to 5G-RG, IP descriptors are matched against header information contained in IP packets sent by Non-3GPP devices; IP descriptors are only applicable for traffic from Non-3GPP devices binds to to Non-3GPP Device Identifier if network address translation (NAT) is performed for that traffic.

-	Non-IP descriptors: For Ethernet traffic binds to Non-3GPP Device Identifier from the Non-3GPP device connected to 5G-RG, Non-IP descriptors are matched against header information contained in Ethernet frames sent by Non-3GPP devices.

-	Non-3GPP Device Identifier: For traffic binds to Non-3GPP Device Identifier from a Non-3GPP device connected to 5G-RG, Non-3GPP Device Identifier in the URSP is matched against the Non-3GPP Device Identifier that the Non-3GPP device traffic is associated with (see clause 4.10x)

# * * * * 5nd change * * * *

Annex X (informative):
Support for differentiated QoS for Non-3GPP device behind 5G RG

This Annex describes how QoS differentiation for Non-3GPP device behind 5G-RG can be provided to a Non-3GPP device considered as NAUN3 defined in clause 4.10b or as Non-3GPP device binds to Non-3GPP Device Identifier defined in clause 4.10x.

FigureAnnex X.1x-1 illustrates an example scenario for the mapping of traffic from individual non-3GPP devices behind 5G-RG to a PDU Session. Non-3GPP devices associated with the same PDU Session can be further differentiated using their Non-3GPP Device Identifiers.  As in this example, two non-3GPP devices mapped to PDU Session A initially used the default QoS Flow (QFI 1); when differentiated QoS is requested for one device, the 5G-RG binds its traffic to a Non-3GPP Device Identifier and its traffic is mapped to a separate QoS Flow (QFI 2). Four non-3GPP devices mapped to PDU Session B based on their Connectivity Group ID X initially used the default QoS Flow (QFI 3); when differentitated QoS is requested for two of those four devices, the 5G-RG binds their traffic to Non-3GPP Device Identifiers and their traffic is mapped to separate QoS Flows (QFI 4 and QFI 5). Similarly, three non-3GPP devices mapped to PDU Session C based on their Connectivity Group ID Y initially used the default QoS Flow (QFI 6); when differentiated, but the same, QoS is requested for two of those three devices, the 5G-RG binds their traffic to Non-3GPP Device Identifiers and their traffic is mapped to a separate QoS Flow (QFI 7).

How the Non-3GPP device is identified by 5G-RG is considered not in the 3GPP scope and it may be either implementation specific (for example based on device MAC address) or specified by other SDOs such as BBF.

The same Non-3GPP device, as in the example, can be either considered as NUN3 device per clause 4.10b or as a Non-3GPP device per clause 4.10x, but it can not assume both roles at the same time. The change of role does not require any actions from the 3GPP network which only see either a NAUN3 or a Non-3GPP device Identifier and in both case the 3GPP network is not aware of any specific identifiers used by 5G-RG to identify the Non-3GPP device role.

Figure Annex X.x-1: Example scenario for mapping traffic from individual non-3GPP devices behind 5G-RG to a PDU Session

# * * * * End of changes * * * *