SA WG2 Meeting #166	S2-2412148

# 14 - 18 November 2024, Orlando, USA

Source:	Xiaomi

Title:	Discussion on UE's user plane availability status in LCS-UPP

Agenda item:	9.6.2

Document for:	DISCUSSION

- Introduction

In S2-2411289, an LS from CT1 about UE's user plane availability status for LCS-UPP has been received. CT1 asks SA2 to evaluate and guide on the issue that the LMF is not fully aware of the UE's user plane availability changes. This discussion paper evaluates why it is necessary for an LMF to be aware of UE's user plane availability changes, and proposes 3 options for LMF to obtain the UE's user plane availability status.

- Background

In 3GPP TS 24.572, a failure cause value "user plane not available" is introduced. When the user plane of the UE is "not available", the UE is not able to use the user plane positioning method for location services. Being not aware of UE's user plane availability, LMF may still attempt to establish a user plane connection with a UE whose user plane is not available, which is the waste of signaling, especially the air interface resources.

LMF is aware of UE's user plane availability changes from "available" to "not available" in following ways:

-	For the case of “no available user plane connection between UE and LMF”, When the user plane of the UE is unavailable, the LMF is unaware of it. When LMF attempts to establish a user plane connection to the UE, the UE rejects the request by indicating "user plane not available" via the user plane connection establishment failure message;

-	For the case of “available user plane connection between LMF and UE”, when the user plane of the UE becomes unavailable, the UE sends the user plane connection release request message with "User plane not available" to release the established user plane connection with the LMF.

When the "user plane not available" is received, the LMF shall not re-attempt to use LCS-UPP for location services of the UE, and may consider using control plane positioning solutions if the location services are still needed.

Observation #1: Based on the existing solutions, the LMF is aware of the UE's user plane availability status change from "available" to " not available", and will not re-attempt to use LCS-UPP for location services of the UE.

However, when the UE's user plane availability changes from "unavailable" to "available", currently there is no dedicated procedure specified to inform the LMF on the changes. The following existing operations may be used for LMF to be aware of UE’s user plane availability becoming available:

-	The UE can initiate UE requested user plane connection establishment procedure. The user plane connection establishment request message from the UE can implicitly indicate UE's user plane availability changes from "unavailable" to "available".

However, this request message is sent by the UE under the condition that the UE has uplink location services signalling and the UE decides to request to use a user plane connection for the uplink signalling. Before receiving the message, if LMF receives other LCS requests of the UE, the LCS-UPP is still not considered by LMF, which may cause additional control plane overload and rejections;

-	The LMF can re-attempt LCS-UPP for other location services of the UE using the network initiated user plane connection establishment procedure.

This may lead to extra multiple unsuccessful user plane connection establishment procedure attempts over the air interface until UE's user plane availability changes from "unavailable" to "available".

Observation #2: Existing operations may be used for LMF to be aware of UE’s user plane availability becoming available, however, they have limitations or cause unnecessary signalling and waste of radio resources .

In summary, the LMF is not timely aware of UE's user plane availability changes. The following problems are identified if LMF is not timely aware of the UE's user plane availability:

- Unsuccessful User Plane Connection Establishment: If the LMF is not aware of the UE's user plane availability, it may initiate an unsuccessful user plane connection establishment procedure. This can lead to unnecessary signalling and waste of resources.

- Delay in Connection Establishment: The LMF may have to wait for the user plane connection establishment request message from the UE. This delay can affect the performance of other LCS requests and potentially cause control plane congestions.

- Extra Signalling Over the Air Interface: The LMF may perform unsuccessful user plane connection establishment procedure attempts, leading to extra signalling over the air interface. This can put additional load on the network and degrade its performance.

- Inefficient Use of LCS-UPP: Without knowledge of the UE's user plane availability, the LMF may not be able to use LCS-UPP as a candidate positioning method. This can lead to inefficient use of resources and potential control plane congestion.

- Potential Control Plane Congestions: The unawareness of the LMF about the UE's user plane availability can lead to additional control plane congestions, affecting the overall network performance.

Observation #3: LMF not being able to timely aware of the UE's user plane availability will lead to Unsuccessful User Plane Connection Establishment, Delay in Connection Establishment, Extra Signalling Over the Air Interface, Inefficient Use of LCS-UPP and Potential Control Plane Congestions.

Proposal #1: LMF shall be able to timely aware of the UE's user plane availability.

- Discussion

To obtain the UE's user plane availability status, the following solutions may be considered as described in 3.1, 3.2, 3.3 and 3.4.

3.1 Using existing parameters (e.g. 3GPP PS Data Off status)

The 3GPP PS Data Off status is activated or deactivated based on the user's choice. However, the user plane availability status can change due to various reasons, not just the activation or deactivation of the 3GPP PS Data Off status.

Changes in the 3GPP PS Data Off status are reported to the SMF via the AMF, but this report is transparent to the AMF. However, the user plane availability status is not transparent to the AMF.

The 3GPP PS Data Off status alone is not sufficient to indicate the user plane availability status. Even when the 3GPP PS Data Off status is deactivated, the user plane may still be unavailable due to other reasons.

The AMF and SMF maintain a session level context of the UE, which includes the status of the PDU session. However, this context is not sufficient to indicate the user plane availability status, which is a UE level status.

Observation #4: The existing parameters (e.g., 3GPP PS Data Off status) are inadequate to indicate UE's user plane availability status. To accurately determine the user plane availability status of a UE, additional parameters are needed.

Proposal #2: Define a new parameter “user plane availability” to indicate UE’s user plane availability.

3.2 UE report of user plane availability status

UE is naturally aware of its user plane availability status. One option for LMF to obtain the UE's user plane availability status is that UE reports its user plane availability status to the LMF via AMF when UE's user plane availability status changes.

The UE can send a message to LMF via AMF to every time the UE's user plane availability status changes. For example, UE uses existing message (e.g., user plane connection establishment request message) or new message (e.g., user plane availability notification message) to include the user plane availability status IE. Upon receipt of a message that indicates the change of user plane availability, the LMF obtains and stores the indicated user plane availability status.

The corresponding service flow could be:

- UE: user plane availability status is not available;

- UE -> AMF ->LMF: user plane connection establishment request (user plane not available));

- UE: user plane availability status is available;

- UE -> AMF ->LMF: user plane connection establishment request (user plane available);

Option #1: The UE sends a message to LMF to indicate UE's user plane availability when the UE's user plane availability changes.

However, the AMF selects the LMF per location request. If the AMF doesn't receive any location request of a UE, the AMF won't select an LMF for the UE. The UE may send the message to indicate UE's user plane availability changes but the destination LMF is not selected yet. Besides, even one serving LMF for one location request of a UE is selected, the AMF may select another LMF for another location request of this UE, the newly selected LMF is not informed of the user plane availability status. Moreover, If the UE has multiple serving LMFs, the UE needs to send the message multiple times to each serving LMF.

3.3 LMF subscribes the UE's user plane availability status

One feasible solution is that the LMF subscribes user plane availability of a UE, and the UE reports the UE's user plane availability to the LMF based on subscription.

After AMF selects an LMF for a location request, the LMF can subscribe the user plane availability of a UE via AMF. For example, the LMF can use existing message (e.g., LCS Periodic-Triggered Invoke request message) or new message (e.g., user plane availability subscription message) for subscription. The UE responses the subscription and sends a message to notify LMF when the UE's user plane availability changes. For example, the UE can use existing message(e.g., event report message) or new message (e.g., user plane availability notification message) to notify LMF. Upon receipt of a message that indicates the change of user plane availability, the LMF obtains and stores the indicated user plane availability status.

The corresponding service flow could be:

- LMF -> AMF -> UE: LCS Periodic-Triggered Invoke request (user plane availability changes event);

- UE -> AMF -> LMF: LCS Periodic-Triggered Invoke response;

- UE: user plane availability status is not available;

- UE -> AMF ->LMF: event report (user plane not available);

- UE: user plane availability status is available;

- UE -> AMF ->LMF: event report (user plane available);

Option #2: The UE sends a message to LMF to indicate UE's user plane availability based on the LMF's subscription.

This option is triggered by LMF, and can avoid the problems of Option#1. Comparing with Option#1, additional subscription procedure is  required before UE sends the it’s user plane availability. If multiple serving LMFs subscribe to UE's user plane availability, the UE sends the message to each serving LMF individually.

3.4 AMF notifies LMF the UE's user plane availability status

The serving LMF(s) of a UE is/are selected by the AMF. If the AMF can maintain the UE's user plane availability status, the AMF can inform the LMF of the UE's user plane availability status with an LCS request.

Another solution is that UE provides UE's user plane availability status to AMF, and AMF provides the user plane availability status to LMF using the determine location request.

The option requires UE sends a message to AMF to provide the user plane availability status. For example, via Registration request or Service request. The AMF stores the received user plane availability status as part of UE context. Upon receiving an LCS request, the AMF provides the stored UE's user plane availability status with the determine location request. The LMF determines the positioning method for the LCS request based on UE's user plane availability status. If the UE's user plane availability changes, the UE further informs the AMF the updated user plane availability status. The AMF provides the updated UE's user plane availability status to the selected LMF until a new LCS request of this UE is received.

The corresponding service flow could be:

- UE: user plane availability status is not available;

- UE -> AMF: Registration request/Service request(user plane not available);

- AMF receives an LCS request;

- AMF -> LMF: Nlmf_Location_DetermineLocation(user plane not available);

- UE: user plane availability status is available;

- UE -> AMF: Registration request/Service request(user plane available);

- AMF receives another LCS request;

- AMF -> LMF: Nlmf_Location_DetermineLocation(user plane available);

Option #3: The UE provides user plane availability status to AMF, and AMF provides the user plane availability to LMF.

In this option, the UE's user plane availability is maintained by AMF. The AMF can provide the stored UE's user plane availability after the LMF is selected, so the issues that the destination LMF is not selected can be avoided. Besides, even the different LMF for another location request of this UE is selected by the AMF, the newly selected LMF is can also be informed of the user plane availability status. Moreover, if the UE has multiple serving LMFs, the UE only needs to send the message one time to AMF.

4 Evaluations

The comparison table between option #1, #2 and #3 is listed below:

Table 1 Comparison between option #1, #2 and #3

| Options | Opt#1 UE report | Opt#2 LMF subscription | Opt#3 AMF notification |
| --- | --- | --- | --- |
| Modifications | UE notifies LMF when user plane availability changes | LMF subscribes the UE for the user plane availability changes UE notifies LMF when user plane availability changes | UE notifies AMF when user plane availability changes |
| Impact to LMF | LMF maintains UE's user plane availability status | LMF maintains UE's user plane availability status | - |
| Impact to AMF | - | - | AMF maintains UE's user plane availability status |
| Invalid scenario | LMF was not previously selected | LMF doesn't subscribe UE's user plane availability changes | - |
| Multiple serving LMFs | Multiple signalling | Multiple signalling | One signalling |

According to Table 1, option #1 requires UE to report its user plane availability status to its serving LMF, and the LMF maintains the UE's user plane availability status. But if the LMF is not selected, this option is invalid. If the UE has multiple serving LMFs, the UE needs to send multiple messages to indicate each LMF its change of UE's user plane availability. Option#2 requires LMF to subscribe the user plane availability status of the UE, and the LMF maintains the UE's user plane availability status. If the LMF doesn’t send the subscription message, this option is invalid. Similarly, the UE needs to send multiple messages to indicate each LMF its change of UE's user plane availability. Option#3 requires UE to reports its user plane availability status to AMF, and the AMF provides it to LMF when receiving a LCS request. If the UE has multiple serving LMFs, the UE only needs to send one message to inform AMF.

Based on above evaluation, option #3 has the least signalling and doesn’t have any invalid scenario.

Proposal #3: Option#3 is adopted, i.e. the UE provides user plane availability status to AMF, and AMF provides the user plane availability to LMF.

5. Conclusion

Observation #1: Based on the existing solutions, the LMF is aware of the UE's user plane availability status change from "available" to " not available", and will not re-attempt to use LCS-UPP for location services of the UE.

Observation #2: Existing operations may be used for LMF to be aware of UE’s user plane availability becoming available, however, they have limitations or cause unnecessary signalling and waste of radio resources.

Observation #3: LMF not being able to timely aware of the UE's user plane availability will lead to Unsuccessful User Plane Connection Establishment, Delay in Connection Establishment, Extra Signalling Over the Air Interface, Inefficient Use of LCS-UPP and Potential Control Plane Congestions.

Observation #4: The existing parameters (e.g., 3GPP PS Data Off status) are inadequate to indicate UE's user plane availability status. To accurately determine the user plane availability status of a UE, additional parameters are needed.

Proposal #1: LMF shall be able to timely aware of the UE's user plane availability.

Proposal #2: Define a new parameter “user plane availability” to indicate UE’s user plane availability.

Proposal #3: Option#3 is adopted, i.e. the UE provides user plane availability status to AMF, and AMF provides the user plane availability to LMF.

This paper discusses issue that LMF is not timely aware of change of UE's user plane availability status. Three options are proposed for LMF to obtain the UE's user plane availability status. After evaluation, it is proposed that Option #3 is adopted which is captured in S2-2412149 & S2-2412150.