3GPP TSG-SA2 Meeting #166	S2-2412175

# 18 – 22 November, 2024, Orlando, USA          			             			(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0629 | rev |  | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | User plane positioning enhancement to support the availability of user plane connection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the CT1 LS (S2-2411289), the user plane becomes unavailable due to e.g. the activation of the PS data off. This leads to the issue that the LMF does not know the UE’s user plane availability changes from “unavailable” to “available”. To solve the issue, two possible solutions are also included in the LS, the analyses of the solutions are as follows: - Sol#1: the UE can initiate UE requested user plane connection establishment procedure, which may not be appropriate unless the UE has UL positioning signalling, i.e. either such notification is delayed or user plane positioning resources are unnecessarily established; Analysis of sol#1: the UE initiates UE requested user plane connection establishment procedure when the PS data off is deactivated to notify the LMF the user plane is available. This solution requires the AMF and LMF to maintain the user plane positioning context for UE, e.g. to record the status of the user plane. To support UE mobility, the context shall be transferred from source AMF to target AMF, from source LMF to target AMF. Furthermore, if the LMF initiated user plane connection establishement is rejected by UE caused by activation of PS data off, the LMF also needs to maintain the context for the failed procedure. - Sol#2: the LMF can attempt reactivation of user plane positioning using the network initiated user plane connection establishment procedure. This leads to extra unsuccessful user plane connection establishment procedure attempts over the air interface until the user plane is "available". Analysis of sol#2: in this solution, when the LMF receives user plane unavailable, the LMF can remove the user plane context for UE and notify the AMF to remove the user plane context. So compared with sol#1, there is no context transfer issue due to UE mobility. Furthermore, UE is not impacted to notify the LMF that the user plane becomes available. The drawback is that the LMF does not the user plane status, it may iniitate user plane connection establishment procedure when the user plane is unavailable. This may lead to a waste of signalling. Based on the analyses above, sol#1 has UE impact and requires context transfer mechanism to support UE mobility. Sol#2 may lead to a waste of signalling. To avoid the drawbacks above and reuse the existing mechanism as much as possible, it is proposed to use the following solution as the way forward. - When the user plane becomes unavailable, e.g. due to the activation of the PS data off, the UE thinks the user plane positioning capabilities changes from “supported” to “not supported”. In this case, - The UE initiates the Registration procedure to update the user plane positioning capabilities from “supported” to “not supported” which is the existing mechanism to update the 5GMM capability. - For the ongoing user plane connection establishment procedure which is initiated by LMF before the UE sends the Registration Request, the UE can reject the procedure using the cause value “user plane not available”. Because the procedure is failed, there is no need for LMF to maintain context for the user plane connection failed to be established. - After the Registration procedure in bullet a), when the LMF receives the positioning request from AMF which includes the UE user plane postioning capabilities indicating not supported, the LMF selects user plane or control plane positioning based on existing mechanism. - When the user plane becomes available, e.g. due to deactivation of the PS data off, the UE thinks the user plane positioning capabilities changes from “not supported” to “supported”. In this case, - The UE initiates the Registration procedure to update the user plane positioning capabilities from “not supported” to “supported” which is the existing mechanism to update the 5GMM capability. - After the Registration procedure in bullet a), when the LMF receives the positioning request from AMF which includes the UE user plane postioning capabilities indicating supported, the LMF selects user plane or control plane positioning based on existing mechanism. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhance the user plane positioning mechanism to support the availability of user plane connection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The availability of user plane connection is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

4.3.5	UE

A target UE may support positioning according to four different modes:

-	UE assisted mode (the UE obtains location measurements and sends the measurements to another entity (e.g. an LMF) to compute a location);

-	UE based mode (the UE obtains location measurements and computes a location estimate making use of assistance data provided by serving PLMN);

-	standalone mode (the UE obtains location measurements and computes a location estimate without making use of assistance data provided by serving PLMN);

-	network based mode (a serving PLMN obtains location measurements of signals transmitted by a target UE and computes a location estimate).

NOTE:	The transmission of UE signals for network based mode may or may not be transparent to the UE.

Positioning procedures used by a UE for NG-RAN access are described in TS 38.305 [9].

A limited set of UE positioning capabilities and UE user plane positioning capabilities can be transferred to the 5GCN in the 5GMM capability during registration of the UE as described in TS 24.501 [11]. Some of these positioning capabilities may be transferred subsequently to an LMF as described in TS 29.572 [12]. UE positioning capabilities may also be transferred directly to a location server (e.g. LMF). The UE can update the user plane positioning capabilities in the 5GMM capability as described in TS 24.501[11] based on availability of the user plane connection.

NOTE X:	For example, when the user plane availability is changed to “not available”, e.g. due to the activation of the PS data off for the UE, the UE updates the user plane positioning capabilities to “not supported” by initiating registration procedure. When the user plane availability is changed to “available”, e.g. due to the deactivation of the PS data off, the UE updates the user plane positioning capabilities to “supported” by initiating registration procedure.

Additional functions which may be supported by a UE to support location services include the following.

-	Support location requests received from a network for 5GC-MT-LR, 5GC-NI-LR or a deferred 5GC-MT-LR for periodic or triggered location.

-	Support location requests to a network for a 5GC-MO-LR.

-	Support privacy notification and verification for a 5GC-MT-LR or deferred 5GC-MT-LR for periodic or triggered location.

-	Send updated privacy requirements to a serving AMF (for transfer to a UDR via UDM).

-	Support periodic or triggered location reporting to an LMF.

-	Support change of a serving LMF for periodic or triggered location reporting.

-	Support cancelation of periodic or triggered location reporting.

-	Support multiple simultaneous location sessions.

-	Support the reception of unciphered and/or ciphered assistance data broadcast by NG-RAN.

-	Support the reception of ciphering keys for the assistance data from the AMF.

-	Support handling of 5GC-MT-LR, 5GC-NI-LR, 5GC-MO-LR and deferred 5GC-MT-LR for periodic or triggered location over a user plane connection between UE and LMF.

-	Support reporting of location events for a periodic or triggered 5GC-MT-LR over a user plane connection to an LCS Client or AF with periodic cumulative event reports being sent over control plane to the LMF, H-GMLC and LCS Client or AF.

* * * End of Change * * *