3GPP TSG-SA2 Meeting #166	S2-2412178

# Orlando, USA, November 18 - 22, 2024          			                 		 (revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0632 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove the Additional ULI |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | F |  |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the following description in clause 5.19.1: - The AMF serving the UE provides the cell ID of serving cell of the UE and indicates, if possible, that the cell-ID belongs to a MWAB to the LMF in the location request. The AMF serving the UE may also provide LMF with the additional ULI Information received from MWAB. The AMF may provide the LMF with the additional ULI which may reflect more accurate UE location than the serving cell ID which is a MWAB cell. This CR is proposed to remove the additional ULI, because without the parameter, the existing mechanism also works. The analysis is as follows: - The AMF provides serving cell ID of the target UE to LMF. The LMF uses the serving cell ID and TRP information to select the TRPs taking part in the positioning measurements. For example, the serving gNB/TRP and neighbour gNBs/TRPs may be selected to perform the measurement as illisutrated in Figure 8.13.3.4-1 in TS 38.305. - Based on the description in clause 8.14.3.2.1 in TS 38.305, the LMF initiates the TRP information exchange procedure to obtain TRP information periodically or triggered by OAM. The TRP information specified in clause 9.2.25 in TS 38.455 is as follows: IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description; Criticality; Assigned Criticality. TRP ID; M; 9.2.24; -. TRP Information Type; 1 .. <maxnoTRPInfoTypes>; -. >CHOICE TRP Information Item; M; -. >>NR PCI; INTEGER (0..1007); NR Physical Cell ID. >>NR CGI; 9.2.9. >>NR ARFCN; INTEGER (0..3279165). >>PRS Configuration; 9.2.44. >>SSB Information; 9.2.54. >>SFN Initialisation Time; Relative Time 1900 9.2.36. >>Spatial Direction Information; 9.2.45. >>Geographical Coordinates; 9.2.46. >>TRP type; ENUMERATED (prs-only-tp, srs-only-rp, tp, rp, trp, …, mobile trp); TS 38.305 [18]; YES; reject. >>On-demand PRS TRP Information; 9.2.65; YES; reject. >>TRP Tx TEG Association; 9.2.79; YES; reject. >>TRP Beam Antenna Information; 9.2.82; YES; reject. >>Mobile TRP Location Information; M; 9.2.88; YES; ignore. >>Common TA Parameters; M; 9.2.89; YES; reject. Mobile IAB-MT UE ID; C-ifMobileTRP; OCTET STRING; The UE ID of the IAB-MT associated with the mobile TRP. Includes the GPSI as defined in TS 29.571 [21]; YES; reject. Based on the IEs highlighted in red in the table above, the TRP information obtained by LMF includes the geographical location and cell ID of TRP. Based on this information and the serving cell ID of target UE, the LMF can select TRPs takening part in the positioning measurements. - Based on the following description in step 4 in clause 6.1.5, the LMF can also obtain the information of TRPs associated with the MWAB-gNB. In this case, the cell ID included in the TRP information is the MWAB cell ID. 4. LMF derives if any MWAB(s) is involved in the positioning of the target UE based on the cell-ID used for positioning measurements in the step 3. The AMF serving the target UE may indicate that the serving cell is an MWAB (if applicable). When the MWAB-gNB was integrated as a TRP, the LMF may determine from in a TRP information exchange procedure that the cell-ID belongs to the MWAB and/or the UE-ID (GPSI) associated with MWAB-UE. The LMF, when the OAM provides a new Cell Id to the LMF, performs a TRP information exchange as specified in TS 38.455 [15] for the new Cell ID. When the MWAB-gNB is integrated in the 5GS, the Cell ID(s) of the MWAB is (are)provided to the LMF. If the LMF selects any cell for UE positioning measurements, and the LMF determines it needs to perform a TRP information exchange with the cell, it shall perform TRP Information Exchange procedure as specified in TS 38.455 [15].As the MWAB can be mobile the LMF may need to determine an updated location of the MWAB by either performing step 5-7 (option 1) or performing step 8-10 (option 2) if option 1 is not feasible. NOTE 2: It is assumed that for the case of MWAB mobility, the OAM can timely update the Cell Id of the MWAB to the LMF so as to enable it to perform a proper TRP information exchange. Until the Cell Id of the MWAB is provided to LMF by OAM, the Cell of the MWAB is not used for location procedures. - If the target UE connects to a MWAB-gNB, when AMF sends location request to LMF, the AMF provides the serving cell ID which is MWAB cell to LMF. Based on the serving cell ID of the target UE and the TRP information (e.g. cell ID, geographical location) stored in LMF, the LMF can select TRP correctly. Based on the analysis above, without the additional ULI, the existing mechanism works. So there is no need to introduce additional ULI for location service. It is proposed to remove the parameter. |
| Summary of change: |  | Remove the additional ULI. |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Useless parameter exists. |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.19.1 |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

5.19.1	General

A MWAB may have location service capability as specified in TS 38.305 [9] and participate in the location service of a UE. As the MWAB may be moving, the location service procedures need to be enhanced as following for an accurate estimation of the UE position:

-	The UE reports the cell IDs of all the cells the UE performed DL positioning measurements on.

-	The MWAB which performed the location service procedures for the UE includes its cell ID in the reported UL positioning measurement.

-	The AMF serving the UE provides the cell ID of serving cell of the UE and indicates, if possible, that the cell-ID belongs to a MWAB to the LMF in the location request.

-	Additionally, the LMF uses the reported cell IDs to derive whether the cell ID(s) corresponds to a MWAB. There can be multiple MWAB cells in the measurement report.

-	The LMF may also decide whether the cell ID(s) corresponds to MWAB(s) based on information received in a TRP information exchange i.e., that the cell-ID belongs to a MWAB and the UE-ID (GPSI) associated with MWAB.

-	To aid the LMF to improve the accuracy of the UE location estimation, the MWAB velocity information and time of obtaining its location measurement data should be obtained by the LMF when available. The LMF uses the received location and velocity of the MWAB(s) when estimating the location of the Target UE.

The reference architectures for MWAB involved location services in different scenarios are specified in Annex S of TS 23.501 [18].

* * * End of Change * * *