3GPP TSG-SA2 Meeting #166	S2-2412180

# Orlando, USA, November 18- 22, 2024          			                 		 (revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0152 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Nnef_UAVFlightAssistance_Create service operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR proposes to update the Nnef_UAVFlightAssistance_Create service operation from the following two aspects: - Based on the approved CR (S2-2411160) in SA2#165 meeting, the USS/UTM can send a Nnef_UAVFlightAssistance_Create request to request in-flight monitoring of a target UAV. This function adds new input (optional) parameters to the service operation. This CR is proposed to update the service operation in clause 4.4.1.1.3 to add the new input (optional) parameters. - Based on the approved CR (S2-2411159) in SA2#165 meeting, the flight path information is clarified which includes a set of 4-point polygons, height information, and corresponding timestamps/time schedule for the flight, it is proposed to update the fligh path parameter in the Nnef_UAVFlightAssistance_Create service operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the Nnef_UAVFlightAssistance_Create service operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misaligned description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.1.1.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

4.4.1.1.3.3	Nnef_UAVFlightAssistance_Create service operation

Service operation name: Nnef_UAVFlightAssistance_Create

Description: Providing a NEF/UAS NF information about the planned flight path(s) and a time schedule for the UAV from a starting point to a destination point (even when the destination point is located in geographical area served by different USS). And providing a NEF/UAS NF information about the in-flight monitoring of a target UAV.

Editor's note:	Representation of the destination point and border crossing point format is FFS.

Input, Required: UAV's identifier (e.g. GPSI).

Input, Optional: Flight path information( which may include a set of 4-point polygons, height information and corresponding timestamps/time schedule for the flight), time schedule for crossing/spending at each of the TAs/cells, monitoring mode (e.g. UAV location monitoring, flight trajectory monitoring, flight environment monitoring, Ranging/Sidelink positioning location monitoring, QoS Sustainability analytics), reporting requirements (e.g. event triggered or periodically), assistance information (e.g. the monitoring results, information generated from the monitoring results), the planned flight path(s), a list of UAVs in the area of interest if the flight environment is set for the monitoring mode, present value for distance if the monitoring mode is set to Ranging/Sidelink positioning.

Output, Required: Flight plan configuration ID (associated with the changeover).

Output, Optional: None.

* * * End of Change * * *