3GPP TSG-WG SA2 Meeting #166	S2-2412250

# 18 – 22 November 2024, Orlando, USA		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0515 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures for ProSe Multihop U2U Relay reselection for Non-IP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In SA2#164 it is approved in 6.4.3.8.3 that the relay reselection procedure can be triggered by a 5G ProSe Remote UE. Similiar approach can be used for ProSe Multihop U2U Relay reselection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add procedures for ProSe Multihop U2U Relay reselection for Non-IP Type PDU. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ProSe Multihop U2U Relay reselection will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.5.x, 6.7.5.X.Y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all new)* * * *

6.7.5.X	Relay reselection and mobility support

6.7.5.X.Y   Multi-hop 5G ProSe Layer-3 UE-to-UE Relay for Non-IP Type PDU

There are two types of situations for relay reselection, i.e. for a single 5G ProSe End UE, and for a 5G ProSe Layer-3 Multi-hop UE-to-UE-Relay.

For the initiating 5G ProSe End UE, when the connection with an old 5G ProSe Layer-3 Multi-hop UE-to-UE-Relay is lost or the link quality drops below a configured threshold, the 5G ProSe End UE triggers the reselection of the relays that offers the same service, identified by the same RSC and Target End UE.

Figure 6.7.5.X.Y-1: Relay reselection for 5G ProSe Layer-3 UE-to-UE Relay for Non-IP Type PDU

Figure 6.7.5.X.Y-1 depicts the Relay reselection operation for this situation. When the initiating 5G ProSe End UE has lost the connection or has an imminent connection loss/deteriorated connection with the 5G ProSe Layer-3 Multi-hop UE-to-UE-Relay-1, the 5G ProSe End UE performs relay reselection by using the relay discovery procedure in clause 6.3.2.6. The 5G ProSe End UE discovered the 5G ProSe Layer-3 Multi-hop UE-to-UE-Relay-2 that offers the same RSC and Target End UE.

The initiating 5G ProSe End UE establishes the Layer-2 link with the 5G ProSe Layer-3 Multi-hop UE-to-UE-Relay-2 as specified in clause 6.7.5.2.2. After the connection establishment, the 5G ProSe Layer-3 Multi-hop UE-to-UE-Relay-2 uses the Layer-2 Link Modification procedure to update the 5G ProSe End UE at other side. The responding 5G ProSe End UE at other side switches the connection/routing context for the initiating 5G ProSe End UE from the Layer-2 link with the 5G ProSe Layer-3 Multi-hop UE-to-UE-Relay-1 to that of the Layer-2 link with the 5G ProSe Layer-3 Multi-hop UE-to-UE-Relay-2.

The initiating 5G ProSe End UE tears down the link with the 5G ProSe Layer-3 Multi-hop UE-to-UE-Relay-1 (or the link failure is detected), which will trigger the 5G ProSe Layer-3 Multi-hop UE-to-UE-Relay-1 to update the 5G ProSe End UE at other side to remove its association with 5G ProSe End UE.

The relay reselection procedure can be used by a 5G ProSe Layer-3 Multi-hop UE-to-UE-Relay, taking the role of the 5G ProSe End UE in the procedure, when its connection with an old 5G ProSe End UE at other side is lost or the link quality drops below a configured threshold.

If the 5G ProSe Layer-3 Multi-hop UE-to-UE-Relay can not complete the reselection procedure, e.g. it is not able to discover the 5G ProSe End UE at other side, or 5G ProSe Layer-3 Multi-hop UE-to-UE-Relay determines to not perform the reselection procedure, it sends reselection indication and optionally with a cause value to 5G ProSe End UE via the established multipath relay. After receving the reselection indication the 5G ProSe End UE triggers the reselection of the relays as described above.

During the transmition of reselection indication to 5G ProSe End UE, after receving the reselection indication a 5G ProSe Layer-3 Multi-hop UE-to-UE-Relay based on configuration can determine to relay the reselection indication to 5G ProSe End UE, or perform relay reselection by iteself, e.g. if it already has another path to the same 5G ProSe End UE of the same RSC at other side.

# * * * * End of changes * * * *