3GPP TSG SA WG2#166	            S2-2412264

Orlando, US, 18 – 22 November 2024    	(revision of S2-2410711)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0252 | rev | 6 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AF provisioning N6 delay measurement information via EDI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, [Intel, ZTE] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In SA2#165, to address the outcome of KI#2 in approved eEDGE_5GC_Ph3 WID SP-240996 on provisioning of N6 delay measurement information, N6 delay measurement assistance and requirement information in EDI is introduced with an EN on maximum measurement session frequency. In SA2#166, this CR proposes to add a note referring to respective clause in TS 23.501 regarding the measurement protocols and to remove the EN and indicate the maximum measurement session frequency as part of protocol-specific configuration parameters for the following reasons: - In TWAMP [RFC 5357], Timeout is the interval that the Session-Reflector MUST wait after receiving a Stop-Sessions message. In case there are test packets still in transit, the Session-Reflector MUST reflect them if they arrive within the Timeout interval following the reception of the Stop-Sessions message. - In STAMP [RFC 8762], a measurement session, also referred to as a "STAMP session", is the bidirectional packet flow between one specific Session-Sender and one particular Session-Reflector for a time duration. The configuration and management of the STAMP Session-Sender, Session-Reflector, and sessions can be achieved through various means. - In OWAMP [RFC 4656], The sender and the receiver both need to know the same send schedule. To implement this, there is a schedule with a given number of slots. Each slot has a type and a parameter. The parameter is expressed as a timestamp and specifies a time interval. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce N6 delay measurement assistance and requirement information in EDI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing functionality |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev6: changes are highlighted in yellow. - Removal of EN on measurement frequency. - Adding a NOTE on supported protocols to refer to 5.2.8.X in TS 23.501. |  |  |  |  |  |  |  |  |

FIRST CHANGE

##### 6.2.3.4.1	General

EAS Deployment Information management refers to the capability to create, update or remove EAS Deployment Information from AF and the distribution to the SMF or to the I-SMF in case I-SMF based Local Offloading Management applies. The NEF is in charge of the management of EAS Deployment Information which may be stored in UDR.

The EAS Deployment Information indicates how edge services are deployed in each Local part of the DN, the description of EAS Deployment Information is shown in Table 6.2.3.4-1.

Table 6.2.3.4-1 Description of EAS Deployment Information

| Parameters | Description |
| --- | --- |
| AF ID | Addressing information of Application Function responsible for the DNAI in the record. [Optional]. See NOTE 1. |
| DNN | DNN for the EAS Deployment Information. [optional] |
| S-NSSAI | S-NSSAI for the EAS Deployment Information. [optional] |
| External Group Identifier/Internal Group Identifier | Group ID for the EAS Deployment information. [optional]. See NOTE 2. |
| Application ID | Identifies the application for which the EAS Deployment Information corresponds to. [optional] |
| FQDN(s) | Supported FQDN(s) for application(s) deployed in the Local part of the DN. |
| DNAI(s) | DNAI(s) for the EAS Deployment information. [optional] |
| DNS Server Information | list of DNS server identifier (consisting of IP address and port) for each DNAI. [optional] |
| EAS IP address range Information | IP address(es) of the EASs in the Local part of the DN or the IP address ranges (IPv4 subnetwork(s) and/or IPv6 prefix(es) of the Local part of the DN where the EAS is deployed for each DNAI. [optional] |
| N6 traffic routing information | Information about how to forward edge traffic in the local part of DN corresponding to DNAI. [optional] |
| N6 delay measurement assistance information | Includes the following parameters per DNAI and/or per EAS to enable N6 delay monitoring: - measurement endpoint address information (consisting of IP address and optionally port number). - measurement protocol(s) supported by the application measurement endpoint, - protocol-specific configuration parameters. [optional]. NOTE X, NOTE Y, NOTE Z. |
| NOTE 1: When an AF ID is provided, all DNAI(s) correspond to the same EHE provider. NOTE 2: The AF may provide External Group Identifier, and NEF can map the External Group Identifier into Internal Group Identifier according to information received from UDM. For HR-SBO roaming scenario, External Group Identifier and Internal Group Identifier, cannot be used by AF in VPLMN. NOTE 3: AF ID can be used in case of AF(s) involving different EHE providers, and the source EHE is unaware of other/target EHE specific deployment details. NOTE X: A measurement endpoint can be used for delay measurements representing a single EAS IP address or a range (IPv4 subnet or IPv6 prefix) of EAS IP addresses. NOTE Y: The N6 delay measurement assistance information is utilized when AF traffic influence request includes indication of considering N6 delay as defined in clause 5.6.7.1 of TS 23.501 [2]. NOTE Z: The measurement protocol(s) considered for N6 delay measurements are listed in clause 5.8.2.X of TS 23.501 [2]. |  |

The EAS Deployment Information management procedures are described in this clause, the procedures are independent of any PDU Session, including:

-	The procedure for EAS Deployment Information management from AF via the NEF.

-	The procedure for EAS Deployment Information management in the SMF or in the I-SMF when I-SMF based Local Offloading Management applies.

-	The procedure for BaselineDNSPattern management in the EASDF.

NOTE:	In order to support EAS discovery when the Edge Hosting Environment is provided by a partner, an SLA is needed between current operator and the partner to provide e.g. the Address(es) and credentials for the DNS servers if the partner hosts the DNS server(s) for the related DNS resolution.

End of CHANGES