3GPP TSG-WG SA2 Meeting #166	S2-2412289

Orlando, USA, 18th Nov – 22th Nov, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	Support for NR Femto local service

Document for:	Discussion

Agenda Item:	19.12.2

Work Item / Release:	5G_Femto / Rel-19

Abstract: This discussion paper proposes to analyse whether the existing mechanism could support the local services and consideration on the access to femto deployed in residential scenario.

# 1. Introduction

During the SA2#165 meeting, there is no conclusion for supporting local services. This discussion paper proposes to analyse whether the existing mechanism could support the local services.

There two scenarios for local service:

- Corporate scenario: Several NR Femto and optionally a local UPF.

- Home scenario: One NR Femto and optionally a local UPF, the UPF may collocated with the NR Fetmo.

UE can access the local service via the NR Femto. A dedicate UPF maybe used to support local services.

# 2. Discussion

In this document we consider the corporate and residential scenario for the access to local services and the deployment consideration on the networking aspects.

## 2.1 Access to local services

Scenario 1 (corporate scenario):

Figure 1: Corporate scenario

In this scenario, a dedicate S-NSSAI or dedicate DNN may be used for local services. The UE which is allowed to access the Femto (e.g. the member of the CAG) could be configured with the allowed S-NSSAI. The PCF can provide corresponding URSP rules to the UE. The Route Selection Validation Criteria of URSP can be configured to limit the local service only in the corporate area (i.e. only can be accessed via the location corresponding to the corporate). The UE can establish a PDU Session to access the local service using the dedicate S-NSSAI/DNN which leads to the selection of the UPF deployed in the Corporate area or that provides the access to the local services in corporate area even if is not directly deployed in the area.

Regarding the SMF aspect, the dedicate S-NSSAI/DNN is bonding with the local UPF based on the configuration. When UE request to establish a PDU Session using the dedicate S-NSSAI/DNN, the SMF selects the local UPF.

The NR Femto are connected to the local UPF via N3 interface without requiring any special functionalities.

The local UPF may only connect to the NR Femto to ensure that no other UEs access the local service out of corporate area. The specific deployment depends on implementation.

Scenario 2 (home scenario):

Figure 2: Residential scenario

In this scenario, the Femto and UPF may be contained in a single box to be deployed in the home. The Femto interfaces with the 5G Core Network CP and UP functions via the N2 and N3 interfaces, respectively.

Based on the analysis in the scenario 1, the dedicate S-NSSAI/DNN also can be used in scenario 2. The corresponding URSP should have a Route Selection Validation Criteria which only contains 1 Femto cell.

If the user wants to have local service at home, the user shall have an agreement with the operator or some special configuration in performed in order to provisioning and associate the dedicate S-NSSAI/DNN to the user. When the Femto and UPF are powered on, the as done in 4G a TR-69 configuration procedure can take place in order to allow the operator to configure the Femto and UPF to support the dedicate S-NSSAI/DNN and to configure properly the User subscription information with the requested information as as the FQDN of the UPF or the IP address which can be used to reach the UPF and the Femto (e.g. via OAM).

The PCF shall provide the corresponding URSP rules to the member of the CAG. The UE request to establish a PDU Session for local service shall use the dedicate S-NSSAI/DNN based on the URSP rules. Based on the S-NSSAI/DNN, AMF shall select the corresponding SMF. SMF select the local service based on the configuration (i.e. SMF is configured with the association between S-NSSAI and UPF).

## 2.1 Networking issue for Femto deployment

The residential scenario is shown in details in figure 3 where the presence of a RG is shown. In fact, a precondition for the Femto is to have IP connectivity, How the IP connectivity is reached, basically via fixed access where a RG is present and the “femto Box” is connected via an Ethernet port or via WLAN. Also in case of a Box which include the RG from connectivity point of view it is not much different since the fixed access (e.g. the fiber) shall be terminated in the RG with all relevant features and therefore the Femto is most probably connected on the LAN side of the home network.

In such scenario it shall be noted that typically the home network has this characteristics:

- the LAN use IPv4 private address or IPv6 from the IPv6 prefix assigned to the RG

- presence of NAPT

- the home network may belong to different operator in respect the femto

- the WAN IP address of the RG is not static  and it often changes overtime (e.g. every 24 hours)

- the LAN IP address is typically dynamic and not static, since the regular user does not have the skill to assign a static local IP address and configure NAPT route.

Figure 3 : Residential scenario

The issue becomes how the AMF, the NR Femto GW, the SMF and the UPF for not local services are reachable from the NR Femto and UPF deployed within the Home where the Ip address assigned is Private, behind a NAT and the IP address of RG also change in the time (the BNG may also have a NAT).

In order to allow the IP routing properly and decouple the IP routing of 5GC from the underlying transport the answer is via some IP tunnelling, as usual. The IP-in-IP may be enough for solving the networking IP, but as done in the past an IPsec can also be used to address at the same time the security aspect. We may see the result of SA3, but equivalent to 4G the resulting architecture can be those in figure 4. Therefore the reference architecture is those shown in figure 5.

Figure 4 : Residential scenario with IP tunneling

Figure 5 : Reference architecture

# 3. Conclusion and proposal(s)

Based on the above consideration is proposed to

- use dedicate S-NSSAI/DNN to support local service without any enhancement.

- to define the architecture in figure 5 to address the issue relate to the networking IP