3GPP TSG SA WG2 #166	            S2-2412396

18 - 22 November 2024, Orlando, USA 	(revision of S2-24111332)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1480 | rev | 5 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#8: Architecture for avatar communications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, [ZTE, vivo, Huawei, HiSilicon] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on KI#8 conclusion in TR 23.700-77, the architecture for avatar communications needs to be specified in Rel-19. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The description of IMS DC based avatar communication is introduced. [R04: SA2#166] - Editor’s Notes in clause AC.X.1 General is cleared. - Note regarding DC AS URL is clarified. - Reference points are added in clause AC.2.3. - Procedures are newly added as clause AC.X.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IMS DC based avatar communication cannot be supported properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.2.3, AC.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

AC.2.3	Reference points

The following reference points are defined to support data channel service in IMS:

-	DC1:	Reference point between the DCSF and the IMS AS.

-	DC2:	Reference point between the IMS AS and MF.

-	DC3:	Reference point between the DCSF and NEF.

-	DC4:	Reference point between the DCSF and DC Application Server.

-	DC5:	Reference point between the DCSF and DCAR.

-	N72/Sc:	Reference point between the DCSF and HSS.

The following reference points are updated to support data channel signalling control in IMS:

-	N70/Cx/Dx:	Reference point between the CSCF and HSS.

-	N71/Sh:	Reference point between the IMS AS and HSS.

The following reference points are defined for data channel media handling:

-	MDC1:	Reference point for transport of data channel media between data channel media function MF and DCSF.

-	MDC2:	Reference point for transport of data channel media between data channel media function MF and DC Application Server.

-	MDC3:	Reference point for transport of data channel media between DCSF and DC Application Server.

-	MDC4:	Reference point for transport of data channel media between BAR and DC Application Server, and between DC Application Server and MF.

# * * * * Second change (all text is new) * * * *

AC.X	Support of Avatar Communication

AC.X.1	General

This clause describes the enhancements to the IMS architecture supporting data channel services to additionally support Avatar communication services.

For Avatar communication over IMS data channel, the list of Avatar ID(s) may be:

- locally configured in the UE or,

- transferred to the UE through bootstrap data channel as described in clause AC.X.3.1 or,

- transferred to the UE through application data channel as described in clause AC.X.3.1.

Avatar ID List contains the list of Avatar IDs that are identifying the Avatar Representation and/or the associated information. The associated information may include the information for the user selection in the avatar calls (e.g. the metadata associated with the Avatar such as thumbnails, the associated data channel applications), or the personal asset information which is applied to the Avatar Representation (e.g. the identities of personal assets such as accessesories, guarmets).

For provisioning of Avatar ID List through bootstrap data channel, the DCAS provides the DCAS URL(s) to the DCSF via DC3/N33 or DC4 for fetching Avatar ID List from the BAR. When the provisioning of Avatar ID List is requested through bootstrap data channel, the DCSF requests to DCAS to fetch the Avatar ID List from the BAR for the authorized user of the UE.

Editor's note:	The service operations provided by the DC AS toward the DCSF will be defined.

The BAR and DC AS may be deployed either in the trusted domain or untrusted domain of operator’s network..

After selection of Avatar ID to use in the avatar call by the user of the UE, according to the rendering mode, the Avatar Representation and the associated information (e.g. personal assets identified with the selected Avatar IDs) is provided from the BAR to the MF, and may be further transferred to the local UE and/or to the remote UE via application data..

Editor's note:	The user identity(ies) that should be used by the user of the local UE (UE#1) and/or the remote UE (UE#2) for provisionig of the the Avatar Representations in peer UE rendering mode will be decided based on the authorization and authentication procedures defined by SA3.

The avatar media is rendered by processing the Avatar Representation and/or the associated information (e.g. personal assets) with the animation data.

-	The avatar media rendering process may be performed by the sending UE, the receiving UE, or the network according to the rendering mode.

-	The animation data may be generated by the sending UE, the receiving UE, or the network (e.g. from audio, video, and/or metadata such as UE sensors collected data).

Editor's note: It is FFS, how to replace the root URL of the Avatar ID List/Avatar Representation with the replacement URLs before provisioning to the UEs.

AC.X.2	Architecture

Figure AC.X.2-1: Architecture to support Avatar communication

To support Avatar communication, the data channel architecture and functions described in clause AC.2 is enhanced as below:

Base Avatar Repository (BAR):

-	The BAR stores Avatar Represenation, its Avatar ID, and the associated information as described in clause AC.X.1. One or more Avatar Representation(s), is stored for a user, and each Avatar Representation is identified with Avatar ID.

-	The BAR may provide Avatar ID List to the DC Application Server, and the DC Application Server may further provide to DCSF.

-	The BAR provides Avatar Representation and the associated information to the MF directly or after URL redirection from the DC AS.

DCSF:

-	If the Avatar ID List is provisioned through bootstrap data channel, the DCSF may fetch the Avatar ID List from the DC AS, and send to MF via MDC1.

MF:

-	The MF supports the UE downloading the Avatar ID List and associated information through bootstrap data channel or application data channel.

-	The MF supports the UE downlaoding the Avatar Representation and the associatied information through bootstrap data channel or application data channel.

-	The MF may support Avatar media processing and transcoding (e.g. ASR/TTS).

DC AS:

-	The DC AS provides the DC AS URL(s) to DCSF via DC3/N33 or DC4 for provisioning of Avatar ID List through bootstrap data channel.

-	The DC AS retrieves the Avatar ID List from BAR(s) via MDC2.

-	The DC AS sends the Avatar ID List either to DCSF via MDC3 for provisioning through bootstrap data channel, or to MF via MDC2 for provisioning through application data channel.

-	The DC AS retrieves the Avatar Representation and the associated information from BAR and sends to MF via MDC2.

-	The DC AS supports Avatar media negotiation and provides the instruction for Avatar media processing.

AC.X.3	Procedures

AC.X.3.1	Avatar ID List provisioning through Bootstrap DC

Figure AC.X.3.1-1 depicts the Boostrap DC setup signalling procedure which includes the downloading of list of Avatar IDs that are stored in the BAR and allowed to use for the user of the UE.

Figure AC.X.3.1-1: Bootstrap DC setup signalling procedure for Avatar communication

The steps in the call flow are as follows:

- DC application server(s) provide the DC AS URL(s) for fetching the Avatar ID List from the BAR via DC3/N33 or DC4.

Editor's note:	The service operations provided by the DC AS toward the DCSF will be defined.

- IMS session and bootstrap data channel are established between the UE#1 and UE#2 as described in clause AC.7.1.

- DC Application list download, dedicated DC Application request, and download of DC Application are performed based on the result of step 1. The UE#1 downloads the avatar call DC application.

- The UE#1 requests the Avatar ID List for the user of the avatar call DC application through bootstrap data channel. The request is sent to DCSF via MF.

- The DCSF checks the DCAS URL for fetching the Avatar ID List from the BAR for the user of UE#1.

Editor's note:	The information that the DCSF should provide to DCAS and BAR to indicate this Avatar ID List request is for the user of UE#1 is decided by the authorization and authentication procedures which will be defined by SA3.

- The DCSF requests to the DCAS via MDC2 to fetch the Avatar ID List for the user of the avatar call DC application by using the DCAS URL.

- The DCAS requests to the BAR via MDC2 the Avatar ID List that is allowed for the user of the avatar call DC application.

- The BAR responses to the DCAS via MDC2 the Avatar ID List that is allowed for the user of the avatar call DC application.

- The DCAS responses to the DCSF via MDC2 the Avatar ID List for the user of the avatar call DC application.

- The Avatar ID List for the user of the avatar call DC application is delivered to the UE#1 through bootstrap data channel. The response is sent from DCSF via MF.

Editor's note:	Whether the UE#1 can download/update/delete the Avatar Representations before the application data channel setup is FFS. For example, if there is only one Avatar ID that can be selected for the user of the UE#1, or if the Avatar ID(s) locally configured or locally created in the UE mismatch with the Avatar ID List provided by the BAR.

# * * * * End of changes * * * *