3GPP TSG-SA2 Meeting #166	 S2-2412428

# 18 - 22 November, 2024, Orlando, USA								    		   (revision of S2-2411879)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0584 | rev | 8 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on Data Collection by LMF procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Lenovo], Qualcomm?, [Huawei], vivo, nokia?, xiaomi? |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CR provides updates to procedure for Data Collection from the UE described in clause 5.18.1 and also addresses the following ENs. Editor's note: Whether the AMF can further select and return SUPIs in step 2 and whether the AMF needs to consider the UE capabilities supporting AI/ML based positioning is FFS. Editor's note: Whether and how LMF is informed about the UE capability to report input data for ML Model training is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following clarifications are made: - LMF may be triggered by NWDAF to collect data for training an AIML model for Direct AIML positioning Rev 1 includes changes from S2-2410673 - Clarify the interaction between AMF and LMF in step 2. - Clarify the procedure between LMF and UE in step 4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedure for data collection from the UE is not complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.1, 6.X (new), 6.x.1 (new), 6.x.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR should be treated together with CR 0574 CR0595 and CR 0597 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | [The following contents are added in rev 8] Since the status of a UE may change during use, e.g. high load or low battery or user interactions, the UE may reject to provide input data for model training. Additionally, the LMF should store the collected data in the ADRF for access by the NWDAF, as agreed in the conclusion, which is currently missing. This CR (rev 8) proposes to add procedures for UE rejecting the data collection and LMF storing data in the ADRF. Changes in the merged revision: Address EN: Conditions for the LMF to trigger data collection is FFS, e.g., whether and how the LMF can consider load to determine when to perform step 4 is FFS Add the AMF discovery procedure; Add the description of that LMF may provide the maximum number of UE to AMF. Remove EN: Editor’s Note: Conditions for the UE to report measurement data are FFS, by adding the description of UE cancellation or rejection. Clarify that data collection by LMF supports AIML model training and AIML model performance monitoring. |  |  |  |  |  |  |  |  |

************************************ FIRST CHANGE *****************************

5.18.1	Void

# * * * * Next change * * * *

6.X	Procedures of data collection for LMF-based AI/ML Positioning

6.x.1	General

This clause describes the procedure for data collection by the LMF/NWDAF for input data to train or monitor the performance of the LMF-based AI/ML models for AI/ML-based positioning.

6.x.2	Data Collection from UE at LMF for the LMF-based AI/ML positioning

The LMF needs to obtain input data for LMF-based AI/ML Positioning model training, performance monitoring and may also request the UE to provide the UE location. The LMF-based AI/ML Positioning model is trained to perform UE positioning for UEs located in an area of interest that may expand over multiple TA or cover multiple NG-RAN nodes.

Figure 6.X.2-1: Data collection from UE by LMF for the LMF-based AI/ML positioning

1.	The LMF starts data collection for the purpose to train an AI/ML Model for UE Positioning (this may be based on an internal trigger in the LMF or a request from the NWDAF). The LMF invokes an Nnrf_NFDiscovery_Request service operation to an NRF to discover AMF(s) that served for the area of interest. The LMF may subscribe at an NWDAF for Network Performance Analytics to decide when to trigger data collection.

2.	The LMF subscribes to AMF to retrieve the list of SUPIs located in an area of interest and the UE Positioning Capability for each UE using Namf_EventExposure_Subscriber_Request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest" and “UE Positioning Capability”.

Editor's Note:	 Whether the maximum number of UEs is needed is FFS.

3.	The AMF send Namf_EventExposure_Subscriber_Response ("list of SUPIs in the area of interest"). If “UE Positioning Capability” is also requested in step 1, AMF includes UE Positinong Capabiliy, UE User Plane Positioning Capabilities if available for each SUPI in the response message sent to LMF.

4.	The LMF further determines the UEs from the list of SUPIs that received from AMF in step 2 for data collection based on UE Positioning Capability, UE User Plane Positioning Capabilities, UE consent check result, the PRU information available in the LMF and operator’s policy.

Editor's note:	Further details on user consent for data collection for model training or model performance monitoring for LMF-based AI/ML Model positioning will be aligned with SA WG3.

5.	For each UE that provides consent to data collection for model training and can support data collection the LMF initiates a request for input data. The UE could reject the data collection request from the LMF considering UE status (e.g. low battery) and user’s input (e.g. user’s not willing). If the UE accecpted data collection request, the UE also could cancel the data collection as defined in clause 6.3.X.

Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the UE is FFS. The procedure will be coordinated with RAN WGs.

Editor's Note:	The input data used for AI/ML based positioning will be decided by RAN WGs.

At the time the LMF decides that the input data for a UE are not needed, e.g. the user consent for data collection for model training is revoked, the LMF stops to collect the input data from the UE.

# * * * * Next change (ALL new text) * * * *

6.3.X	Cancellation of Reporting of data collection by a UE

The procedure described in Figure 6.3.X-1 enables a UE to cancel a data collection by LMF to train the LMF-based AI/ML Positioning model as described in clause 6.X.2, for example if the UE is powered off or if the UE cancels the data collection based on user's input (e.g. UE is not willing to report input data).

Figure 6.3.X-1: UE Cancellation of a data collection

1.	The LMF performs a data collection procedure as defined in clause 6.X.2.

2.	Same as step 2 in clause 6.3.2.

3.	same as step 6 in clause 6.3.2.

************************************ END OF CHANGE *******************************************