SA WG2 Meeting #166	S2-2412433

18 - 22 November, 2024, Orlando , US	(rev of S2-2411560/1195)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1161 | rev | 6 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | High-level description for Vertical Federated Learning when AF is as Server. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI, Ericsson, ZTE, ICS, CMCC, ETRI, Huawei, Samsung, OPPO? |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on conclusions for KI#2: 5GC Support for Vertical Federated Learning in clause 8.2 of TR23.700-84, this CR aims to specify the high-level functionality for vertical federated learning between AF and NWDAF(s) when AF is VFL server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 5.4 is modified to resolve some Editor Notes. Terminology is aligned. Editor's note: Terminology will be aligned to clause 6.2H.2 in future meeting. The detailed references are added. Editor's note: Detailed references to clause 6.2H.2 will be added in future meeting. The ENs are modified to "Editor's note: If additional functionality is specified in clause 6.2H, the reference to the corresponding clause will be added". If the functionality is specified, the feature alignment, accuracy monitoring, and/or any other additional functionality will be detailed in clause 6.2H. In the case, only the reference will be documented in clause 5.4. Editor's note: Details regarding features alignment functionality or whether the functionality needs to be specified are FFS. And Editor's note: Whether and how to provide initial model from VFL server to VFL clients based on VFL client request is FFS. And Editor's note: Accuracy monitoring in VFL when VFL server is NWDAF is FFS. And Editor’s Note: Whether and how change of samples during a VFL training is supported is FFS. In this release, when NWDAF is acting as VFL server or VFL client, the NWDAF is assumed to be containing both AnLF and MTLF. Editor's note: For an NWDAF impacts of the split into AnLF and MTLF are FFS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL functionality when AF is VFL server is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

## 5.4	Vertical Federated Learning (VFL)

Vertical Federated learning is a machine learning technique working without exchanging/sharing of local data set, while maintaining some level of coordination amongst VFL participants, when training and inference are performed on local ML Models, wherein the local data set in different VFL Participant for local model training have different feature spaces for the same samples (e.g. UE IDs). Vertical Federated Learning may involve multiple NWDAFs and AFs.

For Vertical Federated Learning, there may be one NWDAF or one AF acting as a VFL server and one or multiple NWDAF(s) and/or one or multiple AF(s) acting as VFL Client(s). Vertical Federated Learning is available among NWDAFs or between NWDAF(s) and AF(s) within a single PLMN or between an AF and NWDAF(s) in a single PLMN.

The main functionalities of VFL server and VFL client include:

VFL server:

-	An NWDAF or trusted AF acting as VFL server discovers and selects VFL client(s) (NWDAF(s) and/or AF(s)) to participate in a VFL procedure.

NOTE: When an untrusted AF is acting as VFL server, NEF discovers and selects candidate VFL client NWDAFs, then the AF determines final set of VFL clients.

-	It requests VFL clients to do local ML model training for an Analytic ID, it assigns VFL correlation ID, and it requests to report intermediate results.

-	It optionally locally trains ML Model with the available local data set (i.e. data set with label).

-	It combines intermediate results from VFL client(s) and VFL server, and computes intermediate training results (e.g. gradient information, loss information) for updating its own local ML Model and the ML Models of VFL clients during the VFL training process and sends the intermediate training results towards VFL clients involved in the joint VFL training process. VFL server may send and receive separate message for each client.

NOTE: NEF forwards the message from/to NWDAF to/from untrusted AF.

-  It initiates the VFL inference process using VFL correlation ID.

-	It combines local inference result from VFL clients and generates the final VFL inference result.

-	It may send the final VFL inference result to the consumer.

-	 It supports to monitor the accuracy of the VFL model.

VFL client:

-	It locally trains ML Model with the available local data set (i.e. data set without label), which includes the data that may not be allowed to be shared with other VFL clients due to e.g. data privacy, data security, data access rights.

-	It computes the intermediate results for their local ML Models involved in the VFL training and provide reports with the intermediate results to the AF or NWDAF acting as VFL server.

-	It performs inference based on the local model and local data and provides inference results to VFL server.

Vertical Federated Learning includes the following procedures:

- Registration of the NF profile including a list of VFL related information to NRF. Registration of the NWDAF profile to NRF is described in clause 5.2. Registration of the AF profile to NRF is described in clause 5.X. The procedure for registration and discovery of VFL server and VFL client is described in clause 6.2H.2.1.

- Preparation for VFL including sample alignment to ensure that all the VFL participants has common samples when training ML models as described in clause 6.2H.2.2.

- Training for VFL as described in clause 6.2H.2.3.

- Inference for VFL as described in clause 6.2H.2.4.

NOTE: In this release, when NWDAF is acting as VFL server or VFL client, the NWDAF is assumed to be containing both AnLF and MTLF.

Editor's note: If additional functionality is specified in clause 6.2H, the reference to the corresponding clause will be added	.

Editor's note:	For an NWDAF impacts of the split into AnLF and MTLF are FFS.

Editor’s Note: Whether and how change of samples during a VFL training is supported is FFS.

# >>>>END OF CHANGES<<<<