SA WG2 Meeting #166	S2-2412551

# Orlando, USA, 18 – 22 November 2024                         (revision of S2-2412507)

Source: 	CATT, NEC

Title: 	KI#1 Conclusions on assistance information to Reader

Document for: 	Approval

Agenda Item: 	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes to conclude assistance information from CN to RAN/UE Reader.

1	Discussion

This paper discusses the assistance information from CN to Reader and makes conclusion proposals.

In order for the Reader to perform AIoT service requested by AF (e.g. Inventory) properly, some assistance information needs to be provided by CN (i.e. AIoTF) to Reader. On the other hand, some assistance information is useful for radio resource allocation by Reader which has been discussed in both SA2 and RAN WGs. In the KI#1 interim conclusion agreed at the SA2#165 meeting there is also an EN about assistance information to Reader.

Editor's note:	Information needed for radio resource allocation to readers is FFS and requires cooperation with RAN WG2/RAN WG3.

Please not that this paper does not cover Filter/Mask information and Reader ID information aspects which are assumed to be discussed in other papers.

In the following section, this paper summarizes the proposals on assistance information to Reader from solutions of TR 23.700-13, and also considers the RAN WGs agreements to make conclusions.

- SA2 solutions on assistance information to Reader

The TR 23.700-13 solutions on assistance information to Reader are summarized in the following table.

Table-1 TR 23.700-13 solutions on assistance information to Reader

| Solution# | Assistance information to Reader |
| --- | --- |
| #3 | periodicity information, a periodicity value to request the inventory to be performed periodically provided by AF. |
| #4 | the number of AIoT devices, is the exact number or an approximate number range provided by AF. |
| #6 | AIOT service policy includes destination transmission area, priority, service duration (e.g. start and stop time), periodic transmission frequency during the service duration if it’s periodic transmission, etc. |
| #8, #22, #30, #43 | inventory strategy information provided by AF. The inventory strategy information contains, e.g. the inventory frequency and inventory period to guide the readers to perform the inventory periodically. It also indicates whether all the targeted devices need to respond (full inventory), or only those who haven't performed the inventory procedure (delta inventory) should respond. optional location required information provided by AF. The location required indicates whether the AF requests the location information of the AIoT devices provided. The report aggregation info indicates whether the reports need to be aggregated or not for a specific aggregation period, and whether the reports are needed after the aggregation period. Provided by AF. |
| #17 | indication that transfer of AIoT NAS Command message subsequently provided by AIoTMF. indication of data reporting provided by AF. Task ID provided by AIoTMF. |
| #18, #31 | Service Type: This information is used to define Ambient IoT service types, such as Inventory, Read, Write, and so on. Provided by AF. Time: when the service will be carried out exactly provided by AF. Periodical indication: the service operation will be executed periodically. Provided by AF. |

From the above summarized proposals, it can be seen that from AIoT service operation perspective, the following information provided by the AF are supported by most solutions and needs to be provided to Reader for performing AIoT services.

-	periodicity information

-	location required information

-	report aggregation info

Observation 1: In TR 23.700-13, most solutions propose that CN provides AF provided periodicity information, location required information, and report aggregation info as assistance information to Reader.

Proposal 1: AIoTF provides AF provided periodicity information, location required information, and report aggregation info as assistance information to RAN/UE Reader.

- RAN WGs agreements on assistance information to Reader

RAN2 had discussed and reached agreements that assistance information from CN may be useful for Reader radio resource allocation.

The related agreements made in RAN2#127bis meeting are as followings:

Agreements

From RAN2 perspective, it is beneficial for the reader to know an estimate of expected D2R message size.  Two options can be captured: 1) from the CN and 2) from the device (simple message size indication).    Capture the advantages/disadvantages.

The D2R message size would be beneficial but it is not essential.

Ask SA2 if it is possible to provide the expected (e.g. approximate, estimate, exact, max) future response D2R message size.  Is it always available, sometimes, or never.

The related agreements made in RAN2#127 meeting are as followings:

Agreements

- At least the following information are considered useful to be visible to the reader from CN

-	The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful

-	targeted for one or more than one devices;

-	approximate number of target devices (if available).

FFS on mandatory/optional

2	RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIOT interface as upper layer data.

From the above RAN2 agreements, it is clear that service type of A-IoT (e.g. inventory, command and approximate number of target devices (if available) are useful to be visible to the reader from CN. It has not been determined by RAN2 yet whether it is necessary for CN to provide an estimate of expected D2R message size as two options are under discussion in RAN2.

Observation 2: RAN agreements indicate that service type of A-IoT (e.g. inventory, command) and approximate number of target devices (if available) are considered useful to be visible to the reader from CN.

Proposal 2: AIoTF provides AIoTF determined AIoT service type (i.e. Inventory or Command) and AF provided number of AIoT devices as assistance information to RAN/UE Reader.

RAN2 also made some agreements on AIoT QoS aspect in RAN2#125bis meeting.

Agreements - Unless explicitly stated all agreements apply to all device types and for both topologies. - From RAN2 perspective, the aim is that the design on the interface between reader and A-IoT device is common for topology 1 and topology 2. - RAN2 will support two use cases, “inventory” and “command”. - RAN2 assumes that the device will not support tracking/RAN area update procedure. - PDCP layer is not needed. FFS how to handle AS security (if needed pending SA3 discussion) and any other really needed functionalities. - RAN2 will continue the study of ambient IoT assuming no support of AS security until SA3 provides further input. - SDAP is not supported for UP protocol stack. - RAN2 assumes that no per-packet QoS and no per-QoS flow is supported at AS level (for both UL/DL). FFS how to handle the general QoS requirements from SA2 - Periodical System information and MIB are not supported by AIoT devices. - RAN2 assumes, AIoT devices are not required to support ASN.1 encoding/decoding.

Although QoS of AIoT services were not discussed before in SA2, SA1 has defined performance service requirements for all 4 types of AIoT services as specified in TS 22.369. Take Inventory service as an example, the Max. allowed end-to-end latency is typically in seconds level, and the Communication range is 30 m – 50 m indoor and 200 m - 400 m outdoor, etc. To support the SA1 defined service requirements, some QoS information needs to be provided to Reader.

Observation 3: SA1 has defined performance service requirements for all 4 types of AIoT services as specified in TS 22.369, and QoS information needs to be provided to Reader to fulfil these service requirements.

Proposal 3: AIoTF provides AF provided QoS information (including end-to-end latency, communication range) as assistance information to RAN/UE Reader.

2	Proposal

It is proposed to update TR 23.700-13 as follows.

# * * * * First change * * * *

8.1	Interim Conclusion on Key Issue #1

8.1.1	General

Key issue #1 includes the following aspects:

-	System architecture identified along with the solutions for KI#2 and KI#3.

Key issue#2 aspect on "Ambient IoT Device subscription management" and key issue#3 aspect on "Ambient IoT service exposure" is considered in this section.

The following aspects common for Topology 1 and Topology 2 are concluded as principles for normative work:

Editor's note:	Final conclusions are assumed to be taken in coordination with RAN WGs.

Editor's note:	Which architecture options described in the following clauses will be concluded is FFS.

1.	A new core network function is introduced to support Ambient IoT (e.g. AIoTF) service for both the topology 1 and topology 2. The AIoTF performs the following functionality.

a.	The AIoTF manages the device related information.

Editor's note:	Whether and what AIoT Device related information (e.g. AIoT Device last known Reader ID, optionally the result of AIoT device validation result, etc.) is stored in the AIOTF, are FFS.

Editor's note:	Whether the UDM should store the device related information is FFS.

b.	The AIoTF registers itself in the NRF with its NF profile.

Editor's note:	The details of the NF profile are FFS.

c.	The AIoTF receives an AIoT service request from the AF and triggers the BS/UE readers to perform AIoT service operations towards the AIoT Devices (s).

d.	The AIoTF aggregates the service operation results (including the removal of the duplicated devices records) from BS Readers and UE Readers and sends to AF.

e.	The AIoTF may provide the following assistance information to RAN/UE Reader:

-  AIoT service type (.e.g. Inventory, Command)

-  approximate number of AIoT devices based on AF request

-  approximate D2R message size based on AF request

Editor's note:	Other assistance information may be added in the normative phase if necessary.

Editor's note:	Further information on AIoT service type may be determined in normative phase in cooperation with RAN WGs.

NOTE A:	If there are multiple Readers selected for the AIoT Service, the AIoTF may provide the approximate number of AIoT devices to each Reader based on implementation.

NOTE B:	The approximate D2R message size considering the overhead of AIoT Device NAS layer may be determined in normative phase in cooperation with CT1 and SA3.

Editor's note:	For RRC based solution of topology 2, whether the aggregation can be performed by the RAN is FFS and coordination with RAN WGs is needed.

Editor's note:	How the aggregation can be done is FFS.

2.	A Permanent AIoT Device IDis stored in the AIoT Device and the UDM or a Credential Holder's AAA server.

Editor's note:	Whether and how the validation of the Ambient IoT Device ID is done will be concluded by SA WG3.

3.	The AIoT Device does not distinguish whether the connectivity topology is Topology 1 or Topology 2, nor the transport used by the AIoT Reader.

NOTE:	The AIoT device is also agnostic to the potential different architectures if more than one architecture is concluded for both the topology 1 and topology 2.

Editor's note:	Whether an AIoT NAS protocol is supported between the AIoT Device and the AIoTF, and what is the supported functionality, is FFS.

# * * * * End of Changes * * * *