SA WG2 Meeting #166 	S2-2412564

# 18 - 22 Nov 2024, Orlando, USA

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5113 | rev | 3 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UPF event exposure service at N4 session release |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, ETRI, ZTE, Samsung, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are several issues in existing specification - UPF event exposure service handling at N4 session release was integrated including UPF relocation case and was accepted in TS23.501 at the last meeting. However, the related procedure was not included in TS23.502 yet. - The new parameters are missing in the service operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add new parameters in the SMF/UPF service operation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment within the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.8.3.3, 5.2.26.2.2, 5.2.26.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

##### 5.2.8.3.3	Nsmf_EventExposure_Subscribe service operation

Service operation name: Nsmf_EventExposure_Subscribe.

Description: This service operation is used by an NF to subscribe or modify a subscription for event notifications on a specified PDU Session or for all PDU Sessions of one UE, group of UE(s) or any UE.

Input, Required: NF ID, Target of Event Reporting as defined in clause 5.2.8.3.1, (set of) Event ID(s) defined in clause 5.2.8.3.1, Notification Target Address (+ Notification Correlation ID), Event Reporting Information defined in Table 4.15.1-1.

Input, Optional: Event Filter(s) associated with each Event ID; Event Filter(s) are defined in clause 5.2.8.3.1, Subscription Correlation ID (in the case of modification of the event subscription), Expiry time, DNN, S-NSSAI, DNAI, UPFId, UPF event exposure information (Type of measurement, granularity of measurement, reporting suggestion information, Remaining Data reporting indication, etc. associated with the UPF Event IDs as described in clause 4.15.4.5).

NOTE:	The SMF is generally meant to determine the UPF to contact for a subscription related to UPF event exposure. UPF ID is only provided to indicate a UP Path as defined in Table 6.4.1-1 of TS 23.288 [50], i.e. when the NWDAF has received the target UPF as part of statistics of observed service experience on an UP path involving that UPF.

Output, Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator's policy).

Output, Optional: First corresponding event report is included, if available (see clause 4.15.1).

Notification Target Address (+ Notification Correlation ID) is used to correlate Notifications sent by SMF or UPF with this subscription.

* * * Next Change * * *

##### 5.2.26.2.2	Nupf_EventExposure_Notify service operation

Service operation name: Nupf_EventExposure_Notify

Description: This service operation reports the event and information to the consumer that has subscribed (implicitly or explicitly).

Input Required: Event ID, UE address (i.e. IP address or MAC address), Notification Correlation Information.

Input, Optional: UE ID, Event specific parameters as described in clause 5.2.26.2.1, time stamps for the measurements, Application Id(s), Packet Filter Set(s), Achieved sampling ratio, Subscription termination indication (with optionally a Cause code).

Output Required: Result Indication.

Output, Optional: None.

##### 5.2.26.2.3	Nupf_EventExposure_Subscribe service operation

Service operation name: Nupf_EventExposure_Subscribe

Description: This service operation is used by an NF to subscribe or modify a subscription to UPF event exposure notifications e.g. for the purpose of UPF data collection on a specified PDU Session or for all PDU Sessions of one UE or any UE.

Input, Required: NF ID, Target of Event Reporting "any UE" or as defined in Table 4.15.4.5.1-1 when a specific UE is targeted), (set of) Event ID(s) defined in clause 5.2.26.2.1, Notification Target Address (+ Notification Correlation ID), Event Reporting Information defined in Table 4.15.1-1 (TS 29.564 [93] defines the possible parameters).

Input, Optional: Subscription Correlation ID (in the case of modification of the event subscription), Expiry time, DNN, S-NSSAI, either Application ID(s) or Traffic Filtering Information, Type of Measurement, Granularity of Measurement, Reporting suggestion information, Subscription termination reporting indication, Remaining data reporting indication, UE (private) IP address, IP domain, IP address of the remote end, port number of the remote end.

Output, Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator's policy).

Output, Optional: First corresponding event report is included, if available (see clause 4.15.1). Notification Target Address (+ Notification Correlation ID) is used to correlate Notifications sent by UPF with this subscription.

* * * End of Change * * *