3GPP TSG-SA WG2 Meeting #166	S2-2412879

# Orlando, USA, 18 – 22 November 2024						               (revision of S2-2412603)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0499 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the Multi-hop U2N Relay IP address allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the CT1 LS (S2-2411288/C1-246002), the following questions are asked: Q1- Can the Rel-19 multi-hop relay 5G ProSe capabilities be associated with the existing pre-Rel-19 5G Prose capabilities? Q2- Can the Rel-19 multi-hop relay 5G ProSe Requested UE policies bits be associated with the existing pre-Rel-19 5G Prose Requested UE policies? Q3- Does "DHCP proxy" refer to DHCP relay agent as defined in related IETF RFCs, e.g. RFC 3046? In order to address the above CT1 questions, some clarifications are made to TS 23.304. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 6.6.2, add a NOTE to clarify that encoding of the ProSe capabilities is defined by stage 3. In clause 4.3.1, add a NOTE to clarify that encoding of the policy bits is defined by stage 3. In clause 6.4.3.8.2, it is clarified the IM Relay acts as a DHCP server or a DHCP client. In clause 6.5.1.4, add a reference to clause 6.4.3.9.2 (defined by CR#0503). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear descriptions about Multi-hop U2N Relay IP address allocation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.3.1, 6.4.3.8.2, 6.5.1.4, 6.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR refers a new clause 6.4.3.9.2 which is defined in CR#0503. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * 1st Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[3]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[4]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[5]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[6]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[7]	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS)".

[8]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[9]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[10]	Void.

[11]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[12]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[13]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode".

[14]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station in idle mode".

[15]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[16]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[17]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[18]	IETF RFC 3927: "Dynamic Configuration of IPv4 Link-Local Addresses".

[19]	IETF RFC 826: "An Ethernet Address Resolution Protocol".

[20]	Void.

[21]	3GPP TR 23.752: "Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS)".

[22]	3GPP TS 32.277: "Proximity-based Services (ProSe) charging".

[23]	3GPP TS 24.554: "Proximity-services (ProSe) in 5G System (5GS) protocol aspects; Stage 3".

[24]	IETF RFC 2131: "Dynamic Host Configuration Protocol".

[25]	IETF RFC 4039: "Rapid Commit Option for the Dynamic Host Configuration Protocol version 4 (DHCPv4)".

[26]	Void.

[27]	Void.

[28]	3GPP TS 38.351: "NR; Sidelink Adaptation Layer Protocol".

[29]	3GPP TS 33.503: "Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS)".

[30]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[31]	3GPP TS 23.167: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; IP Multimedia Subsystem (IMS) emergency sessions".

[32]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[33]	3GPP TS 22.268: "Public Warning System (PWS) requirements".

[34]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[35]	IETF RFC 7181: "The Optimized Link State Routing Protocol Version 2 (OLSRv2)".

[36]	IETF RFC 6130: "Mobile Ad Hoc Network (MANET) Neighborhood Discovery Protocol (NHDP)".

[xx]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

* * * * 2nd Change * * * *

### 4.3.1	UE

Any 5G ProSe-enabled UE may support the following functions:

-	Exchange of information for 5G ProSe Direct Discovery between 5G ProSe-enabled UE and the 5G DDNMF over PC3a reference point.

-	Procedures for 5G ProSe Direct Discovery of other 5G ProSe-enabled UEs over PC5 reference point.

-	Procedures for 5G ProSe Direct Communication over PC5 reference point, including Broadcast, Groupcast and Unicast mode 5G ProSe Direct Communication.

-	Procedures to act as a 5G ProSe Layer-2 UE-to-Network Relay.

-	Procedures to act as a 5G ProSe Layer-3 UE-to-Network Relay.

-	Procedures to act as a 5G ProSe Intermediate UE-to-Network Relay.

-	Procedures to act as a 5G ProSe Layer-2 Remote UE.

-	Procedures to act as a 5G ProSe Layer-3 Remote UE.

-	Procedures to act as a 5G ProSe Layer-2 UE-to-UE Relay.

-	Procedures to act as a 5G ProSe Layer-3 UE-to-UE Relay.

-	Procedures to act as a 5G ProSe Layer-2 End UE.

-	Procedures to act as a 5G ProSe Layer-3 End UE.

-	Procedures for communication path switching between PC5 and Uu reference points.

-	Procedures for Multi-path communication via Uu and via 5G ProSe UE-to-Network Relay.

-	Indicating UE Policy Provisioning Request in UE Policy Container for UE triggered 5G ProSe Policy provisioning, which requests one or multiple types of policies/parameters as listed below:

-	Policy/parameters for 5G ProSe Direct Discovery;

-	Policy/parameters for 5G ProSe Direct Communication;

-	Policy/parameters for 5G ProSe Layer-2 Remote UE;

-	Policy/parameters for 5G ProSe Layer-3 Remote UE;

-	Policy/parameters for 5G ProSe Layer-2 UE-to-Network Relay;

-	Policy/parameters for 5G ProSe Layer-3 UE-to-Network Relay;

-	Policy/parameters for 5G ProSe Layer-3 Intermediate UE-to-Network Relay;

-	Policy/parameters for 5G ProSe Layer-2 End UE;

-	Policy/parameters for 5G ProSe Layer-3 End UE;

-	Policy/parameters for 5G ProSe Layer-2 UE-to-UE Relay;

-	Policy/parameters for 5G ProSe Layer-3 UE-to-UE Relay;

-	Policy/parameters for 5G ProSe Multi-hop Layer-3 UE-to-Network Relay Discovery;

-	Policy/parameters for 5G ProSe Multi-hop Layer-3 UE-to-UE Relay Discovery;

-	Policy/parameters for 5G ProSe Communication via Multi-hop Layer-3 UE-to-Network Relay;

-	Policy/parameters for 5G ProSe Communication via Multi-hop Layer-3 UE-to-UE Relay.

NOTE:	Encoding of these policy bits is defined in TS 24.554 [23].

-	Receiving the 5G ProSe Policy from 5GC over N1 reference point.

-	Configuration of parameters for 5G ProSe Direct Discovery, 5G ProSe Direct Communication, 5G ProSe Single-hop UE-to-Network Relay (e.g. including IP addresses, ProSe Layer-2 Group IDs, see clause 5.1), 5G ProSe Multi-hop Layer-3 UE-to-Network Relay, 5G ProSe Single-hop UE-to-UE Relay and 5G ProSe Multi-hop Layer-3 UE-to-UE Relay. These parameters can be pre-configured in the UE, or, if in coverage, provisioned or updated by signalling over the N1 reference point from the PCF in the HPLMN or over PC1 reference point from the ProSe Application Server.

-	Reporting the following capabilities to 5GC over the N1 reference point:

-	5G ProSe Capability.

* * * * 3rd Change * * * *

##### 6.4.3.8.2	IP address/prefix management for 5G ProSe Layer-3 Multi-hop UE-to-Network Relay

When IP based PDU session types are used by the 5G ProSe UE-to-Network Relay, the Intermediate UE-to-Network Relay needs to act as an IP Router.

The IP address allocation for Remote UE and Intermediate UE-to-Network Relay is performed by the 5G ProSe UE-to-Network Relay.

To support DHCP based IP address configuration, the 5G ProSe Intermediate UE-to-Network Relay acts as a DHCP client towards the UE-to-Network Relay or the parent Intermediate UE-to-Network Relay and as a DHCP server towards the Remote UE or the child Intermediate UE-to-Network Relay. To support IPv6 Stateless Address auto-configuration, the 5G ProSe Intermediate UE-to-Network Relay relays the Router Solicitation and Router Advertisement messages between the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay.

The 5G ProSe Intermediate UE-to-Network Relay builds its local IP routing table based on the Remote UE info from the link modification procedures as described in clause 6.4.3.8.1. The local IP routing table is per RSC and used based on RSC of the link the packet is received.

* * * * 4th Change * * * *

#### 6.5.1.4	5G ProSe Communication via Multi-hop 5G ProSe Layer-3 UE-to-Network Relay without N3IWF (based on Model B Discovery)

Figure 6.5.1.4-1: Connection establishment via Multi-hop 5G ProSe Layer-3 UE-to-Network Relay without N3IWF support

Connection establishment via Multi-hop 5G ProSe Layer-3 UE-to-Network Relay without N3IWF support as described in clause 6.5.1.1 is used for multi-hop UE-to-Network Relay after Model B Discovery with the following differences and clarifications:

1.	Service authorization and provisioning are performed for the 5G ProSe Layer-3 UE-to-Network Relay, Intermediate Relay(s) and 5G ProSe Layer-3 Remote UE.

2.	The 5G ProSe Layer-3 UE-to-Network Relay may establish a PDU Session for relaying.

3.	The 5G ProSe Layer-3 Remote UE performs Model B discovery of a 5G ProSe Layer-3 UE-to-Network Relay. The Remote UE obtains the path information to the UE-to-Network Relay(s) from the discovery procedure.

4.	The 5G ProSe Layer-3 Remote UE selects a path and establishes a connection for unicast mode communication as described in clause 6.4.3.9.

5.	For IP PDU Session Type and IP traffic over PC5 reference point, the UE-to-Network Relay assigns IPv6 prefix or IPv4 address to Intermediate Relay(s) and the Remote UE as defined in clause 6.4.3.9.2.

6.	The 5G ProSe Layer-3 Remote UE may provide PC5 QoS Info and PC5 QoS rule(s) to the 5G ProSe Layer-3 UE-to-Network Relay using Layer-2 link modification procedure via Intermediate Relay(s). Intermediate Relays may split the PC5 QoS hop-by-hop as described in clause 5.6.2.

Editor's note:	It is FFS how to handle QoS.

7.	For the Remote UE Report, the Remote User ID is an identity of the 5G ProSe Layer-3 Remote UE.

For IP PDU Session Type, the Remote UE info is IP info related to Remote UE.

-	for IPv4, the 5G ProSe Layer-3 UE-to-Network Relay shall report IP address and TCP/UDP port ranges that are assigned to 5G ProSe Layer-3 Remote UE(s) by the UE-to-Network Relay;

-	for IPv6, the 5G ProSe Layer-3 UE-to-Network Relay shall report IPv6 prefix(es) assigned to individual 5G ProSe Layer-3 Remote UE(s).

For Ethernet PDU Session Type, the Remote UE info is Remote UE MAC address which is detected by the 5G ProSe Layer-3 UE-to-Network Relay.

NOTE:	For IP type of PDU Sessions, the Intermediate Relays forward the Downlink packet to the correct PC5 links based on destination IP address of the packet. For Ethernet type of PDU Sessions, the Intermediate Relay forwards the Downlink packets according to the destination Ethernet MAC address.

* * * * 5th Change * * * *

### 6.6.2	Registration procedure

The Registration procedure for UE is performed as defined in TS 23.502 [5] clause 4.2.2.2 with the following additions:

-	The UE includes the 5G ProSe Capability as part of the "5GMM capability" in the Registration Request message. The AMF stores the 5G ProSe Capability for 5G ProSe operation.

-	The 5G ProSe Capability indicates whether the UE supports one or more of the following ProSe capabilities:

-	5G ProSe Direct Discovery;

-	5G ProSe Direct Communication;

-	5G ProSe Layer-2 UE-to-Network Relay;

-	5G ProSe Layer-3 UE-to-Network Relay;

-	5G ProSe Layer-2 Remote UE;

-	5G ProSe Layer-3 Remote UE;

-	5G ProSe Layer-2 UE-to-UE Relay;

-	5G ProSe Layer-3 UE-to-UE Relay;

-	5G ProSe Layer-2 End UE;

-	5G ProSe Layer-3 End UE;

-	5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay;

-	5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay;

-	5G ProSe Layer-3 Intermediate UE-to-Network Relay;

-	5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay;

-	5G ProSe Layer-3 End UE supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay.

NOTE 1:	Encoding of these capabilities is defined in TS 24.501 [xx].

-	The AMF obtains the 5G ProSe subscription data as part of the user subscription data from UDM during UE Registration procedure using Nudm_SDM service as defined in clause 4.2.2.2.2 of TS 23.502 [5].

-	The AMF determines whether the UE is authorised to use 5G ProSe services based on UE's 5G ProSe Capability and the ProSe Service Authorisation included in the subscription data received from UDM as specified in clause 5.7. ProSe NR UE-PC5-AMBR is also provided to the AMF as part of the subscription data for 5G ProSe services. The AMF stores the authorized 5G ProSe Capability.

-	The AMF sends the authorized 5G ProSe Capability for 5G ProSe operation to PCF. Based on the received 5G ProSe Capability from the AMF, the PCF provides the PC5 QoS parameters for 5G ProSe to AMF. The AMF stores such information as part of the UE context.

-	If the UE is authorised to use 5G ProSe services, then the AMF shall include in a NGAP message sent to NG-RAN:

-	"5G ProSe authorised" information, including one or more of the following:

-	whether the UE is authorized to use 5G ProSe Direct Discovery;

-	whether the UE is authorized to use 5G ProSe Direct Communication;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 UE-to-Network Relay;

-	whether the UE is authorized to act as a 5G ProSe Layer-3 UE-to-Network Relay;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 Remote UE;

-	whether the UE is authorized to use multi-path communication via direct Uu path and via 5G ProSe Layer-2 UE-to-Network Relay as a 5G ProSe Layer-2 Remote UE;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 UE-to-UE Relay;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 End UE;

-	whether the UE is authorized to act as a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay;

NOTE 2:	The authorization for "5G ProSe Direct discovery" and "5G ProSe Direct communication" can be used for 5G ProSe Layer-3 UE-to-UE Relay, 5G ProSe Layer-3 End UE, 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay and 5G ProSe Layer-3 End UE supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay.

-	ProSe NR UE-PC5-AMBR, used by NG-RAN for the resource management of UE's PC5 transmission for 5G ProSe services in network scheduled mode.

-	the PC5 QoS parameters for 5G ProSe used by the NG-RAN for the resource management of UE's PC5 transmission for ProSe services in network scheduled mode.

-	If the UE is authorised to use 5G ProSe services, then the AMF should not initiate the release of the signalling connection after the completion of the Registration procedure. The release of the signalling connection relies on the decision of NG-RAN, as specified in TS 23.502 [5].

* * * * End of Changes * * * *