3GPP SA WG2 Meeting #166	S2-2412883

# Orlando, USA, 18 – 22 November 2024

Title:	Reply LS on the supporting 5G ProSe multi-hop Relays

Response to:	LS on the supporting 5G ProSe multi-hop Relays

Release:	Rel-19

Work Item:	5G_ProSe_Ph3

Source:	SA2

To:	CT1

Cc:	CT3, CT4

Contact Person:

Name:	Qiang Deng

E-mail Address:	dengqiang1@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.304 CR 0499, CR 0518

1. Overall Description:

SA2 thanks CT1 for the LS on the supporting 5G ProSe multi-hop Relays (S2-2411288/C1-246002).

SA2 has discussed the raised questions and would like to provide the following answers.

Q1- Can the Rel-19 multi-hop relay 5G ProSe capabilities be associated with the existing pre-Rel-19 5G Prose capabilities?

A1:	First, there is no association between the 5G ProSe Layer-3 Intermediate UE-to-Network Relay capability and the existing capabilities. Then, it can be assumed that a ProSe-enabled UE supporting multi-hop relay capabilities also supports single-hop relay capabilities. It is up to CT1 to specify how to encode these capabilities. It is also up to CT1 to decide how to encode the policies so that the UE can differentiate the policies for multi-hop Relay operation and single-hop Relay operation.

Q2- Can the Rel-19 multi-hop relay 5G ProSe Requested UE policies bits be associated with the existing pre-Rel-19 5G Prose Requested UE policies?

A2:	First, there is no association between the 5G ProSe Layer-3 Intermediate UE-to-Network Relay indication and the existing indications related to UE policies. Then, it can be assumed that a ProSe-enabled UE requesting multi-hop relay related policy/parameters also requests single-hop relay related policy/parameters. It is up to CT1 to specify how to encode these policy bits. It is also up to CT1 to decide how to encode the policies so that the UE can differentiate the policies for multi-hop Relay operation and single-hop Relay operation.

Q3- Does "DHCP proxy" refer to DHCP relay agent as defined in related IETF RFCs, e.g. RFC 3046?

A3:	The Intermediate UE-to-Network Relay acts as a DHCP client towards the UE-to-Network Relay or the parent Intermediate UE-to-Network Relay, and as a DHCP server towards the Remote UE or the child Intermediate UE-to-Network Relay.

SA2 has agreed the attached CR 0499 and CR 0518 to update the specification descriptions.

2. Actions:

To CT1 group:

ACTION:  SA2 kindly asks CT1 to take the above answers into account in their future work.

3. Date of Next SA2 Meetings:

SA2#166-AH-e                           20th - 24th January 2025		 	Online

SA2#167                                    17th – 21st February 2025	 	Athens, GR