3GPP TSG-WG2 Meeting #166	S2-2413026

# Orlando, USA, 2024-11-18 – 2024-11-22										(revision of S2-2412566)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5004 | rev | 9 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on subscription to UPF event exposure service via SMF during UPF relocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Ericsson, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Modifications needed in relocation procedures: - Subscribing to target UPF should be optional), Whether a new subscription should be sent or not: - If the UPF event consumer indicates the certain UPF id in subscription request or trigger direct subscription to certain UPF that UPF id indicates, after UPF relocation, the UPF event consumer may not trigger new subscription request to new UPF. - If the UPF event consumer indicates the certain DNAI in subscription request, and after UPF relocation the DNAI is changes, for example relocates from one DNAI to another DNAI, the UPF event consumer doesn’t not trigger new subscription request to new UPF in new DNAI. - Removal of Editor Note. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Set the SMF to subscribe the target UPF as optional and add clarification about why not to subscribe any more. - Procedure updated for the case SMF subscribes over N4. - Relocation reporting indication renamed to Remaining data reporting indication. - Added PDU Session update with N4 session release in Source UPF. Such release makes Nupf_EE unsubscribe unnecessary. - At N4 session release, UPF terminates the subscription and may send any data collected not yet. - The EN is solved as described above and therefore, removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear that the SMF always subscribes the UPF event. Unspecified procedure for all events and cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.4.5.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

##### 4.15.4.5.6	Information flow for subscription to UPF event exposure service via SMF during UPF relocation and PDU Session release

Figure 4.15.4.5.6-1: Subscription to UPF event exposure service via SMF during UPF relocation and PDU Session release

This information flow is used in the scenario of UPF relocation, e.g. the source UPF is I-UPF or L-PSA which is relocated in SSC mode 1, or the source UPF is PSA UPF which is relocated in SSC mode 3. The information flow also applies to PDU Session release.

1.	The UPF event consumer (e.g. NWDAF) subscribes to the UPF data via SMF by invoking Nsmf_EventExposure_Subscribe service operation, and the Target of Event Reporting indicates a specific UEThe consumer may include a Remaining Data reporting indication, which indicates how to handle the data collected by the source UPF when UPF relocation occurs, e.g. discard the collected data or send the collected data to the consumer.

2.	The SMF selects the source UPF and subscribes to the source UPF by invoking Nupf_EventExposure_Subscribe service operation or via N4 Session Modification, depending on the event.

3.	The source UPF sends the locally collected UPF data by invoking Nupf_EventExposure_Notify service operation to the UPF event consumer.

4.	The SMF determines that the UPF needs to be relocated or PDU Session needs to be released.

5.	In case of UPF relocation, the SMF inititates a PDU Session update to relocate UPF (for example procedure in clause 4.3.5.3) including N4 session release in source UPF. In case of PDU Session release, the SMF inititates a PDU Session release procedure, including N4 session release.

6.	When (source) UPF terminates the subscription upon release of the UE related N4 Session (step 5), the (source) UPF may send  any collected data not yet sent to the UPF event consumer taking into account local configuration and the Remaining Data reporting indication from SMF that indicates that the (source) UPF should send the collected data to the UPF event consumer. For example, for periodic reporting type of the UPF event exposure service when the UPF changes, the (source) UPF may report to the consumer any data collected between the time instance of the last notification to the consumer and the time instance of the UPF change. Or for event triggered reporting with Reporting suggestion information, when the UPF changes, the source UPF may report to the consumer any data that has been collected but not sent to the consumer yet.

7-8.	In case of UPF relocation, to continue the subscription of the UPF event consumer, if the UPF related information is matched with the subscription request in step 1, the SMF may select a target UPF and may re-subscribe to the target UPF based on the subscription from UPF event consumer in step 1. Depending on the event, SMF may have sent the subscription to target UPF in step 5 instead. The target UPF will send the locally collected UPF data by invoking Nupf_EventExposure_Notify service operation to the UPF event consumer.

In case of PDU Session release, steps 7-8 do not apply.

* * * End of Change * * *