3GPP TSG-WG SA2 Meeting #166	S2-2413027

18 - 22 November, 2024, Orlando, USA

Title:	LS on Clarification of Nnef_RetrieveInfoUAVFlight_Get service operation

Response to:

Release:	Rel-19

Work Item:	UAS_Ph3

Source:	SA2

To:	CT3

Cc:

Contact Person:

Name:	Ashok Kumar Nayak

E-mail Address:	ashok.nayak@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.256 CR#0141

1. Overall Description:

SA2 has progressed normative work for the UAS_Ph3 and would like to get feedback from CT3 regarding one service operation used by USS to trigger a retrieval of some assistance information exposed by NEF/UAS NF.

In the attached CR, a serving USS in step 4 (clause 5.13.2) invokes Nnef_RetrieveInfoUAVFlight_Get request to trigger the UAS NF/NEF to collect information for the USS changeover, including to determine a target USS. Based on the Nnef_RetrieveInfoUAVFlight_Get response in Step 8, the serving USS determines the target USS (Step 9), which may also invoke another Nnef_RetrieveInfoUAVFlight_Get request to the UAS NF/NEF to collect information for its part of the flight (Step 10). Once the target USS is selected by the serving USS and the target USS provides information for its leg of the flight, the serving USS invokes another service operation (Nnef_UAVFlightAssistance_Create request, Step 13 in clause 5.13.2) to the UAS NF/NEF with planned flight path information.

At the same time, the Nnef_RetrieveInfoUAVFlight_Get request/response is also used in the procedure for NEF-assisted pre-flight planning (clause 5.12.2, Steps 4 and 7 as specified in S2-2411160) to trigger UAS NF/NEF to collect information for pre-flight planning for the serving USS.

In both cases, the Nnef_RetrieveInfoUAVFlight_Get request does not create subscription to any events and is not expected to be used, for instance, for in-flight monitoring purposes (clause 5.12.3 in S2-2411160).

On the other hand, some company believes that NEF/UAS NF can provide further USS changeover details whenever the event is occurred (e.g. from USS2 to USS3) as the UAV is still flying and hence instead of Nnef_RetrieveInfoUAVFlight_Get (with “Request/Response” semantics), Nnef_RetrieveInfoUAVFlight_Create/Notify service operation (with “Subscribe/Notify” semantics) would be more suitable.

2. Actions:

To CT3 group.

ACTION: 	SA2 kindly asks CT3 to provide feedback on the approach taken by SA2 on the Nnef_RetrieveInfoUAVFlight_Get service operation.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting#166 Ad Hoc-e              January 20 – 24, 2025	Online

TSG-SA2 Meeting#167	         February 17 – 21, 2025	Athens, Greece