# 3GPP TSG-SA WG2 Meeting #166	S2-2411497

Orlando, US, 18 - 22 November 2024	(Revision of S2-xxxxxxx)

Title:	[Draft] Reply LS on use of 3gpp-Sbi-Originating-Network-Id for Indirect Network Sharing case

Response to:	S2-2411294/C4-244497

Release:	Rel-19

# Work Item:	     TEI19_NetShare

Source:	SA2

To:	CT4, SA3

Cc:

Contact Person:	Qian Chen

E-mail Address:	qian (dot) xb (dot) chen (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	23.501 CR #5830 (S2-2411496)

1. Overall Description:

SA2 thanks CT4 for the LS (S2-2411294/C4-244497) on use of 3gpp-Sbi-Originating-Network-ID for Indirect Network Sharing case.

The LS highlights the issue as stated below:

1. Overall Description:

CT4 discussed stage 3 implementation to fulfil stage 2 requirement of Indirect Network Sharing as specified in clause 5.18.1 of TS 23.501:

The serving AMF determines the serving PLMN ID as follows:

-	For procedures involving the Network Function in the participating operator network (e.g. AUSF), the AMF sets the serving PLMN ID to the selected PLMN ID.

-	For procedures involving the Network Function only in the hosting operator network (e.g. PCF), the AMF sets the serving PLMN ID to the PLMN ID of the hosting operator.

In the case of Indirect Network Sharing (as specified in clause 5.18.5 of TS 23.501), the shared RAN will broadcast the PLMN ID representing the participating operator including three broadcast options (i.e. the HPLMN ID, EHPLMN ID or a PLMN ID different from HPLMN ID and EHPLMN IDs). Which broadcast option to be used is determined based on the agreement between the hosting operator and participating operator. The serving AMF is located in the hosting operator network and the selected PLMN ID which is broadcast by shared RAN is the PLMN ID representing the participating operator.Based on SA2 conclusion above, during the UE authentication procedure, the selected PLMN ID is used to form the Serving Network Name.

TS 33.501 specifies the following requirements:

6.1.2	Initiation of authentication and selection of authentication method

Upon receiving the Nausf_UEAuthentication_Authenticate Request message, the AUSF shall check that the requesting SEAF in the serving network identified by the 3gpp-Sbi-Originating-Network-Id header specified in TS 29.500 [74] is entitled to use the serving network name in the Nausf_UEAuthentication_Authenticate Request. For the case that the 3gpp-Sbi-Originating-Network-Id header is not included, the AUSF may authorize the serving network based on its name available in the Nausf_UEAuthentication_Authenticate Request message.

5.9.3.2	Requirements for Security Edge Protection Proxy (SEPP)

Receiving SEPP behavior for the 3gpp-Sbi-Originating-Network-Id header:

- 	The receiving SEPP shall check whether the 3gpp-Sbi-Originating-Network-Id header included in the signalling message belongs to the sending SEPP’s own PLMN or SNPN. It does this by verifying that the asserted PLMN ID in the 3gpp-Sbi-Originating-Network-Id header matches one of the sending SEPP's own PLMN ID(s) or SNPN ID(s) either in the N32-f context, the sending SEPP's certificate, or a locally configured list of PLMN IDs or SNPN-IDs that the sending SEPP represents.

2. Actions:

To SA3

ACTION:

CT4 would like SA3 to clarify how to set the PLMN ID in the 3gpp-Sbi-Originating-Network-Id header in the Indirect Network Sharing case to satisfy the aforementioned requirements during the UE authentication.

SA2 would like to inform SA3 and CT4 that SA2 has agreed the attached CR to resolve the EN in 23.501 related to the formulation of “Serving PLMN ID” when hosting network NF sends a request.

SA2 considers that Indirect Network Sharing is a network sharing scenario with support of multiple PLMN IDs in AMF of the hosting operator and the involved SEPPs can be deployed/configured to support such multiple PLMNs scenario based on agreement between operators.

2. Actions:

To SA3, CT4

ACTION: 	SA2 kindly asks SA3 and CT4 to take above information into consideration.

3. Date of Next SA2 Meeting:

SA2#166-Ad Hoc   			20 – 24 January, 2025		    		eMeeting

SA2#167   				17 – 21 February, 2025			        Athens, GR