3GPP TSG-SA2 Meeting #166	S2-2411698

# Orlando, Florida, 18-22 November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5143 | rev | --- | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#4: 23.502 SCP Event Exposure to detect signaling storm |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Oracle |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-11-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Derivative of agreed CR S2-2410976 and its revision S2-2411432 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define SCP Event Exposure service to enable SCP to help NWDAF in detection of signalling storm |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing input data may impede an NWDAF from detecting a signalling storm, and consequently impede NF(s) or/and SCP(s) from mitigating it. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New 5.2.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.2.X	SCP Services

#### 5.2.X.1	General

The following table shows the SCP Services and SCP Service Operations.

Table 5.2.26.1-1: NF services provided by the SCP

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nscp_EventExposure | Notify | Subscribe/Notify | NWDAF, DCCF |
|  | Subscribe | Subscribe/Notify | NWDAF, DCCF |
|  | Unsubscribe | Subscribe/Notify | NWDAF, DCCF |

#### 5.2.X.2	Nscp_EventExposure Service

##### 5.2.X.2.1	General

Service description: This service can expose information collected by the SCP to other NFs.

The following events can be notified to a NF consumer:

-	Abnormal Signalling characteristics: An NF, NF set, Service Type, may be provided as target.

The SCP reports:

-	Request type received from NF, e.g. Namf_Communication_N1N2TransferFailureNotification, as well as the number of requests received from NF during a time interval.

-	Ingress - Time duration between the request from NF and response to NF.

-	Egress - Time duration between the routed/forwarded request to producer NF and response received from it.

-	Number of successful responses associated to their initial requests during a time interval.

-	Number of failed responses associated to their initial requests during a time interval.

-	Reasons of failed responses associated to their initial requests, e.g. reject, no-response, etc.

-	Number of received redundant signalling during a time interval. The redundant signalling means the signalling which is transmitted multiple times.

-	A posterior Request types triggered from NF, for NF Service request.

-	Load information indicates the current load of the NF and NF Service(s), e.g. CPU, memory, and/or percentage of load information.

-	NF-NRF heart-beat related information such as responding time, Number of retransmissions, heart-beat intervals.

-	The number of different types of signalling received and sent by SCP during a target time period, etc. This parameter is valid for SCP only.

NOTE:	The “Abnormal Signalling characteristics” event is expected to be subscribed by an NWDAF to generate analytics related to abnormal signalling (see TS 23.288 [50]).

##### 5.2.X.2.2	Nscp_EventExposure_Notify service operation

Service operation name: Nscp_EventExposure_Notify

Description: This service operation reports the event and related information to the consumer that has subscribed.

Input Required: Event ID, Notification Correlation Information.

Input, Optional:, Event specific parameters as described in clause 5.2.X.2.1, time stamps for the measurements.

Output Required: Result Indication.

Output, Optional: None.

##### 5.2.X.2.3	Nscp_EventExposure_Subscribe service operation

Service operation name: Nscp_EventExposure_Subscribe

Description: This service operation is used by an NF to subscribe or modify a subscription to SCP event exposure notifications.

Input, Required: Event ID(s) defined in clause 5.2.X.2.1, Notification Target Address (+ Notification Correlation ID),

Input, Optional: Subscription Correlation ID (in the case of modification of the event subscription), Expiry time, Event specific target amd filtering information defined in clause 5.2.X.2.1

Output, Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator's policy).

Output, Optional: First corresponding event report is included, if available (see clause 4.15.1). Notification Target Address (+ Notification Correlation ID) is used to correlate Notifications sent by SCP with this subscription.

##### 5.2.X.2.4	Nscp_EventExposure_UnSubscribe service operation

Service operation name: Nscp_EventExposure_UnSubscribe

Description: The NF consumer uses this service operation to unsubscribe from an existing subscription.

Input, Required: Subscription Correlation ID.

Input, Optional: None.

Output, Required: Operation execution result indication.

Output, Optional: None.