SA WG2 Meeting #166	S2-2411319

# 14 - 18 November 2024, Orlando, USA

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1235 | rev | 3 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AF initiated VFL training procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Nokia], ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR Provides a revision on top of Nokia postponed S2-2410677 and do some EN clean ups. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the intermediate result may contain gradient information or loss information. Modify the name of intermediate result sent from VFL client to VFL server to intermediate training result. Delete the EN: Editor's note: It is ffs whether the NWDAF as VFL client also uses the collected input data to update its model. The client can update the model using gradient or compute gradient using loss and no need to collect input data on this step. Modify the EN and delete following: if loss information and gradient information is contained in intermediate results is FFS. As we agreed in the NWDAF initiated VFL training procedure, the backward local model information or intermediate result contain loss information and gradient information. So we propose to add these two example and modify the EN. Modify the following: Editor's note: Terminology should be aligned related among VFL related descriptions, in particular related to intermediate results and VFL Model correlation ID. We agreed to use VFL correlation ID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedures not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.3.1(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

6.2H.2.3	Vertical Federated Learning (VFL) training procedures

6.2H.2.3.1	Vertical Federated Learning (VFL) training procedures with AF as VFL server

Figure 6.2H.2.3.1-1: VFL model training with AF as VFL server

- The AF acting as VFL training server assigns a unique VFL Correlation ID.

Steps 2 to 7 are repeated until the MTLF acting as VFL training server determines that the training is complete.

2.	For each NWDAF VFL client, the AF acting as VFL training server sends a request to start a VFL training iteration. It includes the VFL Correlation ID. It also includes the intermediate results(e.g. gradient information or loss information) if step 2 is repeated. An untrusted AF identifies the VFL client using the temporary NWDAF ID assigned in the discovery procedure (see Clause 6.2H.2.1.1) and sends the request to the NEF. A trusted AF includes Analytics ID.

Editor's note:	Additional parameters, e.g. to indicate features and samples, are FFS. More clarifications about the intermediate results might be required;

3a.	For an unstrusted AF: The NEF looks up information about VFL client stored in the discovery procedure (see Clause 6.2H.2.1.1). It forwards the request to the VFL client and includes the stored selection criteria. If the message in step 2a contains external identifiers, the NEF maps them to internal identifiers.

Editor's note:	It is ffs whether the NEF stores selection criteria received during discovery and includes them in subsequent messages or if the AF needs to provide related parameters.

4.	If intermediate results were received in step 2 or 3a, each NWDAF/MTLF acting as VFL client updates the own local VFL model based on the received intermediate results.

5a.	Each NWDAF/MTLF as VFL training client further trains the local model and sends the intermediate training results to the VFL server AF. For an untrusted AF, the NWDAF/MTLF sends the message via the NEF.

6a	For an unstrusted AF: For each NWDAF/MTLF as VFL training client, the NEF forwards the intermediate training results to the AF

7.	The AF acting as VFL server updates the own model based on the received intermediate training results. The AF may in addition collect own input data and also use those data to update the model. For each VFL client, it determines intermediate results based on the updated model and provides those intermediate results to the VFL clients in the request to start the next training iteration (step 2).

8.	Once the AF acting as VFL server determines that the trained model is sufficiently stable to terminate the training, it informs each VFL clients that the VFL training is completed. An untrusted AF sends the message via NEF.

Editor's note:	It is ffs whether intermediate results can be included in step 8..

9a.	For an unstrusted AF: The NEF looks up stored information about VFL client and selection criteria. It forwards the request to the VFL client and includes the stored selection criteria.

Editor's note:	Which services are used between VFL server, NEF and VFL client is FFS and message names need to be confirmed.

Editor's note:	Terminology should be alligned related among VFL related descriptions, in particular related to intermediate results.

Editor's note:	The NEF procedures need to be confirmed.

10.	The AF acting as VFL server and the NWDAFs acting as VFL clients store their trained models along with the VFL correlation ID that will be used for inference later.

* * * End of changes* * *