3GPP TSG-SA WG2#166	S2-2411459

Orlando, US, Nov 18 – 22, 2024	(was S2-241xxxx)

Source:	Nokia, CableLabs, Oppo

Title:	Discussion paper on efficiently requesting QoS flows for devices behind UE

Document for:	Discussion / Information

Agenda Item:	19.8.2

Work Item / Release:	Release 19

Abstract of the contribution: This paper explains on how the 5GS enables efficient request of QoS flows for devices behind the UE.

# 1.	Background

Conclusion has the following captured

NOTE 2:	Who (the UE/5G-RG and/or the AF) decides to request for change of QoS Flows will be discussed and documented during the normative stage.

Agreed S2-2411083 with CR 1310 has the following note

Editor’s note: 	Whether and how SA2 specifications deal with the case where the Non-3GPP Device Identifier that the UE provided is not found in the UDR is FFS.

Note: In this discussion paper, all references of UE are to be considered and read as applicable to 5G-RG.

# 2	Discussion

It is to be noted that provisioning and association of non-3GPP devices with the non-3GPP device identifiers is out of the scope of 3GPP. The provisioning of non-3GPP device identifier information into the UDR is carried out by the AF; it is assumed that provisioning into the UDR through the AF may be carried out based on user inputs.

However, there could be scenarios that the information of non-3GPP device identifiers that are provisioned into the UE and the one provisioned into the UDR may not be in sync due to various reasons including human errors.

Considering the same, the UE could request for a QoS flow for a traffic that is associated with the non-3GPP device identifier for which a corresponding information may not be available in UDR. This results in PCF not being able to find corresponding information which tends towards inefficient and additional signaling vowing to ambiguity in the 5GS.

This would require (which also resolves the EN and tries to address the Note as per the above background section) 5GC to handle this a bit more efficiently, by the letting the UE be aware of the non-3GPP device identifiers that are actively available (and during the changes) in the UDR enabling the UE to request the QoS flow accordingly. Without which there is a gap between the steps of AF provisioning the Non-3GPP Device Identifier Information into the UDR and the UE/5G-RG triggering PDU Session procedures to request for QoS for the non-3GPP device. Out-of-scope mechanisms or UE implementations may be insufficient and lead to incompatible approaches.

# 3.	Proposal

The proposal is to discuss the corresponding CR which provide mechanisms to let the UE be aware of the Non-3GPP Device Identifiers available in the UDR, and then the UE may request the QoS flows for traffic associated with the corresponding Non-3GPP Device Identifiers only.

The corresponding CRs include S2-2411461 (23.503 CR 1377).