3GPP TSG-SA WG2 Meeting #166                                                                                          S2-2411721

18 - 22 November, 2024, Orlando, U.S.A.

Source:	NIST, US DHS Science & Technology, MITRE

Title:	Transmission Options of MANET Routing Messages to Support IP Type U2U Multi-Hop Relay

Document for:	Discussion

Agenda item:	19.7.2

Work Item / Release:	5G_ProSe_Ph3 / Rel-19

Abstract of the contribution: This document studies assumptions/requirements of transmitting MANET routing messages according to both NHDP+OLSRv2 protocols and SA2 agreements. Two options are outlined, and the need for clarification is identified.

# 1.	Introduction

Based on the discussion on IP type multi-hop U2U relay in previous meetings, the following principle is captured in Section 8.2 of TR 23.700-03 v19.0.0 under KI#2:

“Subsequent to a Layer-2 link establishment between a pair of 5G ProSe UEs configured to act as U2U Relays, the two U2U Relays initiate the MANET Neighborhood Discovery Protocol (NHDP) [IETF RFC 6130], followed by the OLSRv2 protocol [IETF RFC 7181]. The support for NHDP [IETF RFC 6130] and OLSRv2 protocol [IETF RFC 7181] is thus mandatory for the 5G ProSe UE configured to act as U2U Relay.”

The above text seems to imply that the MANET routing messages, such as HELLOs and TCs as defined in NHDP and OLSRv2, are exchanged between two U2U relays through unicast.

However, when studying the NHDP and OLSRv2  protocols, the following assumptions/requirements from RFC 6130 and RFC 7181 suggest that the MANET routing messages are exchanged among U2U relays through local broadcast or multicast:

RFC 6130 (NHDP): “This protocol makes no assumptions about the underlying link layer, other than support of local broadcast or multicast for communication to 1-hop neighbor routers.”

RFC 7181 (OLSRv2): ”OLSRv2 control messages are all sent using link-local multicast.”

In the context of U2U multi-hop relay scenario that SA2 is working on, a “router” in the above text from RFC 6130 refers to a U2U relay within the MANET cloud. Similarly, ‘1-hop neighbor routers’ correspond to connected next hop U2U relays that: 1) are connected to the source U2U relay via an established layer 2 ProSe Direct Link without any additional relay in between, i.e., the next hop U2U relay; and 2) belong to the same MANET cloud by sharing the same RSC. In addition, “the underlying link layer” in the cited text above refers to 3GPP defined layers (e.g., ProSe [TS23.304] or/and  SDAP {TS37.324} layers.

It is worth pointing out that although it was analyzed in S2-2406197 on how to provide multicast service over U2U multi-hop relay for upper layer applications such as MCPTT, the analysis there was based on the assumption that the MANET routing table is established/maintained properly already. However, the challenge identified in this Tdoc is about how to establish and maintain the MANET routing table, namely, how to exchange routing protocol messages, e.g., HELLOs and TCs.

Therefore, this contribution studies potential options to multicast MANET NHDP and OLSRv2 routing messages to immediate neighbor U2U relays in order to establish and maintain the IP routing table over IP type U2U multi-hop relays deployment. Furthermore, SA2 is kindly asked to evaluate the standards impact if different U2U relays are allowed to adopt different options, and the need for clarifications. This is because it is unclear whether MANET routing messages should be transmitted using unicast or multicast/broadcast after putting TR23.700-03 and several MANET RFCs together.

# 2.	Discussion

In addition to what have been quoted in the Introduction section, RFC 6130, RFC 7181 and RFC 5498 also have specified the following about the establishment and maintenance of the MANET routing table:

- The establishment and maintenance of MANET routing table rely on the periodic transmission of HELLO messages and TC messages by each router; and

- At the source router of these MANET routing messages, HELLOs and TCs are assigned to:

  - the specific well-known link-local scope multicast IP address: the “LL-MANET-Routers”, which is 224.0.0.109 for IPv4 and FF02:0:0:0:0:0:0:6D for IPv6.

The following observation is made based on the interpretations of the RFC texts referred above:

Observation 1: For those MANET routing messages (HELLOs and TCs) that are assigned to the link-local multicast IP address of the MANET routing messages, an IP type multi-hop U2U relay is expected to transmit them to all of its connected next hop U2U relays within the same MANET cloud.

There are at least two options to meet the “local broadcast or multicast’ assumption that is required in RFC 7181, with Option 1 an implicit interpretation of the principle from Section 8.2 of TR 23.700-03 and Option 2 being a direct implementation based on the wording in RFC 7181.

Option 1: The source U2U relay duplicates routing messages, and a separate copy is sent over each of the connected next hop U2U relays within the same MANET cloud individually using ‘Unicast mode 5G ProSe Direct Communication’ as defined in 23.304.

Option 2: The source U2U relay broadcasts routing messages to all next hop U2U relay within range using ‘Broadcast mode 5G ProSe Direct Communication’ as defined in 23.304. It is the responsibility of the receiving U2U relay to check whether the source U2U relay is connected (i.e., in the same MANET cloud and has the unicast ProSe direct link established).

Besides the above two options, other options are also possible. As a variation of Option 2, the local multicast is mapped to Layer 2 groupcast mode instead of broadcast mode.

Each of the above options has its pros and cons and may be implemented on its own without modifying existing 3GPP standards, if all U2U relays choose the same option. However, if U2U relays from different vendors implement different options, there might be compatibility issues when U2U relays with mixed options exchange MANET routing messages with each other. To be more specific, consider the mixture of Option 1 and Option 2 as an example:

Under Option 1, the source U2U relay is responsible to make sure MANET routing messages are only sent to connected next hop U2U relay, and the receiving U2U relay does not need to do filtering/checking. The drawback of this option though is the duplicate copies of the same routing messages over multiple ProSe direct links. The proposed periodicity of HELLO messages varies in a window between 0.5 sec and 2 sec, and the proposed periodicity of TC message varies in a window between 1.25 sec and 5 sec. Depending on the network needs, other values may be used as well. Thus the amount of redundant traffic over the air that is introduced due to the duplication is not negligible.

Under Option 2, the source U2U relay is responsible for passing the MANET routing messages from MANET multicast IP address to lower layer for layer 2 ProSe broadcast communication to all next hop U2U relay within range. If the receiving U2U relay finds that the source U2U relay of a routing message is not a connected U2U relay, the message will be discarded by either layer 2 or layer 3. This option has the potential to save radio resources by utilizing broadcast communication to transmit periodic traffic.

Consequently, if the source U2U relay (e.g., UE 2) implements Option 2 while the receiving U2U relay (e.g., UE 1) assumes Option 1, UE 1 might miss UE 2’s HELLOs or TCs because it is not listening in broadcast or groupcast modes for MANET routing messages. In addition, even if UE 1 can retrieve UE 2’s broadcast/groupcast messages, for the scenario when UE 2 is not a connected next hop U2U relay yet, UE 1 will still add UE 2 to its routing table prematurely. Thus the observation as follows:

Observation 2: If different U2U relays adopt different options to transmit MANET routing messages (e.g., unicast vs broadcast), there might be compatibility issues.

Therefore, it is necessary for SA2 to clarify and select the transmission option for MANET routing messages, so that UEs from different vendors may communicate with each other smoothly to form the U2U multi-hop relay MANET cloud.

Proposal: SA2 is kindly requested to discuss the need for clarification on the transmission option of MANET routing messages to support IP Type U2U multi-hop relay, and to capture the guidance/agreement accordingly.

3.	Conclusion

Based on the analysis of MANET related RFCs and SA2 principles, two transmission options of MANET routing messages to support IP Type U2U multi-hop relay were studied: one unicast, and one broadcast/multicast. The following observations and proposal are made:

Observation 1: For those MANET routing messages (HELLOs and TCs) that are assigned to the link-local multicast IP address of the MANET routing messages, an IP type multi-hop U2U relay is expected to transmit them to all of its connected next hop U2U relays within the same MANET cloud.

Observation 2: If different U2U relays adopt different options to transmit MANET routing messages (e.g., unicast vs broadcast), there might be compatibility issues.

Proposal: SA2 is kindly requested to discuss the need for clarification on the transmission option of MANET routing messages to support IP Type U2U multi-hop relay, and to capture the guidance/agreement accordingly.