SA WG2 Meeting #166	S2-2411768

# Orlando, USA, 18 – 22 November 2024                         (revision of S2-240xxxx)

Source: 	CATT

Title: 	KI#1 Conclusions on Topology 2 NAS-based option

Document for: 	Approval

Agenda Item: 	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes to introduce NAS-based option into Topology 2 architecture conclusions for KI#1.

1	Discussion

For Topology 2 architecture, the User-plane option and RRC-based option were agreed as interim conclusions at the last SA2#165 meeting, and the NAS-based option is left open.

This paper proposes to conclude NAS-based option as candidate Topology 2 architecture solutions.

2	Proposal

It is proposed to update TR 23.700-13 as follows.

# * * * * First change * * * *

8.1.3	Architecture to Support Topology 2

8.1.3.1	General

Three options will be specified for Topology 2:

-	User-plane option as defined in clause 8.1.3.2.

-	RRC-based option as defined in clause 8.1.3.3.

-	NAS-based option as defined in clause 8.1.3.4.

8.1.3.2	User-plane option

The following principles apply:

-	As depicted in Figure 8.1.3.2-1, the UE Reader connects to the AIoTF based on the AIoT Application Protocol (AIoT-AP) using an IP PDU Session between the UE and the UPF as transport. The related protocol stack is shown in Figure 8.1.3.2-2. The AIoT AP protocol will support procedures and information to be exchanged as specified by RAN WG2, RAN WG3 and SA WG2.

Figure 8.1.3.2-1: User-plane architecture for Topology 2

Figure 8.1.3.2-2: Protocol stack for the user-plane architecture for Topology 2

NOTE 1:	Which transport protocol to use for AIoT-AP can be decided by CT WG1 in coordination with RAN WG3.

NOTE 2:	Security for AIoT-AP is assumed to be defined by SA WG3.

Editor's note:	Further details are FFS.

Editor's note:	The terminology, e.g. AIoT RAN is FFS.

8.1.3.3	RRC option

The following principles apply:

-	Messages between the UE Reader and the AIoTF are delivered using RRC and a protocol between gNB/AIoT RAN and AIoTF. The related protocol stack options are shown in Figure 8.1.3.3-1 and Figure 8.1.3.3-2.

Editor's note:	Which option to use, i.e. whether gNB/AIoT RAN connect to AIoTF directly (Option A in Figure 8.1.3.3-1) or via AMF (Option B in Figure 8.1.3.3-2) is FFS.

Editor's note:	How addressing works for UL for Option B is FFS.

Editor's note:	Further details are FFS.

Figure 8.1.3.3-1: Protocol Stack for the RRC option (Option A)

Figure 8.1.3.3-2: Protocol Stack for the RRC option (Option B)

8.1.3.4	NAS option

The following principles apply:

-	As depicted in Figure 8.1.3.4-1, messages between the UE Reader and the AIoTF are delivered using N1 NAS message between UE and AMF. The related protocol stack option is shown in Figure 8.1.3.4-2.

Figure 8.1.3.4-1: NAS option architecture for Topology 2

Figure 8.1.3.4-2: Protocol Stack for the NAS option

# * * * * End of Changes * * * *