3GPP TSG-WG SA2 Meeting #166	S2-2411882

# Orlando, USA, 18th Oct – 22th Nov, 2024		(revision of S2-2410315,08851,10315)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5518 | rev | 3 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of UE-satellite-UE communications when serving satellite changes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, CSCN, Tencent, Honor, [Thales, CATT, NTT DOCOMO] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-11-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To maintain service continuity and consider the user’s QoE when the UE serving satellite changes, the following principles serve as the baseline for implementing this feature: Assumption #1: It is assumed that the gNB, UL CL/L-PSA, and AGW are all onboard the same satellite. Assumption #2: During a change in the serving satellite, an Xn/N2 handover will be executed. To minimize user plane updates, it is proposed that the UL CL/L-PSA does not change simultaneously with the gNB. Assumption #3: When an N2 handover is performed, the 5GC user plane should fall back to the ground. Assumption #4: If the 5GC user plane fall back to ground, it is proposed that the UE-SAT-UE communication should be terminated. Therefore, to achive service continuity based on above mentioned principles: 1). SMF should be enhanced to acknolwdge the HO type, i.e. N2 or Xn HO, which is not supported in current spec. One option is to let AMF notify SMF with such type, the other is to enable SMF find it out by itself, e.g. via verifying the connectivity between source gNB satellite and target gNB satellite. It is proposed to document the second methond, SMF verify HO type based on the source and target satellite IDs. 2). In case of N2 HO, SMF fall back the PDU session to ground-PSA, and notify IMS that the 5GC user plane has been fall back to the ground, i.e. the pre-condition of UE-SAT-UE in 5GC scope cannot be maintained, corresponding procedure of terminating UE-SAT-UE will be captured in TS23.228 In case of N2 CHO, user plane fallback indication is suggested to be sent to the P-CSCF to enable IMS terminate U-S-U. 3). In case of Xn HO, UL CL/L-PSA in the source satellite is used until a new DNAI (AGW on ground or satellite) is informed from IMS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause to describe the feature of when UE’s gNB onboarded serving satellite changes, basically: - SMF needs to know the type of HO based on source and target satellite ID - For N2-HO, UE-SAT-UE should be terminated. - For Xn-HO, UL CL/L-PSA is kept in source satellite until P-CSCF informs a DNAI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Voice/video call drop rate significantly increased when the serving satellite changes |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.x.2.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.4.x.y	Support for UE-satellite-UE communication when serving satellite changes

Editor’s Note:	the following mechanism assumes gNB, UL CL/L-PSA, IMS-AGW onboarding the same satellite, whether and how to adjust it to apply to other deployments is FFS.

In the UE-SAT-UE communication, if one of the UE’s serving satellite changes (e.g. due to UE’s movement, or the satellite’s movement), the Xn/N2-based handover procedure will be performed to change the serving gNB from the source satellite to the target satellite. The AMF will include the target satellite ID in the Nsmf_PDUSession_UpdateSMContext Request to the SMF, and the SMF will derive the handover type based on the N2 SM information.

If N2 handover is executed, the SMF will decide to revert the PDU session to the PSA on the ground, reusing the procedure for removing the UL CL/L-PSA as described in clause 4.3.5.5 in TS 23.502 [3], and SMF will generate a user plane fallback indication that shows the 5GC user plane has fallen back to the ground (i.e. the condition of UE-SAT-UE communications in 5GC is not satisfied anymore) and report the target satellite ID and this indication to PCF via Npcf_SMPolicyControl, and the PCF will then notify them to the P-CSCF via Npcf_PolicyAuthorization.

If Xn handover is executed, the SMF will decide to retain the existing UL CL/L-PSA on the source satellite, reusing the Xn-based handover procedure without relocating the user plane as described in clause 4.9.1.2.2 in TS23.502 [3] and report the target satellite ID to the PCF via Npcf_SMPolicyControl, and the PCF will then notify them to the P-CSCF via Npcf_PolicyAuthorization_Notify. IMS will inform 5GC a decision of whether the UE-SAT-UE can be continued or not, as outlined in clause AX.5.Y of TS 23.228 [15], i.e. the PCF will receive a DNAI associated with the IMS-AGW on satellite or on ground, and an IMS-derived transport address and forward them to the SMF:

-	If the receiving the DNAI associated with the IMS-AGW on satellite, the SMF changes the UL CL/L-PSA to the target satellite by performing a simultaneous change of UL CL and additional PSA for the PDU Session, as described in clause 4.3.5.7 of TS 23.502 [3], where the UL CL rule will be updated with the IMS-derived transport address.

-	If the receiving the DNAI associated with the IMS-AGW on ground, the SMF removes the UL CL/L-PSA on the source satellite by following the procedure described in clause 4.3.5.5 of TS 23.502 [3].

# * * * * End of change * * * *