3GPP TSG-WG SA2 Meeting #166	S2-2411888

# Orlando, USA, 18th Oct – 22th Nov, 2024		(revision of S2-2400125)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1476 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Functional descriptions of UE-satellite-UE communications when satellite changes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of features to support UE-satellite-UE communications for satellite communications based on the conclusions for KI#3 in TR 23.700-29. - mobility support of UE-SAT-UE communication: To ensure service continuity when a UE's serving gNB satellite changes during UE-SAT-UE communication (due to UE or satellite movement), the P-CSCF determines whether the UE-SAT-UE can be continued, if yes, the P-CSCF selects an AGW on the new satellite; if not, the P-CSCF selects an AGW on the ground. - AGW reallocation: AGW re-allocation in UE-SAT-UE communication involves selecting a new AGW, re-negotes transport addresses. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 2 new clauses are added. - mobility support of UE-SAT-UE communication when satellite changes - AGW reallocation when satellite changes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE-SAT-UE communication cannot be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AY.3.y(new), AY.4.y(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# AX.3	Optimized media routing activation

## AX.3.y	At change of satellite

To support service continuity when one of the UE’s serving gNB satellite in the UE-SAT-UE communication changes (e.g. UE’s movement, or satellite’s movement), the P-CSCF in the satellite change network side needs to be notified of the new satellite ID and access information related to the UE, and in addition a user plane fallback indication if applicable.

The continuation/termination of the UE-SAT-UE communication decision is triggered at the satellite change network side.

-	If there is no user plane fallback indication received (i.e. the user plane is still kept on a satellite), the P-CSCF will further determine whether the UE-SAT-UE communication can be continued based on the identifiers of UE-1’s new serving satellite and UE-2’s serving satellite. The P-CSCF obtains the UE’s new satellite identifier from PCF as defined in TS 23.502 [94], and UE-2’s satellite identifier from local storage.

-	If there is user plane fallback indication received (i.e. the user plane is fallback to the ground), the P-CSCF will determine to terminatethe UE-SAT-UE communication.

If the P-CSCF determines that UE-SAT-UE communication can be continued at change of satellite, the P-CSCF shall trigger a re-Invite procedure to relocate the AGW from the old satellite to the new satellite, and send a message to the PCF as defined in TS 23.502 [94] to establish a path on the new satellite for optimized media routing.

If the P-CSCF determines that UE-SAT-UE communication cannot be continued at change of satellite e.g. the UE-1’s new satellite has no ISL with UE-2’s satellite, the P-CSCF shall trigger a re-Invite procedure to relocate the AGW from the old satellite to the ground, and send a message to the PCF as defined in TS 23.502 [94] to delete the path on the old satellite.

# * * * * Second change * * * *

# AX.4	Interaction with IMS AGW

## AX.4.y	At change of satellite

In case when one of the UE’s serving gNB-onboarded satellite in the UE-SAT-UE communication changes,

- A new AGW on the satellite needs to be selected if UE-SAT-UE is to be continued,

- New AGWs on the ground need to be selected on both the originating side and the terminating side if UE-SAT-UE is to be terminated.

As it is during the call session, transport address re-negotiation is needed. The SIP RE-INVITE/200 OK (RE-INVITE) can be used to exchange the updated transport addresses reserved with the newly selected AGW(s).

# * * * * End of change * * * *