3GPP TSG SA WG-2 Meeting #166 	S2-2412196

Orlando, USA, November 18 – 22, 2024

# Source:	vivo, China Mobile, EchoStar, Inmarsat, KT Corp., MediaTek Inc., Qualcomm, Spreadtrum, UNISOC, Viasat, ZTE Corporation

# Title:	New SID on IMS voice call using GEO satellite access

# Document for:	Information

# Agenda Item:	30.8

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on IMS voice call using GEO satellite access

Acronym:	FS_IMS_GEO

Unique identifier:	TBD

Potential target Release:	Rel-20

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No | X |  |  |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  |  |  | N/A |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 1273 | Provisioning of IP-based multimedia services | Stage 2 architecture and procedures for supporting IMS |
| 32045 | PS domain and IMS impacts for supporting IMS Emergency calls (EMC1) | Stage 2 architecture and procedures for supporting IMS emergency call |
| 320005 | 3GPP System Architecture Evolution Specification - Evolved Packet System (non RAN aspects) | Stage 2 enhancements for supporting IMS with EPS |
| 380064 | Support for IMS Emergency Calls over GPRS and EPS | Stage 2 enhancements for supporting IMS emergency call with EPS |
| 930019 | Architecture support for NB-IoT/eMTC Non-Terrestrial Networks in EPS (IoT_SAT_ARCH_EPS) | The work will take into account stage-2 Normative Work on the support of NB-IoT NTN in EPS |
| 920069 | NB-IoT/eMTC support for Non-Terrestrial Networks (LTE_NBIOT_eMTC_NTN) | The work will take into account stage-2 Normative Work on the support of NB-IoT NTN in EPS |
| 980014 | 5GC/EPC enhancement for satellite access Phase 2 | SA2 Rel-18 WID for 5G System enhancement for satellite access |
| 1030042 | Study on satellite access - Phase 4 | Stage 1 study for objective of IMS voice call using GEO in Rel-20 |

# 3	Justification

Satellite communication serves as an essential complement to terrestrial network coverage, ensuring continuous communication access for everyone, everywhere, at any time. This approach introduces new opportunities and value across a wide range of markets, including IoT and smartphone sectors. As 3GPP IoT Non-Terrestrial Networks (NTN) gain traction, smartphones increasingly support small data and SMS capabilities, making seamless, widespread connectivity a reality and sparking innovations across industries that depend on resilient, ubiquitous networks.

Geostationary Earth Orbit (GEO) satellites are particularly suited for providing global coverage for low data rate services. Leveraging existing GEO satellites allows for rapid deployment of messaging and voice services. In certain markets, smartphones already support GEO satellite direct communication for SMS, small data, and low-rate emergency voice calls using non-3GPP technologies. While market acceptance for these solutions has been positive, they lack the economies of scale the 3GPP ecosystem offers.

For GEO satellite access, 3GPP standards in Releases 17 and 18 focus on transparent payloads, while Release 19 shifts toward regenerative payloads, primarily for NGSO systems (LEO/MEO). Four radio access technologies (RATs) are specified for GEO satellites: NB-IoT (GEO), LTE-M (GEO), WB-E-UTRAN (GEO), and NR (GEO). Among these, NB-IoT (GEO) is often preferred due to its narrow bandwidth (3.75 kHz to 180 kHz, as per TS 36.213), making it ideally suited for the high altitude of GEO satellites at 35,786 km and efficient in managing resources required for IMS voice calls. While NB-IoT (GEO) already supports direct communication over GEO satellites (930019), its design inherits NB-IoT’s inherent restriction to support voice as related functionality was explicitly disabled in 3GPP specifications.

3GPP SA1 has studied the IMS voice support over GEO satellite in Rel-20 since Mar. 2024 (1030042). The use case has been well justified with corresponding potential requirements been captured in TR 22.887. Corresponding SA/RAN work in Rel-20 is needed to enable the low data rate (1~3kbps) IMS voice using GEO satellite.

This study aims to improve the IMS service by leveraging NB-IoT (GEO) satellite access (transparent payload), thereby addressing the specific architecture enhancements for IMS voice call capabilities in satellite communications​. This feature may be applicable to UE operating in terrestrial networks that follow a unified design approach.

# 4	Objective

Based on the above justification, the objectives of this study are to enhance IMS services, with a particular focus on voice and emergency calls, by utilizing NB-IoT (GEO) satellite access. The primary tasks of this investigation include:

WT#1: Enhance network architecture

-	WT#1.1: Study how to support IMS signalling and media transmission using NB-IoT (GEO) satellite access, e.g. via Control Plane CIoT Optimizations, User Plane CIoT Optimizations or S1-U

-	WT#1.2: Study how to support IMS voice using NB-IoT (GEO) satellite access via EPC, and 5GC (assuming 5GC supports IoT NTN)

NOTE 1: Regarding protocol overhead reduction/optimization for IMS voice packets, coordination with CT1 and RAN2 is needed

NOTE 2: No mobility enhancement is targeted.

WT#2: Simplify IMS procedures/messages to shorten call setup time

-	WT#2.1: Study whether and how to optimize the end-to-end procedures (e.g. call setup procedure) to reduce the interactions between UE and GEO satellite

-	WT#2.2: Study how to optimize/compress/substitute the SIP messages to enhance the transmission efficiency

WT#3: Support for emergency session procedures over NB-IoT (GEO) satellite access

-	WT#3.1: Study how to support procedures for emergency sessions over NB-IoT (GEO) satellite access e.g. support for domain selection, limited service state procedures, location based routing etc.

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT#1 | 3 | 3 | Yes |  |
| WT#2 | 3 | 3 | No |  |
| WT#3 | 2 | 2 | Yes |  |

Total TU estimates for the study phase: 8

Total TU estimates for the normative phase: 8

Total TU estimates: N + M = 16

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.xyz | Study on IMS voice call using GEO NB-IoT access | TSG#112 Jun. 2026 | TSG#113 Sept. 2026 | TBD |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

TBD

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

The following aspects involving other WGs may arise related to this SID:

-     SA3 for the Security aspects

-     SA5 for the Charging aspects

# 9	Supporting Individual Members

Supporting IM name

vivo

China Mobile

EchoStar

Inmarsat

KT Corp.

MediaTek Inc.

Qualcomm

Spreadtrum

UNISOC

Viasat

ZTE Corporation